3GPP TSG-WG SA2 Meeting #169	S2-2505594

Fukuoka, Japan, May 19 - 23, 2025	(revision of S2-2504889)

Source:	Ericsson, Lenovo, Huawei, HiSilicon, CATT

Title:	Clause 5.7.2: Update on AIoT Device Permanent Identifier

Document for:	Approval

Agenda Item:	19.14.2

Work Item / Release:	AmbientIoT-ARC / Rel-19

Abstract: This pCR updates the AIoT Device Permanent Identifier to resolve the EN.

1. Introduction

In SA2#168, there is an EN left to further study on the use case when the domain information of the AIoT Device Permanent Identifier is empty:

Editor's note:	Whether the Domain Information can be empty needs to be clarified.

The Domain Information or the AIoT Device Permanent Identifier is used to determine the ADM instance or the external server for AIoT Device Profile Data retrieval. It can be performed by NRF query or by local configuration in the AIOTF.

Without Domain Information, the concern is on how to retrieve AIoT Device Profile Data.

Based on SA2 agreement in SA2#168, the AIoT Device Permanent Identifier can be used in this case. It requires the AIOTF needs to be able to determine the ADM instance or the external server based on the remaining part of the AIoT Device Permanent Identifier, the Identification Information (i.e., EPC).

[Proposal-1] It is proposed to resolve the EN and add a NOTE to clarify the discussion above.

2. Proposal

It is proposed to agree the following changes to 3GPP TS 23.369 v0.3.0:

* * * Start of Change * * * *

### 5.7.2	AIoT Device Permanent Identifier

In order to support the AIoT feature in 5G System, a globally unique AIoT Device Permanent Identifier shall be allocated to each AIoT Device. An AIoT Device Permanent Identifier is assigned either by an operator or by a third party. The AIoT Device Permanent Identifier is used to identify an AIoT Device and locate the entity where the AIoT Device related information is stored.

NOTE 1:	How to configure an AIoT Device with the AIoT Device Permanent Identifier is out of scope of this specification.

The AIoT Device Permanent Identifier includes the following components:

-	The ID Type, including:

-	Information indicating whether a PLMN ID is included.

-	Information indicating whether a NID is included.

-	Information indicating whether a third party identifier is included.

-	Identification Information Type, indicating whether the Identification Information contains an  EPC or unstructured information.

-	The Domain Information includes none, one or more of the following:

-	A PLMN Identifier (i.e., MCC and MNC) as specified in TS 23.003 [6] when the information in the ID type indicates it is included

-	A Network Identifier (NID) as specified in TS 23.003 [6] when the information in the ID type indicates it is included.

-	A third party identifier used to identify a third party when the information in the ID type indicates it is included.

-	The Identification Information is used to distinguish different AIoT Devices within the scope identified by Domain Information (if available) and can contain either:

-	An EPC, as defined in clause 14 of GS1 TDS Release 2.1 [7].

-	Unstructured information, where the contents is defined by the allocator.

Figure 5.7.2-1: AIoT Device Permanent Identifier Structure

An operator allocated AIoT Device Permanent Identifier should include the identifier of the network for the operator. The identifier of the network is present as either a PLMN Identifier, NID or both in the AIoT Device Permanent Identifier.

A third party allocated AIoT Device Permanent Identifier may include none of the following information or include any combination of at least one kind of the following information: a PLMN Identifier, NID or the third party identifier.

NOTE 3:	The length of ID Type, PLMN Identifier (if present), NID (if present) and the third party identifier (if present) components is fixed. The length of the Identification Information is variable. The details are specified in TS 23.003 [6] and TS 29.xxx [xx].

Editor's note:	The reference in NOTE 3 needs to be updated, when the appropriate stage 3 document is identified.

NOTE X:	When the Domain Information is empty, the AIOTF uses, e.g., Identification Information (i.e., EPC) to discover and select the ADM instance or the external server for the AIoT Device Profile Data.

The following lengths are supported for the Identification Information in an AIoT Device Permanent Identifier: 96 bits, and 128 bits.

NOTE 4:	The encoding for the length of the Identification Information enables additional shorter or longer fixed lengths to be supported in the future.

* * * End of Change * * * *