3GPP SA WG2 Meeting #169	 S2-2505709

# Fukuoka, Japan, 19-23 May 2025								           (was S2-2504977, S2-2505688)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.401 | CR | 3926 | rev | 2 | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Transition to S&F satellite for legacy UE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated, Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GSAT_Ph3-ARC |  |  |  |  |  | Date: |  |  | 2025-05-19 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The current procedure defined in clause 4.13.9.2 assumes that the UE follows the existing AS and NAS procedures, when the cell that is camped changes from S&F to non-S&F (and vice versa). Nevertheless, for a legacy UE that does not understand the S&F indicator in the SIB, this may mean that in IDLE mode the UE can inadvertedly stay camped on a S&F cell for some time and only realise when it has to initiate MO signalling when it will be rejected by the MME on the satellite. In order to avoid this, two possible configurations are proposed that can be selected by the deployment: - S&F cell has to be barred for UEs not supporting S&F; - S&F cells and non-S&F cells have to be in different TA-list registered by the MME which will force the UE to do a TAU, and, according to existing procedures defined in clause 4.13.9.1, the MME rejects it using a non-S&F cause code. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Proposes new text to avoid a UE not supporting S&F erroneously staying in S&F cell . |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Legacy UEs will be denied service if inadvertedly camped on S&F cell. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.13.9.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# >>>>BEGINNING OF CHANGES<<<<

#### 4.13.9.2	Transition between S&F mode and non S&F mode

If a UE is attached to a PLMN using satellite operating in S&F mode, the UE follows the existing AS and NAS procedures to access a satellite that is not operating in S&F mode (either TN or NTN eNodeB) and vice versa.

NOTE: If it is desired to prevent UEs not supporting S&F to inadvertedly camp on a cell provided by a satellite in S&F mode during idle mode mobility, either 1) the S&F cell need to be configured as barred for legacy UEs, as defined in TS 36.304 [34] or 2) the S&F cells and non S&F cells need to be using different Tracking Areas in order to trigger the UE to perform Tracking Area Update.

In order to ensure smooth idle mode transition from S&F mode to non S&F mode, if the old (S&F capable) MME in inter MME mobility has the knowledge that the UE context is not up to date (e.g. based on implementation) then the old (S&F capable) MME rejects the MM context transfer.

If a satellite has no feeder link connectivity, it can only operate in S&F Mode. When a satellite supporting S&F mode obtains a feeder link, then, the satellite may cease to operate in S&F Mode depending on operator policy and regulatory requirement.

# >>>>END OF CHANGES<<<<