3GPP TSG-WG SA2 Meeting #169	S2-2505802

Fukuoka City, Fukuoka, JP, 19th May – 23rd May, 2025	(revision of S2-2505340, 5595)

Source:	Huawei, HiSilicon, ZTE

Title:	Inventory Procedure Update

Document for:	Approval

Agenda Item:	19.14.2

Work Item / Release:	AmbientIoT-ARC / Rel-19

Abstract: Clause 6.2.2 is updated to solve the Editor’s Note and add more clarifications.

# 1. Introduction/Discussion

The clause 6.2.2 about Inventory Procedure is updated to solve the ENs and add more clarification and reference to make the procedure clearer.

Editor's note:	The parameters for the inventory service operation need further definition.

Clause 5.3 and 5.8 have defined External Target Area information and Filtering Information. The approximate number of AIoT Devices has been defined in step 1. Therefore, all the parameters included in the Nnef_AIoT_Inventory message have been defined and the Editor’s note can be deleted.

Editor's note:	Whether the Inventory Request sent to NG-RAN includes indication about whether there will be a follow up command or not needs to be determined.

LS from RAN3 (R3-252481) on Ambient IoT further progress of RAN3 mentions that there will be a “follow-on command indication” in the AIOTF information container in the Inventory Request message. Therefore, the “follow-on command indication” will be captured in the step 7 in the command procedure in clause 6.2.3. This Editor’s note can be deleted without any update.

In addition to solving the ENs in this clause, further clarifications are added:

- Add reference to clause 5.3 for the Target Area information which is determined by NEF.

- When AIOTF performs authorization of the AF request, it should also determine the Filtering Information, especially when AF does not provide the Filtering Information and there is Filtering Information included in the AF authorization data. Otherwise, all device will be paged and this will waste unnecessary radio resource.

- Add reference to RAN3 specification in step 7, 8 and 10.

- Add reference to RAN2 specification in step 9.

- In order to further optimize the radio resource utilization, AIoT Device can only report the truncated ID to further shorten the length of ID required to report. Therefore, further clarification is added in step 9 and 11. Since ID type in the AIoT Device ID can indicate all the components included in the AIoT Device ID and AIOTF has the information of Filtering Information. Therefore, by using this information, even AIoT Device report the truncated AIoT Device ID, the AIOTF can split and reconstruct the complete AIoT Device ID.

# 2. Text Proposal

It is proposed to capture the following changes vs. TS 23.369.

# * * * * First change * * * *

## 6.1	General

Clause 6.2 describes the AIoT procedures and Network Function services for the 5GS by end-to-end information flows and making use of the NF service operations defined in clause 7, in those information flows.

# * * * * Second change * * * *

## 6.2	AIoT Service Procedures

### 6.2.1	General

Clause 6.2.2 provides the procedure for AIoT Inventory. Clause 6.2.3 provides the procedure for AIoT Command.

### 6.2.2	Inventory Procedure

Figure 6.2.2-1 describes the inventory procedure.

The procedure focuses on the messages and parameters used for the communication between AIOTF and NG-RAN regardless of the path to access NG-RAN, see clause 4.2.2.1. The handling of the different communication paths is described in clause 6.2.4.

Figure 6.2.2-1: Inventory Procedure

1.	The AF invokes Nnef_AIoT_Inventory(AF ID, [External Target Area information], [information about the target AIoT Device(s)], [Approximate number of AIoT Devices], [time interval]) service operation request to the NEF.

The External Target Area information is specified in clause 5.3.

Information about the target AIoT Device(s) may include Filtering Information, as described in clause 5.8, or include complete AIoT Device Identifier(s).

The approximate number of AIoT Devices, if provided, is used to determine the number of AIoT Devices expected to respond to this inventory service operation, which is sent by AIOTF to the NG-RAN in the assistance information for NG-RAN in step 7 for proper radio resource allocation.

The time interval, if provided, is described in clause 5.x.

2.	The NEF may further authorize the AF request as specified in clause 5.6.

The NEF determines the Target Area information, and selects one or multiple AIOTF(s) to handle the request as specified in clause 5.3.1. The Target Area information is specified in clause 5.3.

3.	The NEF invokes the Naiotf_AIoT_Inventory(AF ID, [Target Area information], [information about the target AIoT Device(s)], [Approximate number of AIoT Devices], [time interval]) service operation towards to the selected AIOTF(s).

4.	The AIOTF receives the AIoT service operation request and checks the parameters included in the request. The AIOTF may perform authorization as specified in clause 5.6. If the AIoT service operation request cannot be processed, the AIOTF rejects the AIoT service operation request with an appropriate cause code, and step 7 onwards are skipped.

The AIOTF generates a Correlation ID corresponding to this AF service operation request.

The AIoT Identification Information to be provided to NG-RAN can include Filtering Information, as defined in clause 5.8, or a single AIoT Device Identifier. The AIOTF determines the Filtering Information included in AIoT Identification Information provided to NG-RAN taking into account the parameters provided in the service request.

AIOTF performs Reader Selection, see clause 5.3.3.

The AIOTF may also use the last serving Reader to assist with determining which Readers to use for an AFs request targeting for a specific AIoT Device.

The AIOTF determines assistance information as described in clause 5.4, taking into account the parameters provided in the service request.

5.	AIOTF sends the AIoT Inventory Service Response to the NEF containing the accept or reject result for the AIoT Inventory service operation request based on step 4.

6.	NEF sends the AIoT service operation response to the AF, containing the accept or reject result for the AIoT Inventory service operation request as specified in clause 8.3.

7.	The AIOTF sends the Inventory Request message including the Correlation ID, the AIoT Identification Information to be included in the paging message, and assistance information to the selected NG-RAN as specified in TS 38.413 [y].

8.	The NG-RAN sends an Inventory Response to the AIOTF with the Correlation ID indicating that the Inventory Request is received successfully and will perform the service operation accordingly as specified in TS 38.413 [x1].

9.	Upon reception of the Inventory Request message from the AIOTF, the RAN Reader(s) will execute the inventory operation as specified in TS 38.300[5] and TS 38.391 [x3]. The RAN Reader(s) broadcast the paging message that includes the AIoT Identification Information.

The AIoT Device determines whether it matches the AIoT Identification Information, as described in clause 5.8.

If an AIoT device matches the AIoT Identification Information in the paging message, the AIoT Device responds to the paging message and sends an AIOT NAS message that includes its identity.

Editor's note:	Whether and how the Device ID is concealed or encrypted will be determined and aligned with SA WG3.

10.	NG-RAN sends one or more Inventory Report messages to the AIOTF including the Correlation ID, Reader ID and the AIOT NAS message(s) from the AIoT Device(s) as specified in TS 38.413 [x1]. The NG-RAN may aggregate multiple Inventory Report messages based on the assistance information before reporting the response to the AIOTF as described in clause 5.x. The AIOTF stores the mapping between the Reader ID and AIoT Device ID(s).

NOTE:	When to erase the stored mapping between the Reader ID and AIoT device ID(s) is up to implementation and local configuration.

11.	The AIOTF validates the results, using local stored device information or device profile data retrieved from the ADM. The AIOTF may aggregate the results.

12.	Optionally, if the NG-RAN detects that no more AIoT Devices will respond to the inventory procedure, then NG-RAN informs the AIOTF that the procedure is complete and the last inventory result. After the procedure has completed NG-RAN will not send any further Inventory Reports for this requested Inventory.

Editor’s Note:	The details about completion of the procedure need to be aligned with RAN.

13.	The AIOTF reports the progress of the AIoT inventory request to the NEF by sending the Naiotf_AIoT_Notify message including a list of AIoT Device Permanent Identifier (s). The AIOTF may send multiple reports. The AIOTF in the final Naiotf_AIoT_Notify message indicates it is the last report for this operation. If multiple AIOTFs are involved in the procedure, the NEF may receive the AIoT_Notify from multiple AIOTFs.

14.	When receiving the Naiotf_AIoT_Notify message from AIOTF, the NEF informs the AF of the outcome of the AIoT_Inventory request by sending the Nnef_AIoT_Notify message(s) including the AIoT Device Permanent Identifier(s). The NEF in the final Nnef_AIoT_Notify message indicates that it is the last report for this operation.

# * * * * Second change * * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 22.369: "Service requirements for Ambient power-enabled IoT".

[3]	3GPP TS 23.501: "System Architecture for the 5G System (5GS); Stage 2".

[4]	3GPP TS 23.502: "Procedures for the 5G System; Stage 2".

[5]	3GPP TS 38.300: "NR; Overall description; Stage-2".

[6]	3GPP TS 23.003: "Numbering, Addressing and Identification".

[7]	GS1 TDS Release 2.1: "EPC Tag Data Standard".

[8]	3GPP TS 33.501: "Security architecture and procedures for 5G system".

[9]	3GPP TS 33.369: "Security aspects of ambient IoT services in 5G".

[x1]	3GPP TS 38.413: "NG-RAN; NG Application Protocol (NGAP)".

[x2]	3GPP TS 38.391: "Ambient IoT Medium Access Control Protocol specification".

# * * * * End of changes * * * *