SA WG2 Meeting #169	S2-2505840

Fukuoka, Japan, May 19 – 23, 2025	(revision of S2-2505589 of S2-2505012)

Source:	LG Electronics, Toyota

Title:	KI#1, NewSol: Enhancement of energy related information exposure

Document for:	Approval

Agenda Item:	20.4.1

Work Item / Release:	FS_EnergySys_Ph2 / Rel-20

Abstract of the contribution: This paper proposes a new solution for KI#1.

Discussion

This paper proposes a new solution for KI#1 "Network energy related information exposure".

Proposal

It is proposed to agree the following changes into TR 23.700-67.

* * * * Start of 1st Change * * * *

6.0	Mapping of Solutions to Key Issues

Editor's note:	The number of columns of Key Issues will be updated according to clause 5.

Table 6.0-1: Mapping of Solutions to Key Issues

|  | Key Issues |  |  |
| --- | --- | --- | --- |
| Solution# | 1 | 2 | 3 |
| 1 | X |  |  |
| 2 | X | X |  |
| 3 |  | X |  |
| 4 |  | X |  |
| 5 |  | X |  |
| 6 |  | X |  |
| 7 |  | X |  |
| Z | X |  |  |

* * * * Start of Next Change * * * *

!!!! All New Texts !!!!

6.Z	Solution #Z: Enhancement of energy related information exposure

6.Z.0	High-level solution principles

The proposed solution

-	reuses the existing functionalities to support Energy Efficiency and Energy Saving specified in TS 23.501 [2] and TS 23.502 [3].

-	enhances EIF functionality to support additional information related to contributors in terms of energy consumption.

-	enhances Neif_EventExposure and Nnef_EventExposure service operations to request and expose additional information related to contributors in terms of energy consumption, e.g. in addition to energy consumption information for UE#1, information that Application_a, Application_c and Application_b are the top contributing application that consumed energy by UE#1 in a descending order.

6.Z.1	Description

Along with UE level energy consumption information, it can be considered useful for AF to know which applications contributed the most to energy consumption for the UE. Along with PDU Session level energy consumption information, it can be considered useful for AF to know which applications contributed the most to energy consumption for the PDU Session.

Therefore, this solution proposes that the EIF supports to provide the following information to the AF based on the AF request:

-	List(s) of top contributors that contribute the most to the energy consumption:

-	When the EIF exposes the UE level energy consumption information to the AF, the EIF can provide a list of applications that contribute the most to the energy consumption for the UE.

-	When the EIF exposes the PDU Session level energy consumption information to the AF, the EIF can provide a list of applications that contribute the most to the energy consumption for the PDU Session.

-	List(s) of contributors that ordered in descending order of contributing to the energy consumption.

-	When the EIF exposes the UE level energy consumption information to the AF, the EIF can provide an ordered list of applications that contribute to the energy consumption for the UE.

-	When the EIF exposes the PDU Session level energy consumption information to the AF, the EIF can provide an ordered list of applications that contribute to the energy consumption for the PDU Session.

NOTE:	This solution exposes the information about application only to AF(s) related to this application. For example, for Application-x1, Application-x2 and Application-x3 related to AF-x, the information on these applications described above is exposed only to AF-x while for Application-y1 and Application-y2 related to AF-y, the information on these applications described above is exposed only to AF-y.

6.Z.2	Procedures

6.Z.2.1	Procedure for enhanced energy related information exposure to AF

Figure 6.Z.2.1-1 shows the procedure for enhanced energy related information exposure to AF.

Figure 6.Z.2.1-1: Procedure for enhanced energy related information exposure to AF

1a.	Step 1a (Nnef_EventExposure_Subscribe request) in clause 4.29.1 (Energy Consumption information exposure) in TS 23.502 [3] with the following additional parameters in the request:

a)	Indication to request top contributors (optional): This indication is to request information about top contributors that contribute the most to the energy consumption. See bullet i) in step 6a for the related output information.

b)	Indication to request ordered list of contributors (optional): This indication is to request information about contributors that ordered in descending order of contributing to the energy consumption. See bullet ii) in step 6a for the related output information.

1b.	Step 1b (Neif_EventExposure_Subscribe request) in clause 4.29.1 (Energy Consumption information exposure) in TS 23.502 [3]. The additional parameters described in step 1a also applies to this step.

1c.	Step 1c (Neif_EventExposure_Subscribe request) in clause 4.29.1 (Energy Consumption information exposure) in TS 23.502 [3]. The additional parameters described in step 1a also applies to this step.

2a.	Step 2a (Neif_EventExposure_Subscribe response) in clause 4.29.1 (Energy Consumption information exposure) in TS 23.502 [3].

2b.	Step 2b (Nnef_EventExposure_Subscribe response) in clause 4.29.1 (Energy Consumption information exposure) in TS 23.502 [3].

2c.	Step 2c (Neif_EventExposure_Subscribe response) in clause 4.29.1 (Energy Consumption information exposure) in TS 23.502 [3].

3.	Steps 2 to 7 (Energy related information collection from 5GC NFs) in clause 4.29.2 (Energy Consumption information collection) in TS 23.502 [3].

4.	Step 10 (Energy related information collection from OAM) in clause 4.29.2 (Energy Consumption information collection) in TS 23.502 [3].

5.	The EIF performs the energy consumption information calculation as described in clause 5.51.2.3 of TS 23.501 [2].

6a.	Step 3a (Neif_EventExposure_Notify) in clause 4.29.1 (Energy Consumption information exposure) in TS 23.502 [3] with the following additional parameters in the notification:

i)	If the indication to request top contributors was provided by the AF in step 1a, the following information may be provided by the EIF. The list is ordered by descending order of contributing to the energy consumption.

-	If the energy consumption information requested for exposure is UE granularity (i.e. Event Filter is UE level), a list of applications that contribute the most to the energy consumption for the requested UE.

-	If the energy consumption information requested for exposure is PDU Session granularity (i.e. Event Filter is PDU Session level), a list of applications that contribute the most to the energy consumption for the requested PDU Session.

If all contributors contribute to the energy consumption in similar or same level, such information may be provided to the AF instead of the list of top contributors.

ii)	If the indication to request ordered list of contributors was provided by the AF in step 1a, the following information may be provided by the EIF.

-	If the energy consumption information requested for exposure is UE granularity (i.e. Event Filter is UE level), an ordered list of applications that contribute to the energy consumption for the requested UE.

-	If the energy consumption information requested for exposure is PDU Session granularity (i.e. Event Filter is PDU Session level), an ordered list of applications that contribute to the energy consumption for the requested PDU Session.

If all contributors contribute to the energy consumption in similar or same level, such information may be provided to the AF instead of the ordered list of contributors.

6b.	Step 3b (Nnef_EventExposure_Notify) in clause 4.29.1 (Energy Consumption information exposure) in TS 23.502 [3]. The additional parameters described in step 6a also applies to this step.

6c.	Step 3c (Neif_EventExposure_Notify) in clause 4.29.1 (Energy Consumption information exposure) in TS 23.502 [3]. The additional parameters described in step 6a also applies to this step. In the description in step 6a, step 1a is replaced by step 1c.

6.Z.3	Impacts on existing services, entities and interfaces

AF:

-	Support additional input parameters when subscribing energy related information exposure.

NEF:

-	Support additional input parameters when subscribing energy related information exposure.

-	Support additional output parameters when notifying energy related information exposure.

EIF:

-	Support to provide list(s) of top contributors that contribute the most to the energy consumption when notifying energy related information.

-	Support to provide list(s) of contributors that ordered in descending order of contributing to the energy consumption when notifying energy related information.

* * * * End of Changes * * * *