3GPP TSG-SA WG2 Meeting #169	S2-2505845

Fukuoka, Japan, May 19 – 23, 2025													(revision of S2-2505607)

Source:	CATT, KDDI, Toyota, InterDigital

Title:	KI#3 new solution for UP path adjustment based on energy related information

Document for:	Approval

Agenda Item:	20.4.1

Work Item / Release:	FS_EnergySys_Ph2 / Rel-20

Abstract of the contribution: The contribution proposes a new solution for UP path adjustment based on energy related information for KI#3.

1. Introduction

Transmitting data traffic via different UP paths may have different energy efficiency and/or energy consumption, as different UP paths involves different 5GC NFs (e.g. different PSA UPFs, I-UPFs and/or UL CL/BP UPFs) which may have different energy performance. Therefore, one possible way of improving energy efficiency and/or reducing energy consumption in the network is to adjust the UP paths for transmitting the service data flows (i.e. UP paths of the PDU Sessions) based on energy related information.

The DNAI identifies a user plane access to one or more DN(s) where applications are deployed, and different DNAIs can correspond to different UP paths for transmitting the service data flows of the applications. Thus the existing mechanism of DNAI (re)selection mechanism, e.g. AF influence on traffic routing, can be extended by taking energy related information into consideration to achieve UP path adjustment for energy efficiency and energy saving.

The energy related information to be taken into account for DNAI (re)selection can include the following:

-	total energy consumption of the UP path associated with the DNAI, i.e. sum of the energy consumption of the UPFs (e.g. PSA UPF terminating N6, I-UPF, UL CL/BP UPF) in the UP path, per PDU Session or per Service Data Flow, and

-	transmitted data volume over this UP path per PDU Session or per Service Data Flow; or

-	energy efficiency of the UP path associated with the DNAI, i.e. the transmitted data volume over this UP path divided by total energy consumption of the UP path associated with the DNAI, per PDU Session or per Service Data Flow.

How the 5GC NF (e.g. AF, PCF or the SMF) decides to perform UP path adjustment based on the above information is up to implementation.

Compared to NF (re)selection based on energy related information, UP path adjustment considers the overall energy consumption and/or energy efficiency in the CN for transmitting the service data flow, instead of a partial picture (i.e. energy consumption and/or energy efficiency of only one NF).

2. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-67.

* * * Start of Change * * * *

6.x	Solution #X: UP path adjustment based on energy related information

6.x.0	High-level solution principles

This solution proposes the following principles to be agreed:

-	The AF, PCF and/or SMF takes into account of energy related information for DNAI (re)selection to adjust the UP path of the PDU Session, during the AF influence on traffic routing procedure.

-	The AF, PCF and/or SMF subscribes to energy related information (e.g. energy consumption and corresponding data volume, energy efficiency, and/or renewable energy consumption ratio) of the UPFs from the EIF, and calculates energy related information per DNAI per PDU Session.

-	The AF/PCF selects/updates the DNAI list or the SMF selects the target DNAI based on the energy related information per DNAI per PDU Session, and the SMF reconfigure the user plane of the PDU Session towards the selected DNAI.

NOTE:	The adjustment of UP paths is determined based on operator policy, and frequent adjustment of UP paths for the PDU Session should be avoided.

6.x.1	Description

This solution addresses Key Issue #3.

Transmitting data traffic via different UP paths may have different energy efficiency and/or energy consumption, as different UP paths involves different 5GC NFs (e.g. different PSA UPFs, I-UPFs and/or UL CL/BP UPFs) which may have different energy performance. Therefore, one possible way of improving energy efficiency and/or reducing energy consumption in the network is to adjust the UP paths for transmitting the service data flows (i.e. UP paths of the PDU Sessions) based on energy related information.

The DNAI identifies a user plane access to one or more DN(s) where applications are deployed, and different DNAIs can correspond to different UP paths for transmitting the service data flows of the applications. Thus the existing mechanism of DNAI (re)selection mechanism, e.g. AF influence on traffic routing, can be extended by taking energy related information into consideration to achieve UP path adjustment for energy efficiency and energy saving.

As specified in clause 5.6.7 of TS 23.501 [2] and clause 4.3.6 of TS 23.502 [3], the AF may influence the DNAI selection by providing the list of DNAI(s) used by specific application(s), or the NEF can provide the mapping between the AF-Service-Identifier and a list of DNAI(s) based on local configuration when the DNAI(s) being used by the applications are statically defined. The list of DNAI(s) is then provided to the PCF (except in the case of HR SBO where Traffic Influence information is provided directly from V-NEF to V-SMF bypassing the PCF), and the PCF may update the list of DNAI(s), e.g. based on service experience analytics per UP path as defined in TS 23.288 [x]. Finally the PCF provides the list of DNAI(s) to the SMF and the SMF determines the target DNAI, using the target DNAI to trigger PDU Session modification or establishment to add, replace or remove UPF(s) (acting as UL CL/BP or PSA) in the UP path.

To support DNAI (re)selection based on energy related information, it is proposed to enhance the AF influence on traffic routing as following:

-	The AF may decide the list of DNAI(s) for the application(s) taking into account of energy related information, and provide the selected DNAI(s) with corresponding traffic steering policy identifier(s) to the NEF/PCF.

-	The PCF may update the DNAI(s) in the PCC rule taking into account of energy related information and operator policy.

-	The SMF may determine the target DNAI based on energy related information, and local configuration and/or the PCC rule (if available).

Editor's note:	The triggers for UP path adjustment are FFS.

The energy related information to be taken into account for DNAI (re)selection can include the following:

-	total energy consumption of the UP path associated with the DNAI, i.e. sum of the energy consumption of each UPF (including PSA UPF terminating N6, I-UPF and UL CL/BP UPF if any) involved in the UP path, per PDU Session or per Service Data Flow, and

-	transmitted data volume over this UP path per PDU Session or per Service Data Flow; or

-	energy efficiency of the UP path associated with the DNAI, i.e. the transmitted data volume over this UP path divided by total energy consumption of this UP path associated with the DNAI, per PDU Session or per Service Data Flow; or

-	renewable energy consumption ratio of the UP path associated with the DNAI.

NOTE:	UP paths towards different DNAIs of the same DNN and UP paths towards the same DNAI can be considered when evaluating the energy related information per DNAI.

The 5GC NF (i.e. AF, PCF or the SMF) can obtain or calculate the above information based on the notifications received from the EIF, i.e. Energy Consumption information in the CN at per UE per PDU Session and/or per UE per application (i.e. Service Data Flow) granularity, and the notifications received from the UPF(s) about the data volume.

NOTE:	How the 5GC NF (i.e. AF, PCF or the SMF) decides to perform UP path adjustment based on the above information is up to implementation.

Compared to NF (re)selection based on energy related information, UP path adjustment considers the overall energy consumption and/or energy efficiency in the CN for transmitting the service data flow, instead of considering energy consumption and/or energy efficiency of only one NF.

Editor's note:	How to select the UPF in case that a DNAI refers to multiple UPFs is FFS.

6.x.2	Procedures

Figure 6.y.2-1: Procedure UP path adjustment based on energy related information

0a.	The AF/PCF/SMF subscribes to energy related information (e.g. energy consumption and corresponding data volume, energy efficiency, and/or renewable energy consumption ratio) of the UPFs from the EIF using the Neif_EventExposure_Subcribe service. The PCF includes PCF ID, Energy Consumption as Event ID, and parameters reflecting the granularity request by the PCF (e.g., DNN/S-NSSAI, Application ID, Flow description(s)). The PCF includes reporting parameters such as reporting threshold/period.

NOTE:	The granularity requested by PCF to receive energy information from EIF is up to PCF implementation.

0b.	The AF/PCF/SMF receives notifications of energy related information with corresponding DNAI and PDU Session ID from the EIF for the requested granularity (e.g., per PDU Session, per application ID, per traffic flow).

1-7. An AF influence on traffic routing procedure is performed as described in clause 4.3.6.2 of TS 23.502 [3], with the enhancements as described as below:

-	Step 1, same as step 1 of figure 4.3.6.2-1 with the enhancement that if the AF is a trusted AF, the AF may decide the list of DNAI(s) for the application(s) taking into account of energy related information per DNAI, and provide the list of DNAI(s) to the NEF/PCF. The trusted AF calculates the energy related information per DNAI per PDU Session as described in clause 6.x.1 based on the notifications from the EIF.

-	Steps 2-4 are the same as step 2-4 of figure 4.3.6.2-1.

-	Step 5, same as step 5 of figure 4.3.6.2-1 with the following enhancement:

-	The PCF calculates the energy related information per DNAI per PDU Session as described in clause 6.x.1 based on the notifications from the EIF.

-	The PCF takes the energy related information per DNAI into account to determines if the DNAI is optimal, e.g. by comparing energy efficiency of the path to a certain threshold (e.g., configured at the PCF), and

updates the list of DNAI(s) in the traffic routing information of the traffic flow(s) in the PCC rule.

- The PCF sends a Npcf_SMPolicyControl_UpdateNotify request to the SMF with updated policy information about the PDU Session or traffic flow(s) (e.g., updated DNAI list).

-	Step 6, same as step 6 of figure 4.3.6.2-1 with the enhancement that the SMF may determine the target DNAI taking into account of energy related information per DNAI. The SMF calculates the energy related information per DNAI per PDU Session as described in clause 6.x.1 based on the notifications from the EIF.

-	Steps 7 is the same as step 7 of figure 4.3.6.2-1.

Editor's note:	Which NF, i.e. the AF, PCF or SMF, takes into account of energy related information per DNAI for DNAI selection is FFS.

6.x.3	Impacts on services, entities and interfaces

AF:

-	Subscribes to energy related information (e.g. energy consumption and corresponding data volume, energy efficiency, and/or renewable energy consumption ratio) of the UPFs from the EIF, and calculate the energy related information per DNAI per PDU Session based on the notifications from the EIF;

-	Decides the list of DNAI(s) for the application(s) taking into account of energy related information per DNAI.

PCF:

-	Subscribes to energy related information (e.g. energy consumption and corresponding data volume, energy efficiency, and/or renewable energy consumption ratio) of the UPFs from the EIF, and calculate the energy related information per DNAI per PDU Session based on the notifications from the EIF;

-	Updates the list of DNAI(s) taking into account of energy related information per DNAI.

SMF:

-	Subscribes to energy related information (e.g. energy consumption and corresponding data volume, energy efficiency, and/or renewable energy consumption ratio) of the UPFs from the EIF, and calculate the energy related information per DNAI per PDU Session based on the notifications from the EIF;

-	Determines the target DNAI taking into account of energy related information per DNAI.

* * * End of Change * * * *