3GPP TSG-SA WG2 Meeting #169	S2-2505855

Fukuoka, Japan, May 19 – 23, 2025	(Revision of S2-2505822, 5603,5064)

Source:	Samsung

Title:	NEF Service Update

Document for:	Approval

Agenda Item:	19.14.2

Work Item / Release:	AmbientIoT-ARC / Rel-19

Abstract of the contribution: The contribution proposes to update NEF Service for AIoT.

1. Discussion

NEF Service needs to be described in the normative text in TS 23.369 regarding the following editor’s notes.

Editor’s note:	It is FFS whether to use separate service operation to respectively support read, write or disable command procedure.

For the aboe EN, we can remove the EN as Command operation works with Command Type and AIoT data specific to each of the Command Type.

Editor’s note:	The NEF AIoT service operation definition needs to be revisited to align with AIOTF services.

For the above EN, we can add an optional output that indicates the message is the final message for the operation.

2. Proposal

It is proposed to agree the following changes to 3GPP TS 23.369.

* * * Start of Change * * * *

7.4	NEF services

7.4.1	General

In addition to those defined in clause 7.2.8 of TS 23.501 [3] and clause 5.2.6 of TS 23.502 [4], table 7.4.1-1 illustrates additional NEF services to support AIoT.

Table 7.4.1-1: NF Services provided by NEF

| Service Name | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| Nnef_AIoT | Inventory | Request/Response | AF |
|  | Command | Request/Response | AF |
|  | Notify | Subscribe/Notify | AF |

7.4.2	Nnef_AIoT_Inventory service operation

Service operation name: Nnef_AIoT_Inventory

Description: The consumer requests to perform an inventory operation for an AIoT Device or multiple AIoT Devices.

Input, Required:

1) AF ID

2) At least one of the following parameters are included:

- External Target Area information

- Either AIoT Device ID(s) or AIoT Device ID filter information for the inventory operation

3) Notification Endpoint.

Input, Optional: Approximate number of AIoT Devices, time interval.

Output, Required: AF Transaction ID, Result indication, Failure cause in case of Failure.

Output, Optional: None.

7.4.3	Nnef_AIoT_Command service operation

Service operation name: Nnef_AIoT_Command

Description: The consumer requests to perform a command operation for an AIoT Device or multiple AIoT Devices.

Input, Required:

1) AF ID

2)	At least one of the following parameters are included:

- External Target Area information

-Either AIoT Device ID(s) or AIoT Device ID filter information for the command operation

3) Notification Endpoint.

4)	Command type (Read, Write, or Disable).

Editor’s note:	The AIoT Data specific for the Read/Write/Disable command is FFS.

Input, Optional: Approximate number of AIoT Devices, Approximate message size from the AIoT Device for Read command type.

Editor’s note:	It is FFS whether the Approximate message size from the AIoT Device can also apply for the other command types.

Output, Required: AF Transaction ID, Result indication, Failure cause in case of Failure.

Output, Optional: None.

7.4.4	Nnef_AIoT_Notify service operation

Service operation name: Nnef_AIoT_Notify

Description: The consumer receives notification of the status or results of the requested service operation. If the consumer invokes the Nnef_AIoT_Inventory, or Nnef_AIoT_Command service operation, the consumer implicitly subscribes to the results of the requested service operation.

Input, Required:

1) AF Transaction ID

Input, Optional:

1) a list of AIoT Device ID(s), Failure Cause in case of Failure

2) Read command specific report information: Information obtained from each target AIoT Device.

3) The Last Report Indication, indicating the notify is the last notify for an AIoT service operation.

Output, Required: Result indication.

Output, Optional: None.

* * * End of Change * * * *