3GPP TSG-WG SA2 Meeting #169	S2-2505856

Fukuoka City, Fukuoka, JP, 19th May – 23rd May, 2025	(revision of S2-2505344, …, 5851)

Source:	Huawei, HiSilicon, Qualcomm Incorporated, Samsung, Deutsche Telekom, Lenovo

Title:	AIoT Device Memory and Associated Commands

Document for:	Approval

Agenda Item:	19.14.2

Work Item / Release:	AmbientIoT-ARC / Rel-19

Abstract: Addition of the support of writable memory in an AIoT Device and commands to access it.

# 1. Introduction/Discussion

As discussed in S2-2505343, an AIoT Device can support a memory that can store user data from an AF and the associated commands to read and write to that memory.

This pCR provides the changes necessary to describe that memory and complete the definition of those commands.

# 2. Text Proposal

It is proposed to capture the following changes vs. TR 23.369.

# * * * * First change * * * *

### 5.2.2	AIoT Command service

#### 5.2.2.1	Overview

Three types of Command service operations are supported: Read, Write and Disable.

An AF uses the Command service Read operation to retrieve information from AIoT Device(s), and the AIOTF uses the commands described in clause 5.2.2.2 towards the AIoT Device for the operation.

An AF uses the Command service Write operation to write information to AIoT Device(s), and the AIOTF uses the commands described in clause 5.2.2.2 towards the AIoT Device for the operation.

An AF uses the Command service Disable operation to permanently disable the capability of AIoT Device(s) to transmit RF signals, and the AIOTF uses the commands described in clause 5.2.2.3 towards the AIoT Device for the operation.

# * * * * Second change (all new) * * * *

#### 5.2.2.2	Read and Write Commands

It is optional for an AIoT Device to support of Read and Write Commands. If supported, an AIoT Device contains User Memory that can be used to store application specific data and is accessed using AIoT NAS commands from an AIOTF. An AIoT Device implementation may have other storage outside of the User Memory that is used to store e.g., AIoT Device Permanent Identifier, etc.

The User Memory is accessed by the following commands and responses between the AIOTF and AIoT Device:

-	Read Request: used to read application data from User Memory.

-	Read Response: Response from the AIoT Device to Read Request, providing a status and the application data read from the User Memory.

-	Write Request: used to write application data to User Memory.

-	Write Response: Response from the AIoT Device to Write Request, providing a status for the Write Request.

The Read Request and Response, and Write Request and Response commands are used to access the User Memory when an authorized AF uses the Read or Write command service operations, as described in clause 5.2.2.1. The Read or Write Request is sent in the Command Request step and the Read or Write Response is sent in the Command Response step of the Command procedure described in clause 6.2.3.

The physical memory map of an AIoT Device and where the User Memory is within it is implementation specific.

The commands to access the User Memory include an offset from the start of the User Memory to indicate where to read application data from and where to write application data to. The offset and the application data transferred in the Read Request, Write Request, and Read Response has no special meaning to the network, and the AIOTF or other NFs do not attempt to interpret them.

Figure 5.9.2-1: Logical AIoT Device User Memory

The Write Request command is used to write application data into the User Memory and includes:

-	the application data to write and its length, and

-	the offset where to write the application data.

The AIoT Device responds with a Write Response indicating whether the Write Request was successful.

The Read command is used to read from the User Memory and includes:

-	the offset to read application data from, and

-	the length of the application data to read.

The application data is from the User Memory in the place indicated by the offset and the length.

The AIoT Device responds with a Read Response including whether the read was successful and the application data read from the User Memory.

If an AIoT Device does not support Read Request or Write Request commands, or the parameters in the commands are invalid the AIoT Device shall reject the command.

#### 5.2.2.3	Permanent Disable Command

An AIoT Device may be permanently disabled. A permanently disabled AIoT Device does not respond to the Inventory Procedure, as described in clause 6.2.2.

An AIoT Device is permanently disabled by the Permanent Disable command sent by an AIOTF to the AIoT Device.

The Permanent Disable command is sent to an AIoT Device when an authorized AF uses the Permanent Disable command service operations as described in clause 5.2.2.1, or if the network determines to disable the AIoT Device. The Permanent Disable command is sent in the Command Request step and a response is sent in the Command Response step of the Command procedure described in clause 6.2.3. The AIoT Device responds indicating whether the Permanent Disable command was successful.

NOTE X:	The trigger conditions when a network determines to disable the AIoT Device depends on operator and implementation policy.

When the AIoT Device has received and verified a Permanent Disable command, it shall no longer respond to the inventory procedure. The AIoT Device responds indicating whether the Disable command was successful.

# * * * * Third change * * * *

### 6.2.3	Command Procedure

Editor's note:	Additional information in the steps, parameters, and their naming throughout the procedures requires alignment with other clauses and references adding as required.

Editor's note:	Alignment is required for how to document and describe Direct Connectivity and Indirect Connectivity options in the procedures.

Figure 6.2.3-1 depicts the command procedure.

The procedure focuses on the messages and parameters used for the communication between AIOTF and NG-RAN regardless of the path to access NG-RAN, see clause 4.2.2.1. The handling of the different communication paths is described in clause 6.2.4.

Figure 6.2.3-1: Command Procedure

1.	The AF sends the Nnef_AIoT_Command (in case of untrusted AF) Request (AF ID, Command Type, AIoT Device identification information, [External Target Area information],  [Approximate number of AIoT Devices], [Approximate D2R message size], [Command type specific parameters]) message to NEF.

The AIoT Device identification information may include one or more AIoT Device ID(s) or the filtering information which is used to associate with multiple AIoT devices.

Command Type provides the operation to be performed and the Command type specific parameters provides the required parameters for the operation. The service operations are described in clause 5.2.2.

2.	The NEF selects the AIOTF(s) as described in clause 5.3.1. If no AIOTF can be selected, the NEF rejects the AIoT Command request with an appropriate cause code and step 6 is performed before ending the procedure.

3.	The NEF sends Naiotf_AIoT_Command Request message (AF ID, Command Type, AIoT Device identification information, [Target area information], , [Approximate number of AIoT Devices], [Approximate D2R message size], [Command type specific parameters]) message to the selected AIOTF.

4.	The AIOTF receives the AIoT command operation request and checks the parameters included in the request. The AIOTF performs NG- RAN and optionally RAN Reader selection as specified in clause 5.3. If no NG-RAN or RAN Reader can be selected, the AIOTF rejects the AIoT Command request with an appropriate cause code.

The AIOTF generates a Correlation ID corresponding to this AF service operation request, and is used for the AIOTF to correlate the service operation responses to the request.

The AIOTF determines assistance information as described in clause 5.4.

The AIOTF performs AF authorization for AIoT command operation as described in clause 5.6.

The AIOTF performs AMF selection as described in clause 5.3.4.

5.	AIOTF sends the Naiotf_AIoT_Command Response message (accept or reject, [cause code]) to the NEF.

6.	NEF sends the Nnef_AIoT_Command Response message (accept or reject, [cause code]) to the AF. If the response was a reject the procedure stops here.

7.	Step 7 to step11 of procedure for Inventory specified in clause 6.2.2 are performed with the following clarifications:

-	In step 11, the AIOTF validates the results as specified in TS 33.369 [9], and determines whether the command should be sent to an AIoT Device, e.g., by checking the Target AIoT device information.

If none of successful Inventory response is received, Step 8 -11 is not performed and the AIOTF sends a failure report to the NEF in Step 12.

8.	For each successful Inventory response received, the AIOTF sends Command Request message (Correlation ID, [Reader ID], NAS Command Request, [Approximate D2R message size]) to the NG-RAN directly or as an AIoT Transfer Container via an AMF as specified in clause 6.2.4. The NAS Command Request message includes the AIoT data.

The AIOTF uses the Command Type and Command type specific parameters received in Step 3 to determine the NAS Command Request to send to the AIoT Device, as described in clause 5.2.2.

Editor's note:	What parameter(s) are used in the Command Request to enable NG-RAN node to target a specific AIoT Device requires coordination with RAN3.

Editor's note:	Additional information included in the NAS Command Request for security will be determined and aligned with SA WG3.

9.	The NG-RAN sends the AS R2D message (NAS Command Request) to the AIoT Device.

10.	The AIoT Device sends the AS D2R message (NAS Command Response) to the NG-RAN. The NAS Command Response message may include the AIoT data.

Editor's note:	The AS R2D message and AS D2R message will be aligned with RAN WG's specification.

Editor's note:	Additional information included in the NAS Command Response for security will be determined and aligned with SA WG3.

11.	The NG-RAN responds with a Command Response message (Correlation ID, Reader ID, NAS Command Response) to the AIOTF directly or as an AIoT Transfer Container via an AMF as specified in clause 6.2.4.

12.	The AIOTF reports the result of the AIoT Command request to the NEF by sending the Naiotf_AIoT_Command Notify message (AIoT Device ID(s), AF ID, AIoT data).

13.	The NEF informs the AF of the result of the AIoT_Command request by sending the Nnef_AIoT_Command Notify message (AIoT Device ID(s), AF ID, AIoT data).

# * * * * Fourth change * * * *

### 7.2.3	Naiotf_AIoT_Command service operation

Service operation name: Naiotf_AIoT_Command.

Description: The NF consumer requests a command operation for one or multiple AIoT Device(s).

Inputs, Required:

1)	AF ID.

2)	At least one of the following parameters are included:

-	Target area information for the command operation.

-	Information about the target AIoT Device(s):

-	either the AIoT Device ID(s) or the filtering information for multiple AIoT Devices.

4)	Command Type: Read, Write or Permanent Disable.

Inputs, Optional:

Information to be used for resource allocation:

-	Approximate number of AIoT Devices.

-	Approximate message size from the AIoT Device for Read Operation.

-	If the Command Type is Read, the offset to read application data from and the length of application data to read shall be included.

-	If the Command Type is Write, the offset where to write the application data, the application data to write and its length shall be included.

Outputs, Required: Transaction ID, Result indication (Success or Failure), Failure Cause in case of Failure.

Outputs, Optional: None.

# * * * * Fifth change * * * *

### 7.4.3	Nnef_AIoT_Command service operation

Service operation name: Nnef_AIoT_Command

Description: The consumer requests to perform a command operation for an AIoT Device or multiple AIoT Devices.

Input, Required: AF ID, Target area information or AIoT Device ID(s) or AIoT Device ID filter information for the command operation, Command type (Read, Write, or Permanent Disable).

Input, Optional: Approximate number of AIoT Devices, if the Command Type is Read, the offset to read application data from and the length of application data to read shall be included, if the Command Type is Write the offset where to write the application data, the application data to write and its length shall be included, Approximate message size from the AIoT Device for Read command type.

Output, Required: AF Transaction ID, Result indication, Failure cause in case of Failure.

Output, Optional: None.

# * * * * End of changes * * * *