3GPP TSG-SA2 Meeting #169	S2-2505879

Fukuoka, Japan, May 19-23, 2025	                                                             (wasS2-2505625)

Source:	Futurewei

Title:	KI#3, New Sol: NRF based Sensing Function Registration, Discovery and Selection Procedures for Trusted and Untrusted AF

Document for:	Approval

Agenda Item:	20.2.1

Work Item / Release:	FS_Sensing_ARC / Rel-20

Abstract: Sensing Function Registration, Discovery and Selection

# Discussion

This paper proposes a solution for KI#3: Sensing Entity and Sensing Function Discovery and (Re-)Selection. The proposed solution is focused on registration, discovery and selection of sensing function.

# Proposal

It is proposed to capture below solution in TR 23.700-14.

# * * * * First change * * * *

# 6	Solutions

## 6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

|  | Key Issues |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Solutions | 1 | 2 | 3 | 4 | 5 | 6 |
| X |  |  | X |  |  |  |

# * * * * Second change (All Text New) * * * *

## 6.X	Solution #X: NRF based Sensing Function Registration, Discovery and Selection Procedures for Trusted and Untrusted AF

## 6.X.1	High level principles

The main principles of the proposed solution include:

- In the proposed procedure, the sensing function selection and discovery is initiated by a service request from trusted/untrusted AF.

- Two separate sensing function registration, discovery and selection procedures are proposed for the case of trusted and untrusted AF.

- The proposed procedures include required steps for sensing function registration leveraging existing NRF service operations for NF registration while introducing new parameters as part of sensing function profile.

- The proposed procedures include required steps for sensing function discovery and selection, which is performed by NEF in case of an untrusted AF.

### 6.X.2	Description

This paper proposes a solution for KI#3: Sensing Entity and Sensing Function Discovery and (Re-)Selection. The proposed solution is focused on registration, discovery and selection of sensing function.

### 6.X.3	Procedures

### 6.X.3.1 Sensing Function Registration, Discovery and Selection when sensing service is requested by a trusted AF

The detailed procedure of sensing function registration, discovery and selection for trusted AF is shown in Figure 6.X.3.1-1.

Figure 6.X.3.1-1: Sensing Function Registration, Discovery and Selection procedure when sensing service is requested by a trusted AF.

1. Sensing function may register to NRF, by invoking the Nnrf_NFManagment_NFRegister request service operation with its NF profile, which includes:

- NF Type (i.e. Sensing function type),

- Supported sensing service areas (serving areas for which sensing function has the sensing result/data),

- Supported sensing modes (sensing modes for which sensing function has the sensing result/data),

- Supported sensing types (e.g., tracking, detection).

NOTE: Sensing function per terminology is a logical function which is involved to support Sensing Service, and it can be either a new function or co-located with one of the existing functions.

Editor’s Note: Whether and how all parameters listed above (including supported sensing modes, supported sensing service areas and supported sensing types) are relevant to sensing function registration, discovery and selection is FFS.

2. NRF may receive the registration request from sensing function and store its NF profile.

3. NRF may send a registration response to the sensing function by invoking Nnrf_NFManagment_NFRegister response.

4-5. AF may discover sensing function(s) from the NRF by invoking the Nnrf_NFDiscovery_Request.

6. AF may select sensing function(s), e.g., by matching sensing functions profile to its selection criteria such as a Sensing Service Area of Interest.

### 6.X.3.2 Sensing Function Registration, Discovery and Selection when sensing service is requested by an untrusted AF

### The detailed procedure of sensing function registration, discovery and selection for untrusted AF is shown in Figure 6.X.3.2-1.

Figure 6.X.3.2-1: Sensing Function Registration, Discovery and Selection procedure when sensing service is requested by an untrusted AF.

1. Sensing function may register to NRF, by invoking the Nnrf_NFManagment_NFRegister request service operation, with its NF profile, which includes:

- NF Type (i.e. Sensing function type),

- Supported sensing service areas (serving areas for which sensing function has the sensing result/data),

- Supported sensing modes (sensing modes for which sensing function has the sensing result/data),

- Supported sensing types (e.g., tracking, detection).

Editor’s Note: Whether and how all parameters listed above (including supported sensing modes, supported sensing service areas and supported sensing types) are relevant to sensing function registration, discovery and selection is FFS.

NOTE: Sensing function per terminology is a logical function which is involved to support Sensing Service and it can be either a new function or co-located with one of the existing functions.

2. NRF may receive the registration request from sensing function and store its NF profile.

3. NRF may send a registration response to sensing function by invoking Nnrf_NFManagment_NFRegister response.

4. AF may send a sensing service request to NEF including some selection criteria such as Sensing Service Time Period of Interest and Sensing Service Area of interest.

5. NEF may check based on configured policies whether the AF is entitled to request a sensing service.

6-7. NEF may discover sensing function(s) on behalf of the AF from the NRF by invoking the Nnrf_NFDiscovery_Request using the selection criteria provided by the AF as defined in step 4.

8. NEF may select sensing function(s), e.g. by matching sensing functions profile to the selection criteria received from the AF. In this step, the NEF may assign a sensing coordination ID to each AF sensing request. The NEF may also store the sensing coordination ID(s).

9. NEF may send sensing service response only including the sensing coordination ID as assigned in step 8 and information on how they relate to corresponding selection criteria provided by the AF.

10. AF may store sensing coordination ID(s) and use it in subsequent interactions with the NEF for the requested sensing service.

6.X.4	Impacts on Services, Entities and Interfaces

NRF:

-	Support registration for Sensing Function profile.

NEF:

-	Support Sensing Function discovery and selection.

Editor’s Note: Details for impacts on other services, entities (including NRF and NEF) and interfaces are FFS.

# * * * * End of changes * * * *