3GPP TSG-WG SA2 Meeting #169	S2-2505958

# Fukuoka, Japan, 19th May – 23rd May, 2025		(revision of S2-2505703)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.228 | CR | 1627 | rev | 4 | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on standalone data channel description |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NG_RTC_Ph2 |  |  |  |  |  | Date: |  |  | 2025-03-24 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The current description of the scenarios for each of the procedure is not accurate and concise. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update the general description of each procedure. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incorrection specification |  |  |  |  |  |  |  |  |
| Clauses affected: |  | AC.10.2.1, AC.10.2.2, AC.10.2.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * Start of Changes * * *

## AC.10.2	Procedures

### AC.10.2.1	Originating Standalone Bootstrap DC Setup using PSI

The originating standalone Bootstrap DC setup using PSI is triggered by a UE for downloading application list and/or applications from DCSF. In this procedure, a PSI is configured in the UE for that purpose.

After the standalone bootstrap DC is established, the UE downloads the application list and/or applications via the bootstrap DC. The application list is used for selection and downloading data channel application. The data channel application is used for initiating standalone application DC in a subsequent IMS data channel session. The UE terminates the standalone IMS data channel session when the session timer expires.

NOTE:	Whether and when to initiate the standalone Bootstrap DC setup using PSI is up to UE implementation.

Figure AC.10.2.1-1 depicts a signalling flow diagram for originating standalone bootstrap data channel setup using PSI.

AC.10.2.1-1: Originating Standalone Bootstrap DC Setup using PSI

1.	UE#1 sends the SIP INVITE request with an initial SDP offer to the IMS AS, through P-CSCF and S-CSCF in the originating network. The Request URI of the SIP INVITE message is a specific PSI indicating standalone bootstrap DC. The initial SDP offer includes only the bootstrap data channel establishment request with bootstrap DC stream ID. The SIP precondition is not needed.

NOTE 1:	The PSI is pre-configured in the UE#1. How and when the PSI is pre-configured is left to implementation.

NOTE 2:	UE#1 can perform step 1 at any time before initiating a standalone application data channel, such as, immediately after completing the SIP registration procedure for downloading the list of application information and applications in order to save the duration of consequence (standalone) IMS DC session establishment.

2-4.	Steps 2-4 of clause AC.7.1 apply.

5.	The DCSF, based on the value of the Called ID (i.e. the specific PSI) and the content of the initial SDP offer, determines that the UE#1 requests to establish a standalone bootstrap DC and reserves only originating side MDC1 media information, which is used to receive the UE request for application downloading from the MF.

6.	The DCSF invokes the Nimsas_MediaControl_MediaInstruction (Session ID, Media Instruction Set) service operation based on its policies instructing the IMS AS how to set up bootstrap data channel with the MF for originating side and to terminate the session. The MediaInstructionSet provided by the DSCF, includes its MDC1 media endpoint addresses created in step 5, DC Stream ID, and the replacement HTTP URL representing the application list offered via the MDC1 interface.

7.	Step 7 of clause AC.7.1 applies.

8.	The IMS AS invokes Nmf_MRM_Create (List of Media Termination Descriptors) service operation to instruct the MF to allocate required data channel media resources for originating side only. The IMS AS requests creation of only one Media Termination which representing the local bootstrap media to be terminated.

9-10.	Steps 9-10 of clause AC.7.1 apply.

11.	The IMS AS does not forward the SIP INVITE request based on the specific Request URI and sends the SIP 18X response with the SDP answer through S-CSCF and P-CSCF to UE#1.

12-13.	The standalone IMS data channel session with only bootstrap data channel is answered, and bootstrap data channel between the originating MF and UE#1 is established.

14.	The UE#1 sends application request messages to the MF to request a data channel application or an application list if multiple DC applications are available, via the established bootstrap data channel. The MF forwards the message to the received media point of DCSF. The DCSF provides the application list and proper data channel applications further to the UE#1 based on their data channel capabilities and their choices through MF.

15-16.	If the established bootstrap DC is not used for a period of time (expiry of the session timer), the UE#1 sends SIP BYE request to release the bootstrap DC.

* * * Next Change* * *

### AC.10.2.2	Originating Standalone IMS DC Session if Application not Available in Originating UE

When an originating UE initiates a standalone IMS data channel session and the DC application is not available in the originating UE, the originating UE may initiate standalone bootstrap DC setup towards peer UE. The originating UE may update the IMS session to add application DC after downloading the data channel application.

If the DC application is not downloaded yet by the terminating UE, the terminating UE shall not accept the addition until the DC application is downloaded via the bootstrap DC as specified in AC.7.2.1 or AC.7.2.3.. If the terminating UE did not successfully download the DC Application, the terminating UE, as an example, does not alert the user and tear down the entires IMS session.

Figure AC.10.2.2-1 depicts a signalling flow diagram for originating standalone IMS DC Session if application is not available in the originating UE with the assumption that application list is available in the originating UE.

Figure AC.10.2.2-1: Originating Standalone IMS DC Session if Application not Available in Originating UE

The steps in the call flow are as follows:

1.	The UE#1 sends an initial SIP INVITE request with an initial SDP offer including media component for bootstrap data channel for originating side and optional for terminating side. The SIP precondition is not needed. The originating P-CSCF may include the P-Early-Media header field indicating support of early media authorization in the initial SIP INVITE request if not exists.

2-13.	Steps 2-13 of clause AC.7.1 apply. In this scenario, the SDP offer for bootstrap data channel to UE#2 are included.

14.	The UE#2 returns a SIP 18X response with an SDP answer for bootstrap data channel. The IMS AS includes the P-Early-Media header field indicating early media allowed in the SIP 18X response based on the SDP answer not indicating any application data channel. Simultaneously, the UE#2 may alert the terminating user the incoming session is a standalone IMS data channel session that DC application will be indicated later.

15.	Step 21 of clause AC.7.1 applies, except that the UE#1 may download the application list.

16.	The UE#1 sends a SIP PRACK request towards the terminating side before or during performing step 15, and the UE#2 returns a SIP 200 OK response for the PRACK. After the DC application has been downloaded, the UE#1 initiates the SIP UPDATE request to update the IMS session for adding the application DC and to inform the UE#2 the associated DC application binding information.

The IMS AS, the DCSF, and the MF repeat steps 3-10 with the media information of bootstrap data channel and application data channel, optionally including allocation of MDC2 resources for originating and terminating side. The IMS AS sends the SIP UPDATE request with the modified SDP offer including media component for bootstrap data channel and application data channel towards the terminating network and the UE#2. The IMS AS includes the P-Early-Media header field indicating early media allowed in the SIP UPDATE request based on the previously received SDP answer not indicating any application data channel.

17.	The bootstrap data channels have been established between the originating MF and the UE#2. If the DC application is not available in the UE#2, the UE#2 may alert the terminating user for DC application downloading. The UE#2 sends application request messages to the MF to request downloading the DC application and application list if needed.

18-23.	When the DC application is available in the UE#2, steps 15-20 of clause AC.7.1 apply with the difference that bootstrap data channel and application data channel established is indicated. The IMS AS may include the P-Early-Media header field indicating early media allowed in the SIP 200 response for the UPDATE forwarded to the originating side.

24.	The UE#2 alerts the terminating user for running the DC application. The UE#2 returns a SIP 180 response to indicate the UE#2 is ringing.

25.	Subsequent procedures continue. When the terminating user accepts to run the DC application for communication, the UE#2 answers the IMS session.

* * * Next Change* * *

### AC.10.2.3	Originating Standalone IMS DC Session if Application Available in Originating UE

The originating UE may initiate combined bootstrap DC and application DC if the selected DC application is locally available in the originating UE.

If the selected DC application is not downloaded yet by the terminating UE, the terminating UE will not establish the application DC and indicate the originating UE the DC application is desired to be downloaded. The originating UE further updates the standalone IMS data channel session to add the selected application DC. If the terminating UE did not successfully download the DC Application, the terminating UE, as an example, does not alert the user and tear down the entire IMS session.

Figure AC.10.2.3-1 depicts a signalling flow diagram for originating standalone IMS DC Session if application is available in the originating UE.

Figure AC.10.2.3-1: Originating Standalone IMS DC Session if Application Available in Originating UE

The steps in the call flow are as follows:

1.	The UE#1 sends the initial SIP INVITE request with an initial SDP offer including the media information of the bootstrap DC and application DC, as well as the associated DC application binding information. The SIP precondition is not needed. The originating P-CSCF may include the P-Early-Media header field indicating support of early media authorization in the initial SIP INVITE request if not exists.

2.	The IMS AS determines whether the IMS session is a standalone IMS data channel session, and determines if the session is allowed to be established based on local configuration or subscription data. If standalone IMS data channel session is not allowed, the IMS AS rejects the request, otherwise the IMS AS selects a DCSF for this user based on local configuration or discovery and selection of a DCSF instance via NRF.

3-10.	Steps 3-10 of clause AC.7.1 apply with the media information of bootstrap data channel and application data channel, optionally including allocation of MDC2 resources for originating and terminating side.

11-13.	Steps 11-13 of clause AC.7.1 apply. In this scenario, the SDP offer for bootstrap data channel, application data channel, and the associated DC application binding information to UE#2 are included.

14.	The UE#2 returns a SIP 18X response with the SDP answer for bootstrap data channel and application data channel. If the DC application is not available in the UE#2, the UE#2 may alert the terminating user for DC application downloading. The UE#2 accept the bootstrap data channel and reject application data channel to indicate that the DC application is not available but desired to be downloaded in the SDP answer if needed.

15.	The bootstrap data channel has been established between originating MF and the UE#1.

16.	The bootstrap data channel has been established between originating MF and the UE#2.

17.	The UE#1 sends a SIP PRACK request towards the terminating side before or during performing step 15. If the indication that DC application is desired to be downloaded is received, the UE#1 will consequently send a SIP UPDATE request to update the IMS session for adding the selected application data channel. The IMS AS includes the P-Early-Media header field indicating early media allowed in the SIP UPDATE request forwarded to the terminating side based on the indication.

18.	The UE#2 sends application request messages to MF to request downloading the DC application and application list if needed.

19-24.	If the DC application does not need to be downloaded in the UE#2, the UE#2 returns a 200 response for the PRACK. Otherwise, after the DC application successfully downloaded, steps 15-20 of clause AC.7.1 apply with the difference that bootstrap data channel and application data channel established is indicated. The IMS AS may include the P-Early-Media header field indicating early media allowed in the SIP 200 response for the UPDATE forwarded to the originating side.

25.	The UE#2 alerts the terminating user for running the DC application. The UE#2 returns a SIP 180 response to indicate the UE#2 is ringing.

27.	Subsequent procedures continue. When the terminating user accepts to run the DC application for communication, the UE#2 answers the IMS session.

NOTE:	This procedure covers only the scenario of UE#1 selects a DC application to communicated with UE#2. The bootstrap DC is established only with DCSF of UE#1. If the UE#2 wants to add another DC application of UE#2, another bootstrap DC with DCSF of UE#2 may also be established by UE#2 by sending SIP reINVITE message.

* * * End of Changes * * *