TSG-SA WG2 Meeting #169	S2-2505962

# Fukuoka, Japan, May 19 – 23, 2025                        (was S2-2505708, S2-2505707, S2-2505233)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 6324 | rev | 3 | Current version: | 19.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | N2 interface enhancement for downlink data suspension/resumption |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GSAT_Ph3-ARC |  |  |  |  |  | Date: |  |  | 2025-05-22 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In R3-252454 and R3-252409 approved in RAN3 meeting#127bis “If the Downlink NG Transmission Control IE is included in the RAN CONFIGURATION UPDATE message, the AMF shall, if supported, suspend or resume the downlink NG transmission accordingly.” “For Regenerative payload, an indication to suspend and resume data and signaling procedures over NG interface may be sent to AMF from gNB in the NGAP Configuration Update message when the Hard Feeder Link switch occurs.” To align with agreements in RAN3, which specify that the gNB may indicate suspension or resumption of downlink transmission to the AMF during hard feeder link switchover in regenerative payload scenario. This addition ensures consistent interpretation of existing procedures and supports robust NG interface management in NTN deployments. However, the specific solution is FFS depends on the future enhancement of RAN3. It is proposed: <1>The steps 2-4 of clause 4.8.1.2 in TS 23.502 are performed to suspend the downlink S1-U data transmission Figure 4.8.1.2-1: NG-RAN initiated Connection Suspend procedure <2> the steps 5-7 of clause 4.8.2.3 in TS 23.502 are performed to resume the downlink S1-U data transmission Figure 4.8.2.3-1: Connection Resume in CM-IDLE with Suspend |  |  |  |  |  |  |  |  |
| Summary of change: |  | Enhancement regarding use the RAN Configuration Update procedure for suspending or resuming downlink user plane transmission during hard feeder link switchover in regenerative satellite payload scenarios |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Inconsistencies with RAN3 SPEC. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.4.11.9 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS/TR 38.413 CR ... ,TS38.300 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

5.4.11.9	N2 interface and connection management for regenerative satellite payload

The NG Removal procedure defined in TS 38.413 [34] can be used to remove the interface between a gNB and an AMF in a controlled manner, e.g. when the gNB is leaving the service area of an AMF when using regenerative satellite payload.

If the RAN Configuration Update message as specified in TS 38.413 [34] is sent by a gNB to the AMF to suspend or resume the downlink data transmission for hard feeder link switchover scenario as defined in TS 38.300 [27]. For each PDU Session associated with UEs accessing the AMF via the gNB, the existing procedure can be used, e.g. steps 2-4 of clause 4.8.1.2 in TS 23.502 [3] are performed to suspend the downlink S1-U data transmission and the steps 5-7 of clause 4.8.2.3 in TS 23.502 [3] are performed to resume the downlink S1-U data transmission.

The serving AMF for a UE can remain the same during feeder link switchover.

NOTE 1:	If a gNB IP address changes due to feeder link switchover, then the procedures described in e.g. clause 5.21.1 can be used.

NOTE 2:	RAN transitioning between different NTN GWs, TAIs and potentially between AMFs can be supported by using existing procedures, e.g. the gNB releasing UEs to CM-IDLE before changing NTN gateway, or by using existing load re-balancing, triggering handover when it realizes it is going to leave a tracking area due to its own movement, etc.

# * * * * End of change * * * *