3GPP TSG-SA2 Meeting #169	S2-2506066

Fukuoka, JP, 19-23 May, 2025	was S2-2505816/5619/4892

Source:	China Mobile, Nokia, Nokia Shanghai Bell, Lenovo, KDDI

Title:	New solution for KI#2 Service authorization and revocation

Document for:	Approval

Agenda Item:	20.2.1

Work Item / Release:	FS_Sensing_ARC / Rel-20

Abstract: the KI#2 Solution on Service authorization and revocation are proposed for FS_Sensing_ARC.

1. Introduction

This pCR proposes the KI#2 Solution on Service authorization and revocation.

2. Text Proposal

It is proposed to capture the following new solution of KI#2 in TR 23.700-14.

# * * * * First change * * * *

6	Solutions

6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

|  | Key Issues |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Solutions | 1 | 2 | 3 | 4 | 5 | 6 |
| X |  | X |  |  |  |  |

# * * * * Second change (All text new) * * * *

- X	Solution #X: sensing service authorization and revocation

6.X.1	High level principles

Sensing Function splits into Sensing Control Function (SCF) and the Sensing Processing Function (SPF). SCF is responsible for sensing service control, while SPF is responsible for sensing data collection and processing, etc.

Regarding the sensing service authorization, when the AF decides to trigger the sensing service, SCF will perform authorization for that AF to make sure that only authorized AF can make usage of the specific sensing service. With regard to the sensing service revocation, when the AF determines to stop the specific sensing service due to some reason, e.g., saving resource, the AF can send message to request the SCF to cancel the relevant sensing service, which leads to SCF release the sensing resource, including the sensing entity referring to a Sensing Transmitter or to a Sensing Receiver.

The authorization of sensing entities shall contain following options:

-	sensing entities authorized as sensing transmitters

-	sensing entities authorized as sensing receivers

-	sensing entities authorized as both sensing transmitters and receivers

6.X.2	Description

This solution aims to address key issue#2: Authorization and Revocation to Support Sensing Service triggered by the AF via invoking the sensing service.

NOTE x: 	In this solution, for trusted domain, the AF can trigger the sensing service authorization by directly sending the request to the SCF without going via the NEF.

6.X.3	Procedure for sensing service authorization

Figure 6.X.3-1: procedure for sensing service authorization.

Precondition:

- The sensing entities are registered to the SCF including its sensing capabilities.

Editor’s NOTE: How the sensing entity registers its sensing related info to the SCF is FFS.

The sensing service authorization steps are shown as follows:

1a. The untrusted AF may send Nnef_SensingService request message to the NEF to obtain sensing result from the network, which may include some sensing service parameters, e.g. target sensing service area, sensing service requirement and sensing operation parameters and so on. sensing service requirement may include sensing accuracy, sensing resolution, confidence level, missed detection probability, false alarm probability, max sensing service latency, refreshing rate etc. Sensing operation parameters may include on-demand report/periodical report/event-triggered report.

1b. The trusted AF may send Nnef_SensingService request message to the SCF to obtain sensing result from the network, which may include some sensing service parameters, e.g. target sensing service area, sensing service requirement, sensing operation parameters and so on.

2a. For the request from untrusted AF, the NEF authorizes the AF request of the sensing service and forward the request.

3a. The NEF sends the Nscf_ SensingService request message to the SCF. The request includes some sensing service parameters, e.g. target sensing service area, on-demand report/periodical report/event-triggered report, etc, which are from in step 1. The NEF may perform SCF discovery and selection before sending the message to the SCF, which is to be addressed by KI#3 solution.

- After receiving the request, the SCF performs authorization based on the sensing entities’ capabilities and the sensing service context.

- Editor’s NOTE: The further details about authorization of sensing service request and sensing entities is FFS.

5a. For the untrusted AF, the SCF firstly responds result of the sensing service by sending Nscf_ SensingService response message to NEF.

5b. For the trusted AF, the SCF responds result of the sensing service by sending Nscf_ SensingService response message directly.

6a. The NEF sends Nnef_ SensingService response message to the AF.

6.X.4	Procedure for sensing service revocation triggered by the AF

Figure 6.X.4-1: procedure for ongoing sensing service revocation

1a. When determining to revoke the sensing service, the untrusted AF sends Nnef_ SensingService revocation request message to the NEF.

- Editor’s NOTE: Criteria for sensing service revocation used by the AF is FFS.

- Editor’s NOTE: Whether authorization of sensing entities are revoked when the sensing service is cancelled is FFS.

1b. When determining to revoke the sensing service, the trusted AF sends Nnef_ SensingService revocation request message to SCF.

2a. The NEF sends the Nscf_ SensingService_ revocation request message to the SCF to terminate the sensing operation.

- The SCF performs the sensing revocation procedure to cancel the ongoing sensing service.

4a. The SCF sends Nscf_ SensingService revocation response message to the NEF indicating the service has been revoked.

4b. The SCF sends Nscf_ SensingService revocation response message directly to the trusted AF indicating the service has been revoked.

5a. The NEF sends Nnef_ SensingService revocation response message to the untrusted AF indicating the sensing service has been revoked.

6.X.5	Impacts on services, entities and interfaces

SCF:

-	Supports to perform sensing service authorization and/or revocation triggered by the AF.

NEF:

-	Supports to perform sensing service authorization and/or revocation triggered by the AF.

# * * * * End of changes * * * *