SA WG2 Meeting #S2-169	S2-2504939

19 - 23 May, 2025, Fukuoka, Japan

Title:	Draft LS on Average Window for Alternative QoS

Response to:	S2-2504520/ R3-252397

Release:	REL-18

Work Item:	XRM

Source:	SA2 (CATT)

To:	RAN3

Cc:	-

Contact person:	Chunshan Xiong (chunshan.xiong@cictmobile.com)

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	-

1	Overall description

SA2 thanks for the LS from the RAN3, SA2 has the following answer for the LS:

RAN3 could not reach consensus on introducing averaging window in the alternative QoS Profile, and respectfully requests SA2 to provide clarification on introducing averaging window in the alternative QoS Profile.

[SA2 Answer:] The Understanding 2 is aligned with the averaging window of the AQP (alternative QoS Profile). There are a lot of benefits to introducing averaging window with different values in different AQP (alternative QoS Profile). E.g., the different AQP can provide different GFBR and MFBR, and different GFBR and MFBR can be used by the different codecs in the AS media. e.g., high FPS (frames per Second) video (e.g., 120 FPS) normally needs more bitrate than the low FPS video (e.g., 30 FPS). For the low 30 FPS video, a 2000ms default averaging window can provide 60 times of measurement of GFBR/MFBR in the NG-RAN, but for the high 120 FPS video, the 60 times of measurement of GFBR/MFBR only need a 500ms  averaging window. In such cases, the different FPS videos can have different averaging windows with the same measurement times for the GFBR and MFMR.

The high FPS video can change more quickly in codec, the short averaging window can provide QoS Notification message in more shorter time than the low FPS video (but with the same measure time of GFBR/MFBR) to the AS. In such a short averaging window for the high FPS can provide better adaptation to the bitrate changes in the NG-RAN.

2. Actions:

To RAN3

ACTION: 	SA2 kindly asks RAN3 to take the above information into account and update the related TS.

3. Dates of Next SA2 Meetings:

SA2#170	25th – 29th August 2025 		 Goteborg, SE

SA2#171		13th – 17th October 2025 		 TBD, CN