3GPP TSG-SA2 Meeting #169	S2-2505055

# Fukuoka, Japan, May 19 – May 23, 2025	(revision of S2-2503416)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 6217 | rev | 2 | Current version: | 19.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | TAIs configured by OAM |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | vivo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GSAT_Ph3-ARC |  |  |  |  |  | Date: |  |  | 2025-03-28 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Per RAN3 reply LS R3-250895, RAN3 further clarifies the benefits on OAM configured TAIs in AMF, but also confirm the existing NG signalling procedures for supported TAI list update are expected to work as legacy. When any potential discrepancy between OAM configuration and parameters received via NG signalling, if present, is expected to be resolved via implementation. So for clarification, in SA2 specification it should capture the case of OAM configured TAIs in regenerative payload. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the aggreement of RAN3 that “The AMF may be configured the supported TAI list of the gNB onboard the satellite by OAM and shares the same configuration on supported TAI list with the gNB onboard the satellite” Further clarify that: “The existing NG signalling procedures for supported TAI list update as described in TS 38.413 [34] are still valid, and it is under OAM control whether the gNB further needs to provide its supported TAI list via NG signalling to AMF if the supported TAI list of the gNB has been provisioned to AMF by OAM. When any potential discrepancy between OAM configuration and TAIs received via NG signalling, if present, is expected to be resolved via implementation.” |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It is not aligned with RAN3 agreements of OAM configured TAIs in regenerative payload. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.4.11.9 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * * Start of 1st Change * * * *

5.4.11.9	N2 interface and connection management for regenerative satellite payload

The NG Removal procedure defined in TS 38.413 [34] can be used to remove the interface between a gNB and an AMF in a controlled manner, e.g. when the gNB is leaving the service area of an AMF when using regenerative satellite payload.

The serving AMF for a UE can remain the same during feeder link switchover.

The AMF may be configured the supported TAI list of the gNB onboard the satellite by OAM and shares the same configuration on supported TAI list with the gNB onboard the satellite.

NOTE 1:	If a gNB IP address changes due to feeder link switchover, then the procedures described in e.g. clause 5.21.1 can be used.

NOTE 2:	RAN transitioning between different NTN GWs, TAIs and potentially between AMFs can be supported by using existing procedures, e.g. the gNB releasing UEs to CM-IDLE before changing NTN gateway, or by using existing load re-balancing, triggering handover when it realizes it is going to leave a tracking area due to its own movement, etc.

NOTE 3:	The existing NG signalling procedures for supported TAI list update as described in TS 38.413 [34] are still valid, and it is under OAM control whether the gNB further needs to provide its supported TAI list via NG signalling to AMF if the supported TAI list of the gNB has been provisioned to AMF by OAM. When any potential discrepancy between OAM configuration and TAIs received via NG signalling, if present, is expected to be resolved via implementation.

* * * * End of Changes * * * *