SA WG2 Meeting #170	S2-2506525

25 – 29 Aug, 2025, Goteborg , SE

Source:	Ofinno

Title:	[WT#2] Interworking and migration in 6G

Document for:	Agreement

Agenda Item:	20.6.2

Work Item / Release:	FS_6G_ARC/Rel-20

Abstract of the contribution: This paper proposes the WT scope and introduces a new KI associated with WT#2 to support interworking and migration in 6G.

# 1. Justifications

This contribution proposes the WT scope and a key issue to work on the WT#2 of the approved 6G SID (SP-250806):

WT#2: Study migration and interworking, including - How to support migration to 6GS - How to support interworking with 5GS - Whether and how to support interworking with EPS - How to support interworking between 6GS and 4G/5G NTN/satellite access that use EPS/5GS. NOTE 2: Interworking with 2G/3G are not considered in this study. Interworking between 6GS and 5GS is assumed to work even if the UE has previously registered in 2G, 3G or 4G. NOTE 3: The detailed migration study scope will be coordinated and aligned with RAN.

Architectural Motivation:

As the expecting performance requirements are higher, smoother interworking and flawless service continuity with legacy network is a must-have prerequisite for the 6G. At the same time, operator cost-efficient network design is required and this should be reflected for the 6G design principles. Consequently, the 6G architecture design must reconcile two conflicting requirements of the efficient interworking and migration.

An example, the 6G core may be a common core (e.g., supporting 6G access and 5G access) and provide backward compatibility to a legacy access (e.g., NR). For example, the 6G core provides a new interface for 6G RAN (e.g., xNB) and may provide N2/N3 interface for gNB. From 6G access, the 6G core connectivity is still standalone. As the 6G core will be an evolution from 5G core and continue to follow the design principle of 5G (e.g., cloud native, SBA, modular design), the common core approach may provide the best fit for the two conflicting requirements.

Another example, the 6G core will be designed based on dual registration concept. For 5G, the dual registration wasn’t mature and failed in the market. However, 6G era will be different. As the high end handsets (e.g., two RXs, two TX) is major target of the 6G, dual-registration can be considered again for the interworking and migration solution.

Principles for interworking and migration

- In the long-term, 6G network may be standalone and/or co-exist with legacy networks (e.g., 5GS, EPS)

- Minimize impact to the legacy networks

- Minimize architectural options to prevent market fragmentation

- Considering the roaming from the beginning

- 6G core will be an enhancement of the 5G core and continue to follow the design principle of 5G

- Considering cloud native network design

- Minimize operator cost

Also, interworking with EPS should be considered, as the 5GS deployment is slower phase than EPS.

Gap Analysis: In 5G, the dual registration concept was standardized but not widely deployed due to device and network limitations. In addition, EPS remains widely deployed in many regions, so the relationship between EPS and 6G, including interworking and migration, needs to be clearly defined from the beginning of 6G design.

Technical Impact: The study will impact the overall 6G system architecture by introducing a common/combo core concept and new interfaces to support both 6G-RAN and NG-RAN. The work will also cover interworking and mobility procedures between 6GS, 5GS, and EPS, including how to ensure seamless session continuity during mobility. It will define the interaction between network functions for multi-access scenarios and specify new UE capabilities for dual registration and multi-RAT session continuity. Roaming procedures will be integrated into interworking solutions from the beginning.

**** First Change ****

# Annex A.X. WT#2 Scope

WT#2: Study migration and interworking, including

-	How to support migration to 6GS

-	How to support interworking with 5GS, and EPS

-	How to support optimized use of 5GS and 6GS, with efficient/minimized signalling

-	How a 6G core supports both NG-RAN and 6G-RAN

-	How to define architectures suitable for each migration and interworking/mobility scenarios

How to support roaming cases

NOTE X:	Interworking with 2G/3G are not considered in this study. Interworking between 6GS and 5GS is assumed to work even if the UE has previously registered in 2G, 3G or 4G.

NOTE Y:  The detailed migration study scope will be coordinated and aligned with RAN.

**** Second Change ****

# 5.X. Key Issue #X: Interworking and Migration in 6G

As the expecting performance requirements is higher, smoother interworking and flawless service continuity with legacy network is a must have pre-requisite for the 6G. At the same time, operator cost-efficiency network design is required and this should be reflected for the 6G design principles. Consequently, the 6G architecture design must reconcile two conflicting requirements for the efficient interworking and migration.

The following principles should be considered for interworking and migration in 6G:

- In the long-term, 6G network may be standalone and/or co-exist with legacy networks (e.g., 5GS, EPS)

- Minimize impact to the legacy networks

- Minimize architectural options to prevent market fragmentation

- Considering the roaming from the beginning

- 6G core will be an enhancement of the 5G core and continue to follow the design principle of 5G

- Considering cloud native network design

- Minimize operator cost

In this key issue, the suitable architectures for each migration and interworking deployment scenario will be studied. For example, the set of network functions/network entities that is comprising a common core network or combo core network. The study on the required UE capabilities will follow. Also, whether to support dual registration and/or single registration should be studied together.

The solutions of this key issue will include the following aspects:

- Which scenarios to be supported for migration to 6GS from 5GS (and EPS), based on coordination with RAN.

- Which scenarios to be supported for interworking/mobility, e.g., interworking with 5GS (and EPS), mobility between 6GS and 5GS (and EPS), and how to support session continuity during mobility.

- Whether and how to support dual registration, single registration, and/or dual-steer of 5GS and 6GS.

- Whether and how to support multi access sessions using NG-RAN and 6G-RAN.

- Whether and how to define architectures suitable for each migration and interworking/mobility scenarios:

  - The set of network functions/network entities comprising the common core network. The common core network provides connectivity to both NG-RAN and 6G-RAN.

- How to support roaming cases.

**** End of Changes ****