3GPP SA WG2 Meeting #170	S2-2506864

Goteborg, Sweden, 25 – 29 August 2025	(revision of S2-2504653)

Source: 	LG Electronics

Title: 	KI#1, Solution#2 Update: resolving some editor’s notes

Document for: 	Approval

Agenda Item: 	20.3.1

Work Item / Release:	FS_AIML_CN_Ph2 / Rel-20

Abstract of the contribution: This paper proposes to update the Solution#2 of TR 23.700-04.

1	Discussion

This paper focuses on the following editor’s notes.

Editor's note:	In this solution, which steps are related to UE Data Collection and which steps are related to UE Data Transfer is FFS.

Editor's note:	How to support MNO's controllability and visibility in solution is FFS.

Editor's note:	How to separate "UE data collection" traffic over user plane from end user generated user plane traffic is FFS

Editor's note:	How the UE/NG-RAN knows the data to be collected is FFS.

Editor's note:	It is FFS whether DCF can be newly defined or Location Management Function (LMF) specified in TS 23.273 [6] can be reused/collocated.

1.1 Data collection vs. Data transfer

Based on the description of key issue#1, SA2 are focusing on the standardized transfer of the collected data.

How to support the standardized transfer of standardized data over UP for UE data collection including how to initiate, terminate and manage UE data transfer over the UP connection.

While the details of data collection between UE and RAN are up to RAN WGs, CN NF (DCF in Solution#2) sends the request for the secured user plane connection establishment, and the request for the transfer of the data collected by UE to the UE.

As a result, CN NF receives the data collected by UE via the secured user plane connection, and provides them to the UE side data collection server/OTT server.

Proposal 1. Solution#2 update:

Step 1: This request implies both UE data collection and UE data transfer. It may include the target UE(s), the data configured to be collected, the validity condition of the data transfer (e.g. immediately, or time/location for transfer, etc.), and so on.

Step 5: After the secured user plane connection is established, the DCF sends the requests for “the transfer of the data collected by UE” to UE through UP signalling.

Step 6: NOTE 2:	The configuration and trigger in UE and NG-RAN for data collection per use case are specified in RAN WGs. The data collection between UE and NG-RAN may be independent on the request in Step 1.

1.2 UE data collection traffic vs. End user generated traffic

In Solution#2, URSP and/or UE Local configuration information includes:

-	DCF address.

-	PDU session information for UE data collection (e.g. associated DNN and S-NSSAI), etc.

That is, the UE establishes the PDU session with associated DNN and S-NSSAI for the transfer of "UE data collection" traffic. The associated DNN and S-NSSAI can be configured by operator and (if any) SLA.

Proposal 2. Solution#2 update:

Step 4: “UE data collection” traffic is identified by PDU session with associated DNN and S-NSSAI, and this configuration is decided by operator and (if any) SLA in this Release.

1.3 MNO's controllability

Through the signalling between UE and DCF via the secured user plane connection, MNO can explicitly initiate or terminate the data transfer, or provide the validity condition of the data transfer to UE. It implies to support the MNO’s controllability on the data transfer. The controllability on the data collection between UE and RAN is expected to be specified in RAN WGs.

Proposal 3. Solution#2 update:

Step 5: After the secured user plane connection is established, the DCF sends the requests for “the transfer of the data collected by UE” to UE through UP signalling. It includes the initiation or termination of the data transfer, or the validity condition of the data transfer (e.g. immediately, or time/location for transfer, etc.)

1.4 MNO's visibility

Based on the description of key issue#1, SA2 are focusing on the standardized data.

How to support the standardized transfer of standardized data over UP for UE data collection including how to initiate, terminate and manage UE data transfer over the UP connection.

The details of standardized data are expected to be specified in RAN WGs. CN NF (DCF in Solution#2) can verify whether the data collected by UE is standardized data or not, and whether it matches with the request from UE side data collection server/OTT server or not, before sending them to UE side data collection server/OTT server.

Proposal 4. Solution#2 update:

Step 7: The DCF verifies whether the collected UE data is standardized data or not, and whether the request received from UE side data collection server/OTT server matches the data collected by UE, before sending them to UE side data collection server/OTT server.

1.5 Standalone Functionality

In order to ensure uniqueness of feature for data collection and transfer, we propose to identify DCF as new standalone function and it is worth noting it can be collocated with LMF depending on network deployment.

Proposal 5. Solution#2 update:

NOTE x:	The DCF can be collocated with Location Management Function (LMF) specified in TS 23.273 [6] depending on network deployment.

2	Text Proposal

It is proposed to agree the following changes into TR 23.700-04.

* * * * Start of 1st Change * * * *

6.2	Solution #2: Generalized UE data collection over user plane

6.2.1	High level principles

This solution addresses the key issue#1.

It proposes a solution for Generalized UE data collection over user plane by applying the existing user plane-based LCS, in similar to a user plane connection between UE and LMF specified in clause 6.18 of TS 23.273 [6].

This solution provides MNO’s controllability through the UP signalling between UE and DCF over the secured user plane connection (i.e. explicitly requesting the initiation/termination of data transfer and/or providing the validity condition of the data transfer to UE).

This solution provides MNO’s visibility through DCF verification before providing the collected data to UE side data collection server/OTT server.

6.2.2	Description

This solution addresses the key issue#1.

It proposes a solution for Generalized UE data collection over user plane by applying the existing user plane-based LCS, in similar to a user plane connection between UE and LMF specified in clause 6.18 of TS 23.273 [6].

It assumes the following:

-	Data collection function (DCF) in core network.

Decides to use user plane for UE data collection based on UE subscription data, UE capability (if needed) to perform data collection and local configuration.

Provides the UE the information to establish a secure connection (e.g. DCF address).

Sends the request on UE data transfer via the secured user plane connection

Sends the event reporting (if the event data collection is allowed such as Deferred MT-LR of TS 23.273 [6]) related request to UE.

Expose the collected data to UE side data collection server/OTT server, etc.

-	UE subscription data in UDM:

-	DCF ID.

-	Data collection over user plane between UE and DCF is allowed/not allowed

-	User consent on data collection, etc.

-	URSP and/or UE Local configuration information includes:

-	DCF address.

-	PDU session information for UE data collection (i.e. associated DNN and S-NSSAI), etc.

6.2.3	Procedures

Figure 6.2.2-1: High-level procedure for UE data collection over user plane

- UE side data collection server/OTT server sends UE data collection request to 5GC. This request implies both UE data collection and UE data transfer. It may include the target UE(s), the data configured to be collected, the validity condition of the data transfer (e.g. immediately, or time/location for transfer, etc.), and so on.

In case the UE side data collection server/OTT server is located outside MNO, the NEF authorizes it to accept the request.

2.	The DCF determines to use user plane for UE data collection based on UE subscription data (e.g. Data collection over user plane between UE and DCF is allowed/not allowed), etc. It may be triggered by AMF request.

User consent is checked based on UE subscription data for the purpose of UE data collection and ML model training.

NOTE x:		The DCF can be collocated with Location Management Function (LMF) specified in TS 23.273 [6] depending on network deployment.

3.	In similar to user plane connection establishment procedure specified in clause 6.2.1 of TS 24.572 [7], the DCF invokes an Namf_Communication_N1N2MessageTransfer service operation towards the AMF to request transfer of the USER PLANE CONNECTION ESTABLISHMENT COMMAND message. The AMF forwards it to the UE via DL NAS TRANSPORT message.

4.	The UE establishes a PDU session if there is no established PDU session for UE to DCF PDU connectivity service, and establishes a TLS connection with the DCF. Finally, the secured user plane connection is established between UE and DCF.

NOTE y:	The URSP rules or UE Local configuration is used to enable to routing of collected data via a specific PDU session (i.e. associated DNN and S-NSSAI). “UE data collection” traffic is identified by PDU session with associated DNN and S-NSSAI, and this configuration is decided by operator and (if any) SLA in this Release.

5.	The UE sends USER PLANE CONNECTION COMPLETE message via UL NAS TRANSPORT message the to the AMF, and the AMF forwards it to the DCF by invoking Namf_Communication_N1MessageNotify service operation.

After the secured user plane connection is established, the DCF sends the requests for “the transfer of the data collected by UE” to UE through UP signalling. It includes the initiation or termination of the data transfer, or the condition of the data transfer (e.g. immediately, or time/location for transfer, etc.)

6.	The UE performs UE data collection. The NG-RAN may provide the configuration for the UE data measurement.

NOTE z:	The configuration and trigger in UE and NG-RAN for data collection per use case are specified in RAN WGs. The data collection between UE and NG-RAN may be independent on the request in Step 1.

7.	The collected data and/or event reports (if the event data collection is allowed such as Deferred MT-LR of TS 23.273 [6]) are notified to UE side data collection server/OTT server via the DCF based on the request of step 1.

Before sending to UE side data collection server/OTT server, the DCF verifies whether the collected UE data is standardized data or not, and whether the request of step 1 matches the data collected by UE.

6.2.4	Impacts on services, entities and interfaces

DCF:

-	Decides to use user plane for UE data collection, and provides the UE the information to establish a secure connection (e.g. DCF address, etc.).

-	Sends the request on UE data transfer via the secured user plane connection

-	Exposes collected data and/or event report from UE to UE side data collection server/OTT server.

UE:

-	Establishes a secured user plane for UE data collection with DCF.

-	Performs UE data collection and provides it to the DCF.

* * * * End of Changes * * * *