3GPP TSG-SA WG2 Meeting #170	S2-2507031

Goterborg, Sweden, August 25 – 29, 2025

Source:	vivo, MediaTek Inc.

Title:	Solution#3 update with further RTP reduction consideration

Document for:	Approval

Agenda Item:	20.1.1

Work Item / Release:	FS_5GSAT_Ph4_ARC / Rel-20

Abstract of the contribution: The contribution proposes to update solution #3 for KI#1.

1. Discussion

The DP S2-2507021 analises some RTP reduction issue. One of the issues is how to support DTMF in GEO voice scenario, and proposes that payload type (PT) is necessary to be included to distinguish voice payload and DTMF payload. Although the PT field is included, considering DTMF has different TimeStamp sequence patten then voice packets, PGW needs to know which packet is DTMF based on the PT value and obtain duration information in the DTMF payload for restoring the TimeStamp. In this way, RTP restoration assistant information (e.g., interval of timestamp, PT of DTMF) sent from PCRF to PGW is needed.

In order to minimize the impact in UE, the input and output voice packets of IMS client still using IP/UDP/RTP protocol, which does not require TFT enhancement. The UE communiacation modual can perform RTP reduction before packing the voice packets over PDCP layer, as well as perform RTP restoration right after receiving voice packets over PDCP layer.

2. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-19.

* * * Start of Change * * * *

## 6.3	Solution #3: IP Type IMS PDN connection over User Plane with non-IP voice transfer between UE and PGW

### 6.3.0	High-level solution Principles

This solution is to address Key Issue #1.

This solution uses a single IMS APN PDN connection with two EPS bearers. The IMS PDN is IP type (e.g. IPv4, IPv6 or IPv4v6) PDN Connection. This solution is a User Plane based solution, i.e. both the default bearer and the dedicated bearer are using S1-U data transfer.

In order to reduce the header overhead to the most extreme level:

-	it is proposed to transmit the IMS voice traffic as non-IP traffic over dedicated EPS bearer; and

-	the PGW will translate the non-IP voice packets to IP/UDP/RTP packets before forwarding them to the IMS eneity in the DN, e.g. the IMS AGW (IMS AGW is not aware of the non-IP transport between PGW and UE); and

-	at the same time in order to reduce the impact to the IMS control plane (IMS network is designed based on IP) and IMS entity in the IMS DN, e.g. P-CSCF,

it is proposed to transmit the IMS Signalling (e.g. SIP INVITE ... etc.) over default EPS bearer using IP.

### 6.3.1	Description

Figure 6.3.1-1: IP Type IMS PDN connection over User Plane with non-IP voice transfer between UE and PGW

NOTE:	Assuming data rate is 1Kbps and voice codec rate is 0.7Kbps, then there is only 0.3Kbps remaining for voice packet "header" overhead. Assuming ptime = 80ms, which means there will be 12.5 voice packets per second to consume the 0.3Kbps remaining bandwidth, which means each voice packet header part can only use 300bits/12.5 == 24 bits, i.e. each voice packet can only tolerate maximum 3 Bytes header size, these overhead are shared by multiple layers including RTP/UDP/IP/EPS layer.

The IMS PDN (for voice over GEO) is used to transmit both IMS signalling and IMS voice.

-	For IMS signalling:

-	the main consideration is to minimize the impact to the IMS control plane network (IMS network is designed based on "IP") so the default EPS bearer for IMS PDN connection is configured as IP transmission (i.e. IMS Signalling over IP, no special handling).

-	For voice packets, on the other hand, it is proposed to use non-IP transmission.

Editor's note:		In the context of using "Non-IP" voice packet transfer, only one call or more than one call needs to be supported is FFS based on SA WG1 discussion.

Editor's note:		In the context of using "Non-IP" voice packet transfer, what are the essential RTP header fields for the minimum information that need to be provided in RTP for IMS voice over GEO is FFS based on SA WG4 discussion.

Thus the ideal IMS PDN configuration for IMS voice over GEO would be:

-	the PDN Type for this IMS PDN is IP Type; and

-	a dedicated EPS bearer is configured to transmit non-IP voice packets.

### 6.3.2	Procedures

#### 6.3.2.1	Establishment and Usage of IP Type IMS PDN connection over User Plane with non-IP voice transfer between UE and PGW

In order to support:

-	IP Type IMS PDN connection over User Plane with non-IP voice transfer between UE and PGW, which support to use:

-	IP (IP(v4), IPv6 or IPv4v6) type traffic for default EPS bearer; and

-	non-IP type traffic between UE and PGW for dedicated EPS bearer;

the UE supports this feature indicates support of "non-IP Voice Transfer" in UE network capability in Attach request, and the MME supports the "non-IP Voice Transfer" selects a PGW supporting "non-IP Voice Transfer" for establishing the IMS PDN connection and indicates "non-IP Voice Transfer Allowed" to UE in Attach accept.

During IMS voice call setup procedure, based on the SDP negotiation, the P-CSCF sends necessary media paratemers related to RTP header restoration to the PCRF, e.g., ptime, codec sample frequency, payload type for DTMF, etc. The PCRF triggers the PGW to activate a dedicated EPS bearer for voice with RTP restoration assistance information, e.g., interval of timestamp, payload type for DTMF, to enable the PGW to calculate the timestamp field of a restored RTP headder. During Dedicated EPS bearer context activation procedure, the MME indicates to the PGW, for a dedicated bearer to be activated for voice, the traffic type to be sent over the bearer is non-IP traffic.

NOTE:	Interval of timestamp is inferred from ptime and codec sample frequency, i.e., interval of timestamp = ptime (ms) * frequency / 1000. The ptime is indicated in the "a=ptime" line and the frequency is indicated in the "a=rtpmap" line in SDP.

After receiving the indication from MME, the PGW activates the "non-IP Voice Transfer" funcationality for the dedicated EPS bearer for voice. After SDP negotiation is done and "non-IP Voice Transfer Allowed" indication is received, the UE activates the "non-IP Voice Transfer" functionality for the dedicated EPS bearer for voice.

#### 6.3.2.2	Non-IP voice transfer between UE and PGW

##### 6.3.2.2.1	Uplink Voice Data Transmission using non-IP over dedicated EPS bearer

6.3.2.2.1.1	UE side consideration

For the uplink voice packets to be sent over a dedicated bearer using non-IP, i.e. without IP/UDP header, the UE strips IP/UDP/RTP header after TFT maching approach, and inserts 1-byte PT+M (7bits payload type and 1 bit Marker) and 1-byte RTP SN, then sends the "Non-IP Voice packet" over the dedicated EPS bearer.

6.3.2.2.1.2	Network side consideration

When the PGW receives uplink voice packets from the dedicated EPS bearer, the PGW converts the "Non-IP Voice packet" to "Voice over RTP/UDP/IP packet".

In order to build the context of the conversion operation, when the PDN GW triggers the dedicated EPS bearer creation procedure as specified in step 2-11 in clause 5.4.1 of TS 23.401 [5], the PGW also builds and stores the context (e.g. IP(es) and Port(s), interval of timestamp, payload type for DTMF), based on information received from PCRF, in order to perform the conversion, i.e. to construct the RTP/UDP/IP headers for the received uplink voice packet and then forward the conversed voice packets (with RTP/UDP/IP header) to the DN.

The PGW restores the IP/UDP based on TFT and restores the RTP header as following:

-	the 2 bits of "V" field for version is set to default, i.e., "10";

-	the 1 bit of "P" field for padding is set according to whether padding is needed or not;

-	the 1 bit of "X" field for extension header is set to default, i.e., "0", means no extention header;

Editor's note:	whether some extension header is needed or not is FFS.

-	the 4 bits of "CC" field for number of CSRC is set to default, i.e., "0", means no CSRC field;

-	the 7 bits of "PT" field for payload type and 1 bit of "M" field are set according to the 1-byte PT+M field in the received voice packet;

-	the 16 bits of "SN" field for sequence number is inferred by the 1-byte RTP SN in the received voice packet;

-	the 32 bits of "TS" field for timestamp is set according to the inferred RTP SN, the interval of timestamp, and the payload for DTMF received from PCRF, i.e., if the payload type in the received voice packet is for DTMF, the value of "TS" field is set according to the payload of the received voice packet, otherwise the value is set to inferred RTP SN * interval of timestamp;

NOTE:	The payload of DTMF is structured with 4-byte length, which includes duration and end-mark fields, if end-mark is not set, the value of "TS" field is not changed, otherwise the value of "TS" field is increased by the duration.

Editor's note:	Considering the ptime for GEO voice is large, whether slience and talk voice packets use same time interval needs coordination with SA4 WG.

-	the 32 bits of "SSRC" field is set to a number generated (e.g., randomly) by the PGW for the session.

##### 6.3.2.2.2	Downlink Voice Data Transmission using non-IP over dedicated EPS bearer

For the downlink voice packet, the PGW may still leverage the current DL TFT as usual, but for DL traffic steered to the dedicated EPS bearer the PGW converts the DL Voice over RTP/UDP/IP packets to non-IP Voice packet. The conversion can be simply removing all the RTP/UDP/IP headers and inserting 1 Byte PT+M and 1 Byte RTP SN (e.g. use the 1 Byte SN value and 1 Byte PT+M values originally indicated in the removed RTP header) as the converted header part of the voice packet before forwarding the voice packet over the dedicated EPS bearer.

Up to implementation, the UE may restore the essential RTP information (e.g., RTP SN, TS) similar as PGW does in clause 6.3.2.2.1.2 taking into account the received SDP information as well as the 1-byte PT+M and 1-byte RTP SN in the non-IP voice packet.

### 6.3.3	Impacts on Services, Entities and Interfaces

Editor's note:	Whether a new GBR or non-GBR QCI is needed for the dedicated EPS bearer used for voice traffic is FFS. Whether an existing GBR or non-GBR QCI can be used for the dedicated EPS bearer used for voice traffic is FFS.

UE:

-	support dedicated EPS bearer for voice for NB-IOT.

-	indicate support of "non-IP Voice Transfer" to the MME.

-	transfer uplink IMS voice packet over dedicated bearer using non-IP.

-	for downlink voice packets, may restore the necessary RTP information.

Editor's note:		In the context of using "Non-IP" voice packet transfer, only one call or more than one call needs to be supported is FFS based on SA1 discussion.

Editor's note:		In the context of using "Non-IP" voice packet transfer, what are the essential RTP header fields for the minimum information that need to be provided in RTP for IMS voice over GEO is FFS based on SA4 discussion.

MME:

-	indicates support of "non-IP Voice Transfer Allowed" to the UE.

-	selects PGW supporting "non-IP Voice Transfer" for IMS PDN connection.

-	indicates "non-IP Voice Transfer" to the PGW.

P-CSCF:

-	informes media parameters related to RTP restoration to the PCRF.

PCRF:

-	informes RTP restoration assistance information to the PGW.

RAN:

-	support dedicated EPS bearer for voice for NB-IOT.

Editor's note:	Detail impacts on RAN, e.g. whether and how to enable/disable RoHC in the RAN for the voice bearer, is FFS.

PGW:

-	support dedicated EPS bearer for voice for NB-IOT.

-	for uplink voice packets, convert the non-IP voice packets into IP/UDP/RTP voice packets according to the RTP restoration assistance information and TFT received from PCRF before forwarding the packets to the IMS DN.

-	for downlink voice packets, convert the IP/UDP/RTP voice packets into non-IP voice packets before forwarding the packets to the UE.

* * * End of Change * * * *