3GPP TSG|WG-SA2 Meeting #170 	S2-2507371

Goteborg, Sweden, August 25-29 2025

# Source:	Nokia

# Title:	New WID: TEI-20 on PDU set support exposure

# Document for:	Information

# Agenda Item:	30.1

3GPP™ Work Item Description

Information on Work Items can be found at http://www.3gpp.org/Work-Items 
See also the 3GPP Working Procedures, article 39 and the TSG Working Methods in 3GPP TR 21.900

Title: PDU set support exposure

Acronym: TEI20_PSSexp

Unique identifier:

{A number to be provided by MCC at the plenary}

Potential target Release:	Rel-20

# 1	Impacts

| Affects: | UICC apps | ME | AN | CN | Others (specify) |
| --- | --- | --- | --- | --- | --- |
| Yes |  |  |  | x |  |
| No | x | x | X |  | x |
| Don't know |  |  |  |  |  |

# 2	Classification of the Work Item and linked work items

## 2.1	Primary classification

### This work item is a …

|  | Study |
| --- | --- |
|  | Normative – Stage 1 |
| x | Normative – Stage 2 |
|  | Normative – Stage 3 |
|  | Normative – Other* |

* Other = e.g. testing

## 2.2	Parent Work Item

For a brand-new topic, use “N/A” in the table below. Otherwise indicate the parent Work Item.

| Parent Work / Study Items |  |  |  |
| --- | --- | --- | --- |
| Acronym | Working Group | Unique ID | Title (as in 3GPP Work Plan) |
| N/A |  |  |  |

### 2.3	Other related Work Items and dependencies

| Other related Work /Study Items (if any) |  |  |
| --- | --- | --- |
| Unique ID | Title | Nature of relationship |
| 980016 | (Stage 2 for XRM) Architecture Enhancements for XR (Extended Reality) and media service | SA2 Work Item on XRM Phase 1 in Rel-18, introducing the concept of PDU Set QoS. This TEI20 proposal exposes the PDU set QoS status to the Application Function |
| 1040032 | Extended Reality and Media service (XRM) Phase 2 | SA2 Work Item on XRM Phase 2 in Rel-19 extending QoS Notification Control, and AQP to support PDU Set QoS. The application receives the same notification independently of whether PDU set QoS or normal QoS is applied by 5GS. This TEI20 proposal addresses the issue by exposing the PDU set QoS status to the Application Function |

# 3	Justification

Rel-18 XRM work introduced the notions of “PDU set” and “PDU Set based QoS handling” to enable more efficient utilization of the available network resources according to the XR application requirements (see WID: Architecture Enhancements for XR and media services - XRM in SP-230092). The AF may provide PDU set QoS parameters (PSDB, PSER) along with normal QoS parameters, and depending on 5GS configuration and NG-RAN support, PDU set based QoS handling is applied or not.

As per Rel-19 XRM_ph2 work the QoS Notification Control and AQP mechanisms have been also extended to support PDU Set QoS parameters, when available, and the notification has been enhanced with an explicit indication of the affected direction (UL and/or DL).

Throughout the lifetime of a session, depending on whether the serving NG-RAN supports PDU set based QoS handling (and Rel-19 notification, AQP enhancements) or not, either the “PDU set QoS parameters” or the normal “QoS parameters” will be used respectively. This scenario will be very common as the upgrade of the access networks to support enhancements of a new Release is a gradual procedure that may take several years.

The AF has no control over the selected NG-RAN and cannot be aware of its capabilities regarding PDU set QoS handling. Nevertheless, the application needs to be aware of whether the requested PDU set QoS or normal QoS has been used (and achieved or not), i) to adapt accordingly, ii) properly evaluate the usefulness of PDU set QoS enhancements and iii) identify with certainty whether both directions have been affected. Thus, it is proposed to expose this information to the AF.

# 4	Objective

WT 1:	Specify how to expose “PDU set QoS activation/deactivation” from 5GC to the AF.

## TU estimates and dependencies

| Work Task ID | TU Estimate (Study) | TU Estimate (Normative) | RAN Dependency (Yes/No/Maybe) | Inter Work Tasks Dependency |
| --- | --- | --- | --- | --- |
| WT 1 |  | 0.5 | No | None |

# 5	Expected Output and Time scale

{If this WID covers both stage 2 and stage 3, clearly indicate the different completion dates.}

| New specifications {One line per specification. Create/delete lines as needed} |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Type | TS/TR number | Title | For info at TSG# | For approval at TSG# | Rapporteur |

| Impacted existing TS/TR {One line per specification. Create/delete lines as needed} |  |  |  |
| --- | --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# | Remarks |
| 23.501 | Updates to 5G System architecture to support XRM | TSG SA#114 (December 2026) |  |
| 23.502 | Updates to 5G System procedures to support XRM | TSG SA#114 (December 2026) |  |
| 23.503 | Updates to 5G System Policy Control to support XRM | TSG SA#114 (December 2026) |  |

# 6	Work item Rapporteur(s)

Gkatzikis Lazaros, Nokia, 	lazaros.gkatzikis@nokia.com

# 7	Work item leadership

SA2

# 8	Aspects that involve other WGs

None

# 9	Supporting Individual Members

{At least 4 supporting Individual Members are needed. There is an expectation that these companies will provide resources to progress the work. Note that having 4 supporting companies is a necessary but not sufficient condition: the usual TSG approval process by consensus is needed for the WID approval}

Supporting IM name

Nokia