3GPP SA WG2 Meeting #170	S2-2507399

25 – 29 Aug, 2025, Goteborg , SE

Source:	Qualcomm Incorporated

Title:	[WT#2] Update to WT#2 and new Key issue to clarify migration options for multi-RAT connectivity.

Document for:	Approval

Agenda Item:	20.6.2

Work Item / Release:	FS_6G_ARC/Rel-20

Abstract of the contribution: This contribution elaborates on the need to study migration options in the CN for multi-RAT connectivity.

# 1. Justifications

WT#2 describes the need to study how to support migration to 6GS. However, the agreed text as in current SID SP-250806 lacks enough detail on the scope of WT#2.

This contribution proposes a more detailed scope of the study for migration to 6GS.

It is to be expected that 6G rollouts will include a mix of new spectrum and existing 5G spectrum. It is crucial to be able to dynamically utilize 5G bands for 6G wide-area rollouts without disrupting existing 5G services. While Multi-RAT Spectrum Sharing (MRSS), expected to be defined in RAN WGs, can provide a tightly integrated 6G-based robust low band coverage anchor for 6G mid to higher bands.

However, it is unlikely to cover all operator deployment scenarios and needs.

Without an alternative solution, for scenarios where MRSS is not possible to be deployed, operators may find themselves constrained in 6G bands, which in turn can cause a user experience in 6G that could be worse than the user experience in 5G.

Therefore, it is important to study Multi-RAT connectivity solutions that leverage both the 5G-based robust low band coverage as well as 6G bands in scenarios when MRSS is not available. These solutions not only need to support traffic steering and switching but also aggregation.

Proposal #1: Study Multi-RAT connectivity solutions for 5G and 6G, that leverage both the 5G-based robust low band coverage as well as 6G bands.

Depending on the possible solutions, there may be impacts in both CN and RAN, mostly in CN, or mostly in RAN.

In particular,

- Solutions where the traffic steering/switching/aggregation is performed in the RAN require RAN WGs expertise (RAN2 and RAN3).

- Solutions where the traffic steering/switching/aggregation is performed above the RAN require SA2 expertise.

No solution should be discarded a priori at scope discussions. Rather, the feasibility of migration solutions, including multi-RAT connectivity, should be studied at the WG with the right expertise.

After feasibility study, coordination across RAN2/RAN3/SA2 groups may be needed to converge on a single solution (preferred) or set of solutions.

Proposal #2: SA2 to focus on feasibility study on Multi-RAT connectivity solutions for 5G and 6G that are CN-centric (above the RAN). After feasibility study, SA2 to coordinate with RAN WGs for final decision.

Note that the scope of SA2 study should be determined in SA2, and coordination with RAN WGs should occur after discussing feasibility, i.e. to have final decision on the outcome of the study. For that reason we propose the following:

Proposal #3: Change Note 3 to WT#3 from coordination of study scope to coordination of study outcome between SA2 and RAN WGs.

# 2. Proposal

It is proposed to include the following changes to TR 23.801-01:

**** First Change ****

## 5.X	Key Issue #X: Key Issue on 5G to 6G migration and multi-RAT connectivity.

It is to be expected that 6G rollouts will include a mix of new spectrum and existing 5G spectrum. It is crucial to be able to dynamically utilize 5G bands for 6G wide-area rollouts without disrupting existing 5G services. While Multi-RAT Spectrum Sharing (MRSS), expected to be defined in RAN WGs, can provide a tightly integrated 6G-based robust low band coverage anchor for 6G mid to higher bands.

However, it is unlikely to cover all operator deployment scenarios and needs.

Without an alternative solution, for scenarios where MRSS is not possible to be deployed, operators may find themselves constrained in 6G bands, which in turn can cause a user experience in 6G that could be worse than the user experience in 5G.

Therefore, it is important to study Multi-RAT connectivity solutions that leverage both the 5G-based robust low band coverage as well as 6G bands in scenarios when MRSS is not available. These solutions not only need to support traffic steering and switching but also aggregation.

This key issue shall address the feasibility of:

- Multi-RAT connectivity solution between 5G and 6G

- Feasibility of solutions for traffic steering, switching and/or aggregation across 5G and 6G.

This key issue should focus on solutions above the RAN.

NOTE: 	RAN centric solutions can be studied in RAN WGs (RAN2 and RAN3 in particular). Coordination with RAN WGs is necessary for final conclusions of this Key issue.

**** Second Change ****

## A.X 	Work Task 2

WT#2: Study migration and interworking, including

-	How to support migration to 6GS

-	Whether and how to support Multi-RAT connectivity between 5G and 6G, including traffic steering, switching and aggregation across 5G and 6G.

-	How to support interworking with 5GS

-	Whether and how to support interworking with EPS

-	How to support interworking between 6GS and 4G/5G NTN/satellite access that use EPS/5GS.

NOTE 2:	Interworking with 2G/3G are not considered in this study. Interworking between 6GS and 5GS is assumed to work even if the UE has previously registered in 2G, 3G or 4G.

NOTE 3:  The detailed migration study outcome will be coordinated and aligned with RAN.

**** End of Changes ****