SA WG2 Meeting #170	S2-2506115

25 - 29 August, 2025, Goteborg, Sweden

SA WG2 Meeting #169	S2-2504577

19 - 23 May, 2025, Fukuoka, Japan

SA WG2 Meeting ##168	S2-2502826

07 - 11 April, 2025, Goteborg, Sweden

3GPP TSG-RAN WG3 Meeting #127	R3-250796

Athens, Greece, 17 – 21 Feb 2025

Title:	Reply LS on LMF-based AI/ML Positioning for case 3b

Response to:	R3-250074 (S2-2501341)

Release:	Release 19

Work Item:	AIML_CN, NR_AIML_Air, NR_AIML_air-Core

Source:	RAN3

To:	SA2

Cc:	RAN1, RAN2

Contact person:	Chen Jiajun

Chen.jiajun1@zte.com.cn

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	-

# 1	Overall description

RAN3 thanks SA2 for the reply LS on LMF-based AI/ML Positioning for case 3b in R3-250074/ S2-2501341.

The feedback from RAN3’s perspective is provided separately for each of the following Editor’s Notes.

- Editor's note:	What input data collected from UE and NG-RAN to LMF for LMF-based AI/ML Positioning will be determined by RAN WG1. How to collect the input data for LMF-based AI/ML Positioning calculation need coordination with RAN WGs.

In response to the 1st Editor’s Note, " the collection of input data for LMF-based AI/ML Positioning in Case 3b can be achieved by reusing the Measurement Information Transfer procedures defined in NRPPa in TS38.455, which supports requesting and reporting the relevant measurements to the LMF for Case 3b. And the details of the measurements for case 3b are still pending and subject to further discussion in RAN1, which is out of RAN3 scope. "

- Editor's note:	The procedure to collect the input data for AI/ML based positioning calculation by the LMF from the NG-RAN is subject to RAN WG progress and feedback.

In response to 2nd Editor’s Note, " LMF reuses the Measurement Information Transfer procedure defined in NRPPa in Section 8.5 in TS38.455 to obtain the measurement data input data from the NG-RAN node. "

- Editor's note:	Whether and how the LMF takes the RAN load into account in this procedure is FFS.

In response to the 3rd Editor’s Note, " The existing cause value "Requested Item Temporarily Not Available" can cover the situation where the NG-RAN node cannot perform the measurements due to high load. From RAN3’s perspective, there is no need for the LMF to consider the RAN load when performing AI/ML direct positioning (Case 3b). "

The corresponding updates on NRPPa Measurement procedures will be available when the Rel-19 TS 38.455 is published.

# 2	Actions

To SA2

ACTION: 	RAN3 kindly asks SA2 to take information above into account.

# 3	Dates of next RAN3 meetings

Updated meeting schedule can be found at: https://portal.3gpp.org/?tbid=373&SubTB=381#/

RAN3#127bis			2025-04-07 – 2025-04-11				China, CN

RAN3#128				2025-05-19 – 2025-05-23				Malta, MT