SA WG2 Meeting #S2-170	S2-2506121

25 - 29 August, 2025, Goteborg, Sweden

3GPP TSG-CT WG1 Meeting #155	C1-254010

# Bratislava, Slovakia, 19-23 May 2025

# Title:	LS on parameters for ProSe additional parameters announcement request message

# Response to:	-

# Release:	Release 19

# Work Item:	5G_ProSe_Ph3

Source:	CT1

To:	SA2

Cc:

Contact Person:

Name:	Sunghoon Kim

Tel. Number:	-

E-mail Address:	sunghoon@qti.qualcomm.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments:	None

1. Overall Description:

As per stage-2 requirements in clause 6.3.2.5.4 of TS 23.304 which states the following:

2. The 5G ProSe Remote UE sends to the 5G ProSe Intermediate UE-to-Network Relay an Additional Parameters Announcement Request including the following additional information to obtain additional parameters.

-	Relay Service Code: information about the connectivity service provided by the 5G ProSe UE-to-Network Relay.

-	Root Relay Info: identify information (i.e. User Info ID) of the 5G ProSe UE-to-Network Relay.

CT1 would like to ask questions for including both relay service code and root relay info in the ProSe additional parameters announcement request message.

Given that the ProSe additional parameter announcement request message is a PC5-S message, it is assumed that the 5G ProSe multi-hop remote UE, the 5G ProSe intermediate UE-to-network relay UE, and the root UE-to-network relay UE have established the PC5 direct link for the relay service code. Hence, the ProSe additional parameter announcement request message can be routed to the root UE-to-network relay UE through the established PC5 direct link. In this sense, both the relay service code and the root relay info seem not necessary to be included in the ProSe additional parameter announcement request message.

Question 1: CT1 would like to get clarification from SA2 regarding what the usage of both relay service code and root relay info for ProSe additional parameter announcement request procedure is.

Question 2: CT1 would like to ask SA2 if both relay service code and root relay info can be excluded from the message for protocol efficiency.

2. Actions:

To SA WG2 group.

ACTION: 	CT1 kindly asks SA2 to provide feedback for the questions above.

3. Date of Next CT1 Meetings:

CT1#156                           25th August- 29th August 2025.....................Goteborg, Sweden

CT1#157                           13th October- 17th October 2025..................Sophia Antipolis, France

CT1#158                           17th November- 21st November 2025............Dallas, US

CT1#159                           9th February- 13th February 2025..................India