SA WG2 Meeting #S2-170	S2-2506146

25 - 29 August, 2025, Goteborg, Sweden

3GPP TSG-RAN WG3 Meeting #128	R3-253941

Malta, Malta, 19 – 23 May, 2025

Title:	LS on the latest Ambient IoT progress of RAN3

Response to:

Release:	Release-19

Work Item:	Ambient_IoT_solutions

Source:	RAN3

To:	SA2, CT4

Cc:

Contact person:	Yan Wang

Wangyan7@huawei.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	N/A

# 1	Overall description

RAN3 would like to provide the following latest RAN3 progress of Ambient IoT:

- In case of indirect connectivity, as parallel sessions between gNB and AMF are supported, the AIOTF Identifier and the Correlation Identifier are included outside of the containers in all the Inventory related NGAP messages (in addition to including the Correlation Identifier inside the containers as previously agreed).

- In case of indirect connectivity, as parallel Command procedures for different devices between gNB and AMF within the same session are supported, the AIOTF Identifier, the Correlation Identifier, and the RAN A-IoT Device NGAP ID are included outside of the containers in all the Command related NGAP messages (in addition to including the Correlation Identifier and the RAN A-IoT Device NGAP ID inside the containers as previously agreed).

- Introduce a new A-IoT CN triggered Class 1 NGAP A-IoT Session Release procedure.

- Introduce a new gNB triggered Class 2 NGAP A-IoT Session Release Request procedure.

- In the case of Inventory only scenario, introduce Inventory Complete indication to inform the AIoT CN about the completion of the triggered Inventory session. FFS on how to provide such indication.

- NGAP: Command Request procedure is a per single device procedure.

- CN A-IoT Device NGAP ID is not needed.

- No NGAP signalling is introduced for providing the A-IoT RAN Information (served reader list, supported A-IoT areas, reader location) from the gNB to the AIOTF.

- About Reader Selection:

  - Upon only receiving the area in Inventory Request, the gNB selects readers within the indicated area.

  - Upon receiving neither the area nor the reader list in Inventory Request, the gNB selects all the served readers.

  - WA: Upon only receiving the reader list in Inventory Request, the gNB selects the readers indicated by the reader list.

# 2	Actions

To SA2, CT4

ACTION: 	RAN3 kindly asks SA2, CT4 to take the above information into account.

# 3	Dates of next RAN3 meetings

Updated meeting schedule can be found at: https://portal.3gpp.org/?tbid=373&SubTB=381#/

RAN3#129	2025-08-25 - 2025-08-29		Bangalore, IN

RAN3#129-bis	2025-10-13 - 2025-10-17		Prague, CZ

RAN3#130	2025-11-17 - 2025-11-21		Dallas, US