SA WG2 Meeting #170	S2-2506187

# 25 - 29 August 2025, Goteborg, Sweden

Source:	ZTE

Title:	Model Evaluation Metrics Enhancement

Document for:	Agreement

Agenda Item:	30.1

Abstract of the contribution: This discussion paper presents some explanations for metrics of regression model.

Discussion

Extension of ML Model Evaluation Metrics in the Core Network

Currently, the standardized ML Model metric only contains accuracy. In the current TS 23.288 specification, the definition of accuracy (i.e. correct predictions divided by total number of predictions) is only applicable to assess the performance of classification models.

However, the prediction(s) of almost every Analytics defined in TS 23.288 include continuous variables rather than discrete ones (Please see the table 1 below for more information). For instance, in the commercially deployed Observed Service Experience (OSE) analytics,  NWDAF may predict continuous variable like the Service Experience over the Analytics target period.

As shown in Figure 1 below, continuous prediction and classification prediction are fundamentally different. The accuracy defined in TS 23.288 is not suitable for evaluating regression models.

Figure 1. Comparison of Classification and Continuous Prediction

| - Analytics | - Important continuous output example |
| --- | --- |
| - Slice load level | - Number of predicted UE registrations at the Network Slice instance (average, variance); Number of predicted PDU Session establishments of the Network Slice instance (average, variance). |
| - Service experience | - Service Experience (e.g. QoE) over the Analytics target period (average, variance). |
| - NF load | - The average load of the NF instance over the Analytics target period; The average usage of assigned resources (CPU, memory, disk). |
| - Network Performance Analytics | - gNB resource usage: Usage of assigned resources (average, peak). |
| - UE related Analytics | - UE location:Predicted location prediction during the Analytics target period. |
| - UE communication Analytics | - Traffic Volume: Volume UL/DL (average and variance). |
| - Abnormal behaviour | - Estimated number of UEs affected by the Exception (applicable when the Target of Analytics Reporting = "any UE") |
| - User data congestion | - The application's throughput as a percentage of the total throughput in the Area of Interest. |
| - QoS Sustainability | - Crossed Reporting Threshold(s): The Reporting Threshold(s) that are met or exceeded or crossed by the statistics value or the expected value of the QoS KPI. |
| - Dispersion | - Data volume dispersed at some location |
| - WLAN performance | - Predicted RSSI/RTT |
| - Session Management Congestion Control Experience | - No predictions |
| - Redundant Transmission Experience | - UL/DL packet drop/delay/loss rate GTP-U |
| - DN performance | - Average/maximum/minimum traffic rate; Average/maximum/minimum packet loss rate |
| - PFD determination | - No predictions |
| - Location Accuracy | - Percentage of NLOS/LOS measurements of positioning method in applicable area； - Percentage of UE in indoor/outdoor in applicable area |
| - E2E data volume transfer time | - E2E data volume transfer time UL/DL |
| - Relative proximity | - Time to collision |
| - PDU session traffic | - No predictions |
| - Movement behaviour | - Average speed of all UEs in the area of interest |
| - Signalling Storm | - Received number of signalling of the specific signalling type. |
| - QoS and Policy assistance | - Predicted QoE: The predicted QoE of the application when the corresponding QoS parameter set is applied, e.g. average, variance, maximum, minimum of QoE. |

Table 1. Analytics defined in TS 23.288 with continuous prediction outputs

- Proposals

- A proposal is to introduce at least one ML Model metric for evaluating the performance of regression models. Some candidate metrics are listed below:

- The Mean Square Error (MSE),  can be calculated as follow:

- MSE = 1ni=1n(Yi −γi)2

The Mean Absolute Error (MAE) (datatype = double or float) of a regression ML model, can be calculated as:

MAE = i = 1nYi −γin

The R-square, which indicates the godness of fit of a regression model can be calculated as:

R2 =1−i=1n(Yi −γi)2i=1n(Yi  −ym )2

Where n is the number of data in sample, Y is the observed value (ground truth data) and γ is the predicted output, ym is the mean of data sample.

A comparison table for three candidate metrics:

|  | Full name | Advantages | Disadvantages | Popularity |
| --- | --- | --- | --- | --- |
| MSE | Mean Squared Error | - Penalizes larger errors more heavily (due to squaring) - Differentiable everywhere (useful for optimization) | Not in the same unit as the target variable (squared units) More sensitive to outliers | Most commonly used in machine learning |
| MAE | Mean Absolute Error | Same unit as the target variable (easier interpretation) More robust to outliers than MSE | Not differentiable at zero (can complicate optimization) Doesn't penalize large errors as much as MSE | Common in practical applications where interpretability matters |
| R-square | R-squared (or called Coefficient of Determination) | Scale-independent (always between 0 and 1 for regular linear regression) | Not always appropriate for nonlinear models Can be misleading if model assumptions are violated | Commonly used in machine learning |

Table 2. Comparison table of three most commonly used regression model metrics

For now, the draft CR implements the loss which contain all of the listed three metrics.