3GPP TSG-SA WG2 Meeting #170	S2-2506195

Goteborg, Sweden, Aug 25 – 29, 2025					               (was )

Source:	Nokia

Title:	KI#4_5_6 new solution – Communication aspects for Configuring and transport of sensing data results

Document for:	Approval

Agenda Item:	20.2.1

Work Item / Release:	FS_Sensing_ARC / Rel-20

Abstract of the contribution: The contribution proposes a new solution for KI#4,5 and 6.

1. Discussion

The proposed solution aims to implement flexible solution suiting to the current scope of 5GA and at the same time have the solution future ready, with the aspects covering on how to configure a Sensing Entity and how to collect Sensing Data with an aim to optimize transport and communication of the same from the Sensing Entity to Core Network and vice-versa. It is also to be noted that, the current scope of work is related to monostatic gNB scenario.

This paper does not cover about what parameters are provisioned to the CN and accordingly sent to the Sensing Entity. This paper also does not cover aspects related to what is the content of the Sensing Data and the Sensing Result. Both these aspects are covered in other paper of the corresponding KI.

2. Proposal

It is proposed to agree to have the following text in 3GPP TR 23.700-14.

# * * * * First change * * * *

# 6	Solutions

## 6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

|  | Key Issues |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Solutions | KI#1 | KI#2 | KI#3 | KI#4 | KI#5 | KI#6 |
| #X |  |  |  | X | X | X |

# * * * * Second change (All new) * * * *

## 6.X	Solution #X: Communication aspects for Configuring and transport of sensing data results

### 6.X.0	High-level solution principles

This solution proposes below high-level principles are proposed

- Sensing Function (SF) receives sensing configurations from the Sensing Client, SC(AF) with or without NEF (in case of trusted AF) using the exposure framework.

- SF provisions the Sensing Entity (SE) (i.e., gNB) through AMF, the configurations required to address the sensing requirements of SC.

- SF and SE exchange the required configurations for SE to transport the sensing results to the SF.

- SF provides the sensing results to the SC based on one-time request or a subscription-based request for periodic or event-based result notification.

### 6.X.1	Description

The Sensing Function (SF) implements collection of the sensing request as configurations from the AF (Sensing Client - SC) with or without NEF and providing the corresponding translated configurations to the Sensing Entity (SE); it is also to be noted that, the current scope of work is related to monostatic gNB scenario. On the other hand, SF is responsible for collecting the sensing data from the SE and provide the generated sensing result to the SC for the associated request. The same is depicted in the figure 6.X.1-1. As seen, the aspects of configurations, collection, generation and results are managed by the SF, and it is required that the communication aspects of configurations and sensing data and results between various entities are defined. Based on the deployment, the operator may host the Configuration and Sensing data collection and result generation as one entity or as different entities in SF; and when they are different entities; these entities act as Sensing Control Function and Sensing Processing Function of the SF.

Figure 6.X.1-1: High-level communication view

#### 6.X.1.1	Configuration communication

The configuration to the SE is required to be sent by the SF, based on the requirements received from the SC. The communication interface to from the SC to the SF is based on the services exposed by the SF; however, as per the existing methods the SC could use the services of SF directly or through the NEF services. Both the options shall be available.

The SF then accordingly shall send the translated configuration requirements to the SE. The SF shall maintain the co-relation of the requests from the SC and the one sent to one or more SE. However, for the communication from the SF to the SE, AMF acts as the gateway and communicates the configurations to the corresponding SE.

#### 6.X.1.2	Sensing Data communication

Based on the configurations provided to the SE, the SE carries out the sensing activities and collects the sensing data. The sensing data traffic, depending on the sensing configuration determined at the SF, can include considerable volume of data that should be transmitted from the SE to the SF.

The SF when it selects the SE (based on the solution defined for KI3) and while providing the configurations for sensing, shall also provide parameters for the SE to transfer the sensing data to the SF.

Editor's note:	Parameters and interface for transfer of sensing data from gNB to SF is FFS.

The sensing data may include below non-exhaustive list of information.

- Sensing Operation ID, to distinguish among different sensing procedures that may take place in the SE.

- SE ID.

- Sensing data.

Editor's note:	What is transferred as Sending data and any other associated parameters shall be co-ordinated with RAN groups and is FFS.

#### 6.X.1.3	Sensing Results communication

The SF collects sensing data as described in 6.X.1.2 from the SE and generates sensing results that is to be communicated to the SC based on the configuration request received at the SF from the corresponding SC as described in 6.X.1.1.

Once the sensing results are generated, the SF notifies the SC of the results generated by the SF based on the request of the SC.

Editor's note:	It is FFS to identify whether and how larger volume of sensing results are transferred to the SC by the SF.

### 6.X.2	Procedures

#### 6.X.2.0	General

Figure 6.X.2.0-1 describes the various steps covering the aspects described in section 6.X.1.

Figure 6.X.2.0-1: Sensing configuration aspects

- AF (SC) sends a Sensing Service Request with the requirements of sensing to the SF; the details of the provisioning parameters include for example Sensing Service type, target area, etc. The SF evaluates and once the request is validated for authorization shall accept / reject the request.

- The SF provides the information as per below

  - SF uses the Namf N1N2 Message Transfer of AMF and provides the generated request parameters based on the input request parameters from the SC.

  - The AMF shall send the Sensing service container to the SE using the N2 Message request and receives the response from SE a corresponding N2 Message response.

- SF provides an intermediatory status update to the SC based on the request negotiation with the SE.

- Based on the information exchanged as described in step 2, the SE transfers the collected sensing data to the SF.

- The SF prepares sensing results by processing the sensing data it collected from SE(s) of the corresponding request it originated.

- Once the SF prepares the sensing results based on the request of the SC, the SF notifies the AF (SC) and results are provided.

### 6.X.3	Impacts on services, entities and interfaces

SF:

- Provides the configurations to the SE using the N1N2 Message transfer service exposed by AMF.

- Once the sensing result is generated, notify the SC of the sensing results.

AF (SC):

- Provides the sensing requirements to the SF directly or enroute through NEF.

- Receives the notification when the sensing result are generated.

AMF:

- Accepts sensing request from SF using the N1N2 Message Transfer service.

- Provide the Sensing container to the SE.

SE:

- Receive the Sensing container from the AMF.

- Provide the sensing data to the SF based on the sensing activities carried out using the request, parameters received from the SF.

# * * * End of changes * * * *