3GPP TSG-SA2 Meeting #170	S2-2506197

Goteborg, Sweden, August 25 – 29, 2025	(revision of S2-25xxxxx)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.304 | CR | 0555 | rev | - | Current version: | 19.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on additional parameters announcement request |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | LG Electronics |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_ProSe_Ph3 |  |  |  |  |  | Date: |  |  | 2025-07-14 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | LS from CT1 on parameters for ProSe additional parameters announcement request message (C1-254010/S2-2506121) provides the following questions to SA2: Question 1: CT1 would like to get clarification from SA2 regarding what the usage of both relay service code and root relay info for ProSe additional parameter announcement request procedure is. Question 2: CT1 would like to ask SA2 if both relay service code and root relay info can be excluded from the message for protocol efficiency. All the Layer-2 links between Remote UE and Intermediate U2N Relay and between Intermediate U2N Relay and U2N Relay are established per RSC. Therefore, the Additional Parameters Announcement Request sent by the Remote UE can be routed up to the U2N Relay through each Layer-2 link associated with the RSC without the RSC and Root Relay Info. |  |  |  |  |  |  |  |  |
| Summary of change: |  | §6.3.2.5.4 - Remove Relay Service Code and Root Relay Info in step 2 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The usage of RSC and Root Relay Info included in the Additional Parameters Announcement Request may by unclear. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.2.5.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * * Start of 1st Change * * * *

6.3.2.5.4	Additional parameters announcement procedure over 5G ProSe Multi-hop UE-to-Network Relay

Additional parameters announcement procedure outlined in figure 6.3.2.5.4-1 is used by a 5G ProSe Remote UE to request a 5G ProSe UE-to-Network Relay to announce additional parameters (using Model A). The Additional parameters announcement procedure specified in clause 6.5.1.3 is used with the following differences described in the procedure below.

Figure 6.3.2.5.4-1: Procedure for additional parameters announcement for Multi-hop UE-to-Network relaying

1.	5G ProSe Remote UE has discovered a 5G ProSe UE-to-Network Relay for connectivity to the network as described in clause 6.3.2.5.2 and requires additional parameters.

2.	The 5G ProSe Remote UE sends to the 5G ProSe Intermediate UE-to-Network Relay an Additional Parameters Announcement Request to obtain additional parameters.

3-4.	The Additional Parameters Announcement Request is sent up to the 5G ProSe UE-to-Network Relay.

5.	The 5G ProSe UE-to-Network Relay acknowledges receipt of the request in step 4 with an Additional Parameters Announcement Response (Additional_Parameters_Announcement_Request_Refresh Timer).

6-7.	The Additional Parameters Announcement Response is sent to the 5G ProSe Remote UE, relayed via the 5G ProSe Intermediate UE-to-Network Relay.

8.	The 5G ProSe UE-to-Network Relay reuses the Relay Discovery Additional Information message as defined in clause 5.8.3 with the following additional information:

-	Hop-Count: the value is set to 1.

The Destination Layer-2 ID to send the Relay Discovery Additional Information message is selected based on the configuration as described in clause 5.1.4.1.

9.	The 5G ProSe Intermediate UE-to-Network Relay forwards the Relay Discovery Additional Information message. The 5G ProSe Intermediate UE-to-Network Relay updates the Hop-Count as below and may include additional information, Announcer Info and NCGI of 5G ProSe Intermediate UE-to-Network Relay if its NCGI is available:

-	Hop-Count: the value of the received Relay Discovery Additonal Information message is incremented by 1.

-	Announcer Info of 5G ProSe Intermediate UE-to-Network Relay: identity information (i.e. User Info ID) of the 5G ProSe Intermediate UE-to-Network Relay transmitting this message.

-	NCGI of 5G ProSe Intermediate UE-to-Network Relay: indicates the NCGI of the serving cell of the 5G ProSe Intermediate UE-to-Network Relay.

10.	The 5G ProSe Intermediate UE-to-Network Relay forwards the Relay Discovery Additional Information message, with the Hop-Count incremented. The 5G ProSe Intermediate UE-to-Network Relay shall not forward the Relay Discovery Additional Information message if the Hop-Count is bigger than the configured maximum number of hops.

11.	The 5G ProSe UE-to-Network Relay detects new or updated additional parameters.

12-14.	Same as steps 8-10.

If the 5G ProSe Intermediate UE-to-Network Relay connected to 5G ProSe UE-to-Network Relay detects new or updated additional parameters (e.g. due to change of its serving cell, moving into network coverage), it may send a Relay Discovery Additional Information message including the new or updated additional parameters. In this case, the 5G ProSe Intermediate UE-to-Network Relay acts as the 5G ProSe UE-to-Network Relay in figure 6.3.2.5.4-1, i.e. to send Relay Discovery Additional Information message. The 5G ProSe Intermediate UE-to-Network Relay also keeps a record of the additional parameters from the 5G ProSe UE-to-Network Relay.

NOTE:	The NCGI of the 5G ProSe Intermediate UE-to-Network Relay connected to 5G ProSe UE-to-Network Relay can be used as supplementary by the 5G ProSe Remote UE and not provided to the network side from the 5G ProSe Remote UE.

* * * * End of Changes * * * *