SA WG2 Meeting #170	S2-2506219

# Goteborg, Sweden, August 25 - 29, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.273 | CR | 0745 | rev | - | Current version: | 19.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on the input data from NG-RAN for LMF-based AI/ML positioning |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | NTT DOCOMO |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIML_CN |  |  |  |  |  | Date: |  |  | 2025-08-05 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | SA2 sent LS to RAN1, RAN2, and RAN3 requesting clarification on the input data for Case 3b (UL positioning with the LMF-based model) in S2-2405833 and S2-2501341. The action made by each RAN WG is as follows - RAN1 replied to LS from SA2 as S2-2504568 in RAN1#117. - RAN2 noted LS from SA2 in RAN2#127. - RAN3 replied to LS from SA2 as S2-2504577 in RAN3#127. In S2-2504577, RAN3 stated as follows: “the collection of input data for LMF-based AI/ML Positioning in Case 3b can be achieved by reusing the Measurement Information Transfer procedures defined in NRPPa in TS38.455, which supports requesting and reporting the relevant measurements to the LMF for Case 3b. And the details of the measurements for case 3b are still pending and subject to further discussion in RAN1, which is out of RAN3 scope.” In S2-2504568, RAN1 requested SA2 to take the following agreements into account. Agreement For training data collection of AI/ML based positioning, the collected data sample can include the following components: Part A: · channel measurement · quality indicator of channel measurement · time stamp of channel measurement Part B: · ground truth label (or its approximation) · quality indicator of label · time stamp of label Note: “Part A” and “Part B” terminologies are only for RAN1 discussion purpose, and may not be used in specification. Note: contents in Part A and Part B may or may not be generated by different entities. Note: Part A and/or Part B, and their contents may or may not apply for each case FFS: detailed definition of channel measurement Agreement For Rel-19 AI/ML based positioning, for Case 3b, in addition to path-based measurement that is referring to the measurement in the existing specifications (up to Rel-18), additionally support the following enhancement to the measurement, - The measurement is composed of Nt' values of the estimated channel response in time domain. The Nt’ values are selected from a list of Nt consecutive channel response values, which have timing granularity T. - The timing information for the Nt' values are reported with a timing granularity T, where T=2kxTc. k represents the timing reporting granularity factor. Tc is the basic time unit for NR. - The associated measurement (e.g., power if reported) corresponds to the measurement for the reported Nt' values. · The timing information is defined relative to a reference time, same as the path-based measurement. - The Nt’ selected time domain channel measurement values are expected to be those with the highest power. - The starting time of the list of Nt consecutive values is determined as: starting time = first detected path rounded down with timing granularity T. - LMF can signal parameter values of Nt, Nt', k to gNB via NRPPa. Candidate set values: - Nt'<=24. FFS: Nt' values. - Nt = {32, 64, 128} - FFS: k - The gNB/TRP may use different Nt', Nt and/or k values other than the signalled parameter for measurement reporting. In this case, it’s up to LMF implementation to process the reported measurement · FFS: whether transmit offset from gNB to LMF Note: measurement by UE is a separate discussion. Note: the purpose of the time domain channel measurements, such as for Rel-19 AI/ML based positioning, is not specified In summary, the input data for Case 3b shall consist of channel measurements (e.g., UL SRS-TDCT and UL SRS-TDCP as defined in TS 38.215), quality indicators of the channel measurements, and timestamps of the channel measurements. As RAN1 has completed its work on AI/ML for positioning, SA2 specifications should capture the input data from NG-RAN for LMF-based AI/ML positioning in TS 23.273, based on the information above. Since the agreed input data for Case 3b are all included in the measurement results specified in Clause 8.10.2.3 of TS 38.305, this clause should be referenced in a note. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The input data from NG-RAN for LMF-based AI/ML positioning is clarified. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Ambiguity in the input data from NG-RAN for LMF-based AI/ML positioning |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.18.0 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

********** First Change **********

## 5.18	Support for UE positioning based on a ML Model at the LMF

### 5.18.0	General

The LMF may calculate the UE location and estimate the achieved accuracy by using LMF-based AI/ML Positioning. When receiving the request from AMF for determining a UE location, the LMF selects an appropriate positioning method as described in clause 5.2 to determine the result of the positioning. The result of the positioning may be calculated by using LMF-based AI/ML Positioning ML model supported by LMF. The LMF collects input data from UE or NG-RAN for the LMF-based AI/ML Positioning to perform location calculation and provide the location to the consumer.

NOTE 1:	Whether to select LMF-based AI/ML Positioning for location result calculation is determined by LMF.

NOTE 2:	The specific measurement data collected by LMF from NG-RAN for LMF-based AI/ML Positioning are in the scope of RAN specifications and not in the scope of this specification.

NOTE 3:	The input data from NG-RAN for the LMF-based AI/ML Positioning consists of measurement results as specified in clause 8.10.2.3 of TS 38.305 [9].

The ML model that is used for LMF-based AI/ML Positioning may be trained by LMF. The trigger for data collection required for model training in LMF is up to implementation. LMF collects input data from UE for ML model training as described in clause 6.22.2. The LMF collects input data from NG-RAN for ML model training as described in clause 6.22.3.

********** End of Changes **********