SA WG2 Meeting #170	S2-2506222

Goteborg, Sweden, 25 – 29 August 2025

Title:	[DRAFT] LS on UE data collection

Response to:	-

Release:	Rel-20

Work Item:	FS_AIML_CN_Ph2

Source:	Nokia [will be SA2]

To:	RAN2, RAN3

CC:	RAN, RAN1, SA5, SA3

Contact person:	Yannick Lair

yannick.lair@nokia.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

# 1	Overall description

As part of FS_AIML_CN_Ph2 study, SA2 continues investigating potential solutions for UE data collection to address requirements from RAN. Candidate solutions so far are captured in 3GPP TR 23.700-04.

Many candidate solutions rely on a UE model training server (OTT server) providing a request for data collection to some 5G Core Network function responsible for data collection, such a request from the server indicating the target for data collection (e.g. individual UEs, set of UEs), a possible geographical area for data collection, a time window for data collection, and some indication of the data to be collected or the use case for which data needs to be collected. The 5G Core Network function responsible for data collection would then relay the request and the data collection configuration to either RAN or the UE in order to configure UE data collection, together with information on the user plane endpoint where the collected data should be sent to.

In order to fully scope SA2 work and to finalize solutions selection from an SA2 and system architecture perspective, SA2 would need some further clarifications and guidance from RAN2.

1 – UE data collection and UE states

Question 1.1

Some solutions in SA2 TR (e.g. solutions #1, #5, #18 and #19) assume that data collection procedure can be triggered from Idle mode UEs, in which case the UE would be first paged while in Idle mode and would afterwards be provided with data collection configuration when entering Connected mode. SA2 would like to ask RAN2 whether triggering data collection from UEs in Idle mode is required or if waking up UEs for the sole purpose of data collection should rather be avoided.

Question 1.2

Many solutions assume the UE data collection will be triggered for some specific geographical area.

- Does RAN2 confirm this assumption?

- In that case what is the expected granularity (cell level, cells of a given frequency band within a geographical area, tracking area level….)?

- Can RAN2 confirm that data collection continuation is required upon UE connected mode mobility, as long as the UE stays within the initial geographical area?

Question 1.3

If answer to question 1.2 is positive, does RAN2 identify any impact from data collection configuration perspective, e.g. is the data collection configuration going to be different depending on RAN nodes?

Question 1.4

When the UE, collecting data to be sent to the UE model training server (OTT server), does not have any other traffic, should the UE move to Idle mode or should the UE continue to collect and report data for as long as data collection is configured?

2 – Triggers for UE data collection

Question 2.1

The different end-to-end solutions in the SA2 TR consider different entities capable of triggering UE data collection procedure: most of the solutions assume that the UE side model training server (OTT server) would trigger the overall data collection procedure (including data collection configuration and data transfer) via the core network, while a couple of solutions consider that the UE would trigger data collection request. SA2 would like to ask RAN2 whether the trigger for UE data collection should be from UE side model training server (OTT server), from the UE, or from both.

3 – Selection of the UEs for UE data collection

Question 3.1

In order to decide how to select UEs for UE data collection, SA2 would like to know if the UE data collection is a functionality that will be triggered by the UE model training server on individual UEs or if it will be triggered on a set of UEs, for example on UEs with a Type Allocation Code (TAC) corresponding to a target set of TACs, i.e. UEs for a specific chipset.

Question 3.2

To better understand where the UE selection should be performed for the case where data collection needs to happen on a set of UEs, SA2 would like to understand if RAN would need to be active in UEs selection, e.g. if the selection of UEs would eventually depend on e.g. radio conditions.

4- Selection and configuration of the gNBs for UE data collection

Question 4.1

If a 5G Core Network function responsible for data collection were to configure RAN Nodes for UE-side data collection, what is the minimum information required for the RAN Node to configure the UE(s) with reference signal (RS) configurations and measurement configurations for standardized quantities?

# 2	Actions

To RAN2, RAN3

ACTION: 	SA2 requests RAN2, RAN3 to answer SA2 questions above and provide any additional feedback that would guide SA2 on finalizing the solution for UE data collection.

# 3	Dates of next TSG SA WG 2 meetings

SA2#171		October 13-17, 2025      		Wuhan, China

SA2#172		November 17-21, 2025		Dallas, US