3GPP TSG SA WG2 Meeting #171											S2-2506226

Goteborg, Sweden, 25 – 29th August, 2025

Title:	6G system architecture to support of NTN

Source:	Thales, Novamint, ST Engineering iDirect

Agenda Item:	20.6.1 FS_6G_ARC

Document for:	Discussion/Decision

# Introduction

This contribution presents some considerations on the 6G system architecture for the support of NTN.

# Recall of the Study item

## Study item objectives

Among the objectives of the study, here below are highlighted in yellow the objectives relevant to satellite/NTN.

WT#2: Study migration and interworking, including

-	How to support migration to 6GS

-	How to support interworking with 5GS

-	Whether and how to support interworking with EPS

-	How to support interworking between 6GS and 4G/5G NTN/satellite access that use EPS/5GS.

NOTE 2:	Interworking with 2G/3G are not considered in this study. Interworking between 6GS and 5GS is assumed to work even if the UE has previously registered in 2G, 3G or 4G.

NOTE 3:  The detailed migration study scope will be coordinated and aligned with RAN.

..

WT#7: Study how to support 6G RAT for NTN, based on RAN decision, and support service continuity aspects.

NOTE 8:	The detailed scope for WT#7 will be coordinated and aligned with RAN

# Discussion on the study

As per the RAN led study on 6G Scenarios and requirements (RP-250810) and the RAN1 led study on 6G radio (RP-251881), RAN aims at a harmonized 6G Radio design for TN and NTN, including their integration. Hence, it is expected that the 6G RAT will support NTN and therefore the 6G system should be designed to provide a radio access technology supporting different deployment scenarios among which TN and NTN ones.

Proposal 1: 6GS shall be defined to support both Terrestrial and non-terrestrial network components

Given that the deployment of IoT-NTN and NR-NTN is on-going, the related service will likely be available for a long time even beyond the deployment of initial 6G deployment therefore

Proposal 2: Interworking between 6GS and 5G IoT-NTN/NR-NTN access shall be ensured at least in terms of mobility

A number of NTN related use cases have been agreed as part of the SA1 led Study on 6G Use Cases and Service Requirements (SP-241391) resulting in the TR 22.870. They require a number of features which may impact the 6G system architecture:

- GNSS independent communication service provided by NTN

- GNSS independent positioning service provided by NTN

- Public Warning Service to indoor

- Emergency messaging from/To Indoor

- Broadband connectivity to VSAT mounted satellites

- Interworking between NTN and TN for a resilient 6G network

- Off load of broad/multicast to NTN for energy efficiency purposes

- Quasi seamless NTN-TN service continuity

- Support of a diversity of devices

Other use cases are currently being considered in SA1 led 6G study. They also have potential impact on the system architecture:

- Mesh connectivity via satellite: (Relay) UE – Satellite - (Relay) UE communication

- Energy efficient coverage provision

- ISAC via NTN to assess aeronautic/space context

- Edge computing on board: store and forward (non-real time) or ISL (real time)

- Multi-tenant satellite network

- Cooperative UE for reliable uplink transmission in NTN

Proposal 3: As part of SA2 led study on 6G architecture, the system architectural impacts related to the NTN related use cases introduced in TR 22.870 should be addressed in priority

The focus of the study should be on 6G system and it should conclude with preferred solutions for each of the key issues.

However, as part of the study, some of the identified solutions could be considered for 5G

Proposal 4: The applicability of some solutions identified as part of SA2 led study on 6G architecture, to 5G advance network should not be precluded

# Conclusion

Proposal 1: 6GS shall be defined to support both Terrestrial and non-terrestrial network components

Proposal 2: Interworking between 6GS and 5G IoT-NTN/NR-NTN access shall be ensured at least in terms of mobility

Proposal 3: As part of SA2 led study on 6G architecture, the system architectural impacts related to the NTN related use cases introduced in TR 22.870 should be addressed in priority

Proposal 4: The applicability of some solutions identified as part of SA2 led study on 6G architecture, to 5G advance network should not be precluded

END