3GPP TSG-SA2 Meeting #170	S2-2506233

# Stor-Göteborg, Sweden, 25th Aug 2025 - 29th Aug 2025	(revision of S2-250xxxx)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 6340 | rev | - | Current version: | 19.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on always-on PDU Session handling when the S-NSSAI of an always-on PDU Session is subject to area restrictions and I-SMF/V-SMF is inserted |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | LG Electronics |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNS_Ph3, TEI19 |  |  |  |  |  | Date: |  |  | 2025-08-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | When I-SMF/V-SMF is inserted, the area restrictions of a slice is handled by the I-SMF/V-SMF as described in clause 5.15.18. Therefore, for always-on PDU Session, reactivation of the PDU Session should be done by I-SMF/V-SMF. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarify that when I-SMF/V-SMF is inserted for always-on PDU Session, I-SMF/V-SMF reactivates PDU Session when the UE enters NS-AoS. Terminology alignment (from "PDU session" to "PDU Session"). |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unclear specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.6.13 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * * Start of 1st Change * * * *

5.6.13	Always-on PDU Session

An always-on PDU Session is a PDU Session for which User Plane resources have to be activated during every transition from CM-IDLE mode to CM-CONNECTED state.

Based on an indication from upper layers, a UE may request to establish a PDU Session as an always-on PDU Session. The SMF decides whether the PDU Session can be established as an always-on PDU Session. In Home Routed roaming case, based on local policies, the V-SMF shall be involved to determine whether the PDU Session can be established as an always-on PDU Session. If the S-NSSAI of an always-on PDU Session is subject to area restrictions (as defined in clause 5.15.17 and clause 5.15.18), the SMF, upon detecting that the UE enters the NS-AoS of the S-NSSAI, shall reactivate the always-on PDU Session by executing the Network Triggered Service Request procedure for the case of UE in CM-CONNECTED mode (steps 3a to 4a only) in clause 4.2.3.3 of TS 23.502 [3].

NOTE:	When I-SMF/V-SMF is inserted for the PDU Session it is the I-SMF/V-SMF to handle the area restrictions for the S-NSSAI. The I-SMF/V-SMF reativates the always-on PDU Session and SMF/H-SMF is not involved in this procedure.

If the UE requests the 5GC to modify a PDU Session, which was established in EPS, to an always-on PDU Session after the first inter-system change from EPS to 5GS, the SMF decides whether the PDU Session can be established as an always-on PDU Session based on the procedure described above.

The UE shall request activation of User Plane resources for always-on PDU Sessions even if there are no pending uplink data for this PDU Session or when the Service Request is triggered for signalling only or when the Service Request is triggered for paging response only.

If the UE has one or more established PDU Sessions which are not accepted by the network as always-on PDU Sessions and the UE has no uplink user data pending to be sent for those PDU Sessions, the UE shall not request for activating User Plane resources for those PDU Sessions.

* * * * End of Change * * * *