3GPP TSG-SA2 Meeting #170	S2-2506240

Gothenburg, Sweden, August 25-29 2025

Source:	Nokia

Title:	KI#2 new solution: Location and time-based adjustments

Document for:	Approval

Agenda Item:	20.4.1

Work Item / Release:	FS_EnergySys_Ph2 / Rel-20

Abstract: This document proposes that the PCF can implement time and location-based policies for QoS adjustment

# 1.	Introduction

This solution proposes that the PCF is configured with time and location-based policies by the operator to achieve energy-related goals. For instance the UE is using a gNB that operates on battery and needs to save battery, so lower energy consumption could be induced by reducing the overall data volume transmitted at the gNB, or it is using a particularly advantageous power source at a specific time of day (e.g. cheaper due to e.g. solar energy coming on stream that is particularly cheap on a given day of the year due e.g. to excess production, see for instance negative-energy-price-in-europe ), so more data volume transmission is desirable so the UE-AMBR constraints may be lifted or UE-AMBR may be increased. These rules can be updated at runtime by the operator. The location is derived from the ULI of the UE and operator logic determines what policies apply at a given location and time.

There is no expected normative impact in SA2. It is proposed this could be provided as an informative annex in e.g. 23.501, from SA2 perspective.

# 2. Text Proposal

It is proposed to capture the following changes in TR 23.700-67.

# * * * * First change * * * *

# 6	Solutions

## 6.0	Mapping of Solutions to Key Issues

Editor's note:	The number of columns of Key Issues will be updated according to clause 5.

Table 6.0-1: Mapping of Solutions to Key Issues

|  | Key Issues |  |  |
| --- | --- | --- | --- |
| Solution# | 1 | 2 | 3 |
| 1 | X |  |  |
| 2 | X | X |  |
| 3 |  | X |  |
| 4 |  | X |  |
| 5 |  | X |  |
| 6 |  | X |  |
| 7 |  | X |  |
| x |  | x |  |

# * * * * Third change (All text new) * * * *

## 6.X	Solution #X: Time-based and location-based QoS adjustments due to energy-related policies

### 6.X.0	High-level solution principles

This solution proposes the following principles to be agreed:

1)	The PCF is provisioned with QoS policies (e.g. changing the UE-AMBR, Sessions AMBR, MFBR, GBR, if possible, for specific application domains) that are location and time dependent to achieve energy goals.

2)	The location of the UE is determined e.g. based on ULI reported to PCF by SMF via AMF.

3) 	What policy applies at a given location and time is determined by the operator

4)	SA2 specifications impact is non normative as existing specifications can be used. An informative annex could be used or notes in specific parts of the specifications

### 6.X.1	Description

This solution proposes that QoS adjustments can be triggered by PCFs (both SM and AM policies could be impacted) based on the location or time of day, or vent driven if a specific even occurs.

For instance the UE is using a gNB that operates on battery and the gNB needs to save battery. Or, a UE is using a particularly advantageous power source at a specific time of day (e.g. cheaper due to e.g. solar energy coming on stream that is particularly cheap on a given day of the year due e.g. to excess production, see for instance negative-energy-price-in-europe ), so more data volume transmission is desirable at these rates (e.g. for downloads etc) so the UE-AMBR constraints may be lifted or UE-AMBR may be increased. These rules can be updated at runtime by the operator. The location is derived from the ULI of the UE and operator-specific logic determines what policies apply at a given location and time.

The following solution components are envisaged:

-	The PLMN operator is responsible for the overall energy-related QoS policy on a location and time basis. It provisions and updates QoS policies accordingly at PCFs.

-	AM PCF and SM PCF obtains the necessary location information for the provisioned location-based policies if any (e.g. the ULI of UE).

-	The AM PCF and SM PCF update the QoS policies for the UE depending on time and location.

### 6.x.2	Procedures

The procedures are not impacted.

### 6.x.3	Impacts on existing services, entities and interfaces

The impacts are:

-	PCF:

-	Determine the QoS policies for the UE based on location and time (No standard impact expected)

-	OAM

-	Configure/update the PCFs as necessary based on operator logic (no standard impact expected)

# * * * * End of change * * * *