3GPP TSG-SA WG2 Meeting #170	S2-2506265

Goteborg, Sweden, August 25 – 29, 2025

Source:	CATT

Title:	KI#3 interim agreements

Document for:	Approval

Agenda Item:	20.4.1

Work Item / Release:	FS_EnergySys_Ph2 / Rel-20

Abstract of the contribution: The contribution proposes interim agreements for KI#3.

1. Introduction

For KI#3 “NF (re)selection and/or UP path adjustment based on energy related information”, 5 solutions have been included in the TR, which are summarized and analysed in the following table:

| Sol. | High level principles | Whether and how to perform NF (re)selection based on energy related information |  |  | Whether and how to perform UP path adjustment based on energy related information |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Whether and how to extend NF profile (e.g. include energy related information) | Which energy related information is to be considered | Whether and how to enhance NF (re)selection procedure | Whether and how to trigger UP path adjustment | What information is to be considered |
| 19 | Take energy related information into account for AMF (re-)selection |  |  |  |  |  |
| 20 | 1. The NF profile is augmented with Energy Priority Information. 2. The NF profile is augmented with a Schedule of Energy Priority Information to enable providing a schedule of Energy Priority information, if one exists, instead of requiring additional updates at a time when the Energy Priority information is known to change. 3. The NF profile is augmented with Schedule of EnergySaving/not EnergySaving State (See TS 28.310 [9]) and the EnergySaving/NotEnergySaving state attributes to enable the NF service consumer and NRF to know when the NF may be taken out of service according to a schedule or to enable the NRF to know a NF is temporarily out of service to save energy. 4. The UPFs in the UP path are selected by the SMF taking into account the Attributes above in the NF profile, if any are present, based on a UP Path Selection policy. | Add the following attributes to NF profile: - EnergySaving /Not EnergySaving State - Schedule of EnergySaving/not EnergySaving - Energy Priority Information. - Schedule of Energy Priority Information | NA | No |  |  |
| 21 | - The AF, PCF or SMF takes into account of energy related information for DNAI (re)selection to adjust the UP path of the PDU Session, during the AF influence on traffic routing procedure. - The AF, PCF or SMF subscribes from the EIF for energy related information (e.g. energy consumption and corresponding data volume, energy efficiency, and/or renewable energy consumption ratio) per DNAI per Service Data Flow or per DNAI per PDU Session, which is calculated by the EIF based on the collected Energy Consumption information from the OAM and UPF (via the SMF). - The AF or PCF selects/updates the DNAI list or the SMF selects the target DNAI taking into account the energy related information per DNAI per Service Data Flow or per DNAI per PDU Session, and the SMF reconfigures the user plane of the PDU Session towards the selected DNAI. NOTE: The adjustment of UP paths is determined based on operator policy, and frequent adjustment of UP paths for the PDU Session should be avoided. |  |  |  | Taking energy related information into account to (re)select DNAI to trigger UP path adjustment, during the AF influence on traffic routing procedure | energy related information (e.g. energy consumption and corresponding data volume, energy efficiency, and/or renewable energy consumption ratio) per DNAI per Service Data Flow or per DNAI per PDU Session |
| 22 | - the energy usage information of renewable energy per slice or a set of slices and corresponding priority of renewable energy per slice in NF profile in NRF; - the EIF can obtain the energy resource usage information per slice or for a set of slices from OAM. The information of the renewable energy included in NF profile can be configured by operator policy. The SMF can subscribed to EIF to obtain the energy resource usage information per slice or for a set of slices, i.e., whether the renewable energy is used or not per slice. The SMF can be notified by EIF whether renewable energy per slice is used and then the SMF is triggered to query NRF by considering whether to reselect a UPF or not for the slice related to the existing PDU Sessions. | Add the following attributes to NF profile: - Indication of renewable energy is used or not per slice per application or for a set of slices; - energy efficiency. | energy usage information of renewable energy per slice or a set of slices | The SMF (re)selects a UPF based on: 1) energy related information, e.g. whether renewable energy is used for the slices, and 2) NF profile, i.e. the new attribute together with the existing parameters (NF Priority, NF capacity, NF load). |  |  |
| 23 | - Multiple UPFs (not necessarily the same from energy perspective) are deployed in the operator network and each PDU session has a UP path in the 5GC. - The operator may have various energy objectives such as maximizing renewable energy ratio or minimizing energy consumption. The objectives may be triggered by e.g. operator's decision, time, unexpected energy consumption of PDU sessions, etc. - Upon a trigger, the SMF based on the operator's energy objective and the energy-related information adjusts the UP path of some of the already established PDU sessions; i.e., reselects appropriate UPFs for the PDU session (subject to SSC mode) to optimize the operator's energy objective. - The newly adjusted UP paths are implemented in the network using the existing mechanisms defined in clause 4.3.5 of TS 23.502 [3]. | NF profile of UPFs including the Energy Priority Information |  |  | Triggered by configuration and energy-related information, the SMF (re)configures UP path by using UP path management procedure as defined in clause 4.3.5 of TS 23.502. | 1) configured operator's energy objective (e.g. maximizing renewable energy ratio, or minimizing energy consumption) and triggers (e.g. activation time window, energy consumption beyond a threshold, manual activation by the operator) 2) Energy-related information |

Based on the above summary and analysis, KI#3 interim agreements are proposed.

2. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-67.

* * * Start of Change * * * *

7.1.Y	Agreed Principles for KI#3

Editor's note:	This clause will include the principles that are agreed as work progresses for the specific KI#Y. This may be populated directly or e.g. also when a topic in clause 7.2.Y gets resolved and a principle is agreed.

The interim agreements on principles for KI#3 are as follows:

-	UPF may be (re)selected considering the following energy related information of the UPF, based on operator policy:

-	energy consumption.

-	renewable energy consumption ratio.

-	energy efficiency.

-	EnergySaving State.

-	Energy Priority Information, which may be determined based on one or more of the above energy related information.

-	UP path(s) adjustment for PDU Session(s) may be triggered via DNAI (re)selection and/or UPF (re)selection, taking into account energy related information, e.g. the overall energy consumption, renewable energy consumption ratio and/or energy efficiency of the UP path(s) for transmitting the service data flow.

* * * Next Change * * * *

7.2.Z	Topics for further consideration for KI#3

Editor's note:	This clause will include the topics for further consideration as work progresses for the specific KI#Z. Eventually this clause should only contain topics for further consideration that did not result in agreements (i.e. in agreed principle(s) in a clause 7.1.Z) and can either be then marked as not pursued or postponed to a future Release.

The following topics/principles are for further consideration:

-	Whether and how to extend NF profile (e.g. include energy related information) to support NF (re)selection.

* * * End of Change * * * *