3GPP TSG-SA WG2 Meeting #169	S2-2506270

Goteborg, Sweden, 25th Aug 2025 - 29th Aug 2025

Source:	OPPO

Title:	Interim agreement principles for KI#1

Document for:	Approval

Agenda Item:	20.3.1

Work Item / Release:	FS_AIML_CN_Ph2 / Rel-20

Abstract of the contribution: The contribution proposes the interim agreement principles for KI#1.

- Discussion

21 solutions for KI#1 are already captured in the TR 23.700-04 to study how to support the standardized transfer of standardized data over UP for UE data collection. Although there are still some details that need to be further discussed. Some basic principles have been consolidated including which NF in the 5GC should support the full visibility and full controllability of the UE data collection, how to send the data collection NF UP information to UE to support the data transfer via UP connection, etc. Based on the current progress, this paper proposes some interim agreemnt principles for the KI#1.

- Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-04.

* * * Start of Change * * * *

7	Interim agreements

7.1	Agreed Principles

7.1.Y	Agreed Principles for KI#1

Editor's note:	This clause will include the principles that are agreed as work progresses for the specific KI#Y. This may be populated directly or e.g. also when a topic in clause 7.2.Y gets resolved and a principle is agreed.

Supporting transfer of data over UP for UE data collection is based on the following principles:

  - A new network function in the 5GC named Data Collection Function (DCF) is introduced to support the UE data collection.

  - The UE side server/UE model training entity server (UMTES) configure the DCF about the data collection information including the Event ID and the corresponding data type the UE need to be measured and reported.

  - The DCF provision the UE Data Collection configuration to the UE via control plane signalling including the Event ID and the corresponding data type the UE need to be measured and reported.

  - The UE side server/UE model training entity server sends the UE data collection request to the DCF including the AoI and the Event ID.

  - The DCF selects the UEs to be used for the data collection and establish the UP based connection with the UEs.

  - The DCF sends the Event ID to the UE via UP to tigger the UE to perform the data measurement with the RAN.

  - The UE sends the measured data to the DCF via UP.

  - The DCF verify and match the UE transmitted data based on the data collection information stored in the DCF and the measured data reported from UE.

  - The DCF expose the data collected from UE to the UE side server/UMTES.

* * * End of Change * * * *