3GPP TSG-SA WG2 Meeting #170	S2-2506283

Goteborg, SE, Aug 25 - 29, 2025	(revison of S2-2504604)

Source:	OPPO

Title:	New Solution for KI#5: Sensing result delivery to the consumer

Document for:	Approval

Agenda Item:	20.2.1

Work Item / Release:	FS_Sensing_ARC / Rel-20

Abstract of the contribution: The contribution proposes a new solution on Sensing result delivery to the consumer.

- Discussion

KI#5 have been defined in TR 23.700-14 as below:

This key issue addresses the exposure of sensing results to support sensing services. The aspects to be studied in this key issue include: - Study what types of sensing results with or without the sensing contextual information defined in TS 22.137 [2] could be exposed from the Core Network to the sensing service consumer. - How the Core Network exposes sensing results to the sensing service consumer (e.g., one time, periodic or event trigger).

For sensing result exposure, new sensing result may be derived from sensing data. Data collection often triggers sensing measurements. However, since multiple requests from diverse applications may seek identical results, reusing stored outcomes avoids redundant processing. This approach not only streamlines operations but also aligns with energy efficiency goals by minimizing unnecessary data generation and computation.

Sensing data storage preserves historical and real-time environmental context. Stored data ensures consistency for cross-service validation (e.g., AI training, network optimization) and supports regulatory requirements. For result exposure, centralized storage simplifies access via standardized APIs, allowing third parties or internal systems to retrieve processed insights (e.g., obstacle maps, interference patterns) without raw data duplication. It also facilitates on-demand querying, reducing redundant data collection. By decoupling storage from processing, stored datasets enable scalable, repeatable analytics, ensuring high-quality sensing results are dynamically exposed for applications like autonomous mobility or predictive network maintenance.

Sensing services, as a type of data service, can share the same data collection and capability exposure architecture with other data services (e.g., AI, positioning). However, due to the unique requirements of sensing services, there are also dedicated operations like sensing entity selection and sensing data processing. Thus, sensing service management is jointly implemented by the DSMF (Data Service Management Function) and SSMF (Sensing Service Management Function). The DSMF manages unified data collection and capability exposure. The SSMF handles sensing entity selection, sensing result generation, and storage.

This solution primarily focuses on a DSMF-based capability exposure framework, leveraging shared infrastructure for efficiency while ensuring specialized processing for sensing-specific tasks through the SSMF. By integrating sensing into the broader data service ecosystem, operators avoid redundant architectures while maintaining tailored support for sensing requirements like real-time entity coordination and environmental context handling.

- Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-14.

* * * Start of Change * * * *

# 6	Solutions

## 6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

|  | Key Issues |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Solutions | #1 | #2 | #3 | #4 | #5 | #6 |
| #1 | X |  | X | X | X |  |
| #2 |  | X |  |  | X |  |
| #3 | X | X | X | X | X | X |
| #4 | X |  |  |  |  |  |
| #5 | X |  |  | X |  |  |
| #6 | X | X | X |  |  |  |
| #7 | X |  |  |  |  |  |
| #8 |  | X |  |  |  |  |
| #9 |  | X |  |  |  |  |
| #10 |  | X |  |  |  |  |
| #11 |  | X |  |  |  |  |
| #12 |  | X |  |  |  |  |
| #13 |  |  | X |  |  |  |
| #14 |  |  | X |  |  |  |
| #15 |  |  | X |  |  | X |
| #16 |  |  | X |  |  |  |
| #17 |  |  | X |  |  |  |
| #18 |  |  |  | X |  |  |
| #19 | X | X |  |  | X |  |
| #20 |  |  |  |  |  | X |
| #X | X |  |  | X |  |  |

* * * Next Change (All new text)* * * *

## 6.X	Solution #X: Sensing result delivery to the consumer

### 6.X.0	High-level solution Principles

This solution is for Key Issue#1 and Key Issue#5 on Sensing result delivery to the consumer.

- DSMF (Data Service Management Function) is introduced to manage sensing data and the associated information collection and sensing result exposure.

- SSMF (Sensing Service Management Function) is introduced to handle Sensing Entity selection, sensing result generation, and storage.

- The sensing result is stored in the SSMF or a centrally deployed NF.

- The sensing service consumer(s) can be a UE, AF, gNB, or CN NF.

- The sensing result is transferred over Control Plane (CP) or User Plane (UP) / Data connection for storage and exposure.

- If there is available sensing result matching the service request in the centrally deployed data storage NF, it can be provided to the consumer without triggering sensing procedure to the Sensing Entity.

### 6.X.1	Description

DSMF (Data Service Management Function) is introduced to manage sensing data and the associated information collection and sensing result exposure and SSMF (Sensing Service Management Function) is introduced to handle Sensing Entity selection, sensing result generation, and storage. The sensing result can also be stored in a centrally deployed NF. The sensing service consumer(s) can be a UE, AF, gNB, or CN NF.

To enable sensing result exposure, the DSMF needs to:

- Determine the sensing result provider: SSMF or centrally deployed NF.

- Determine the sensing result transferring mode: Control Plane (CP) or User Plane (UP) / Data connection.

- Contral delivering the sensing results from the SSMF to the sensing service consumer(s).

To enable sensing result storage, the DSMF needs to:

- Determine the sensing result transferring mode: Control Plane (CP) or User Plane (UP) / Data connection.

- Contral delivering the sensing results from the SSMF to the sensing sensing result storage NF.

If User Plane (UP) / Data connection is required to deliver sensing results to the consumer and the sensing result storage NF, a User Plane (UP) / Data connection needs to be established before transferring the sensing result.

DSMF determines the sensing result provider based on the following considerations:

- Sensing consumer information: location, whether selected as Sensing Receiver, capability of acting as Sensing Receiver

- Sensing Receiver information: location

- SSMF load for result generation

- SSMF selected sensing result producer

- Operator configuration

DSMF determines sensing result transferring mode based on the following info:

- Capability of the sensing result provider for transferring sensing result over UP/data connection

- Capability of the sensing service consumer or the centrally deployed NF for sensing result storage for transferring sensing result over UP/data connection

- Sensing result format

- Sensing service type

- Operator configuration

If there is available sensing result matching the service request in the centrally deployed data storage NF, it can be provided to the consumer without triggering sensing procedure to the Sensing Entity.

### 6.X.2	Procedures

Figure 6.X.2-1: Sensing result exposure and storage service flow

- DSMF receives a sensing service request, and the service request is successfully authorized. The request includes:

- Consumer ID/Address

- Sensing Service Type

- Expected Sensing Result，including sensing result format

- Capability of the sensing service consumer for transferring sensing result over UP/data connection

If the consumer is UE, the service request is transmitted via AMF.

If the consumer is AF, the service request is transmitted via NEF.

- DSMF checks with the centrally deployed data storage NF if there is available sensing result matching the service request, by sending the information received from the Sensing Consumer.

If there is matching sensing result, data collection/storage entity responds to DSMF including one or multiple following parameters:

- Sensing result

- Indication of transferring sensing result over UP

- data collection/storage entity address

- If no sensing result or indication of transferring sensing result over UP or data storage entity address is provided in step 3, DSMF sends the sensing service request to the SSMF, including what is received in step 2

- If there’s no matching sensing result as requested in the sensing service request, the DSMF performs sensing transmitter/receiver discovery and selection.

- SSMF responds to DSMF one or multiple of following parameters:

- sensing result (in case there’s available sensing result)

- Indication of transferring sensing result over UP/data connection

- the receiver ID/address (in case there’s no available sensing result)

- Sensing result provider ID/address

- DSMF determines the sensing result provider.

- DSMF determines sensing result transferring mode.

- If sensing result is already received at step 3 or step 6, DSMF responds to sensing service consumer of the sensing result; otherwise, it responds with the Indication of transferring sensing result over UP/Data connection and sensing result provider ID/Address

- DSMF configures the sensing result provider with one or multiple following parameters:

- Indication of transferring sensing result

- Indication of transferring sensing result over UP/Data connection

- The sensing service consumer ID/Address

- If Indication of transferring sensing result over UP/Data connection is received, and there’s no available UP/Data Connection, new UP/Data Connection is established between sensing result provider and sensing service consumer.

- SSMF triggers the sensing transmitter/receiver to perform sensing.

- Sensing data collection and result generation are performed.

- If Indication of transferring sensing result over UP/Data connection is received, the sensing result is transferred to the sensing service consumer and the sensing result storage entity over UP/data connection. Sensing result is stored. Step 14, 15 and 17 are skipped.

- SSMF notifies the DSMF about the sensing result.

- DSMF notifies the sensing service consumer about the sensing result.

- If DSMF determines the sensing result provider is the SSMF in step 6, the sensing result is stored at SSMF, and step 17-18 are skipped.

- Sensing receiver/SSMF notifies data storage NF about the sensing result.

- If DSMF determines the sensing result provider is data storage entity in step 6, sensing result is stored at data storage NF.

### 6.X.3	Impacts on services, entities and interfaces

DSMF:

  - Determine the sensing result provider(s).

  - Determine the sensing result transferring mode.

  - Contral delivering the sensing results from the sensing result provider(s) to the sensing service consumer(s).

  - Determine the sensing result storage entity(ies).

  - Contral delivering the sensing results from the sensing result provider(s) to the sensing result storage NF.

SSMF:

  - Support sensing result generation

  - Support sensing result storage

Receiver:

  - Support sensing data collection to SSMF

  - Support sensing measurement

Data storage NF:

  - Support sensing result storage

* * * End of Change * * * *