3GPP TSG-SA WG2 Meeting #170	S2-2506285

Goteborg, SE, Aug 25 - 29, 2025	(revison of S2-2504601)

Source:	OPPO

Title:	New Solution for KI#1: SF distribution in the general architecture

Document for:	Approval

Agenda Item:	20.2.1

Work Item / Release:	FS_Sensing_ARC / Rel-20

Abstract of the contribution: The contribution proposes a new solution on how the Sensing Function is distributed in the general architecture.

- Discussion

KI#5 have been defined in TR 23.700-14 as below:

This key issue will address the general system architecture and function enhancements required to provide end-to-end support for sensing. The general architecture aspects that need to be studied include any new and/or existing functions in the Core Network and their interactions that should be involved in providing support for sensing. Potential privacy requirements if identified will be considered for this key issue. NOTE: Solutions for other key issues may also impact general architecture aspects covered by this key issue.

Terminology of Sensing Function is defined as below:

Sensing Function: Indicating the logical function which is involved to support Sensing Service. NOTE: The Sensing Function cannot be a Sensing Entity.

However, there’s no clear definition on what are the logical functions of the Sensing Function.

This paper defines the logical functions of Sensing Function and how the SF are distributed in the overall architecture.

- Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-14.

* * * Start of Change * * * *

# 6	Solutions

## 6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

|  | Key Issues |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Solutions | #1 | #2 | #3 | #4 | #5 | #6 |
| #1 | X |  | X | X | X |  |
| #2 |  | X |  |  | X |  |
| #3 | X | X | X | X | X | X |
| #4 | X |  |  |  |  |  |
| #5 | X |  |  | X |  |  |
| #6 | X | X | X |  |  |  |
| #7 | X |  |  |  |  |  |
| #8 |  | X |  |  |  |  |
| #9 |  | X |  |  |  |  |
| #10 |  | X |  |  |  |  |
| #11 |  | X |  |  |  |  |
| #12 |  | X |  |  |  |  |
| #13 |  |  | X |  |  |  |
| #14 |  |  | X |  |  |  |
| #15 |  |  | X |  |  | X |
| #16 |  |  | X |  |  |  |
| #17 |  |  | X |  |  |  |
| #18 |  |  |  | X |  |  |
| #19 | X | X |  |  | X |  |
| #20 |  |  |  |  |  | X |
| #X | X |  |  |  |  |  |

* * * Next Change (All new text)* * * *

## 6.X	Solution #X: SF distribution in the general architecture

### 6.X.0	High-level solution Principles

Sensing logical functions are distribued to either newly introduced NFs or existing 3GPP NFs as below:

- Sensing Service Management Function (SSMF): Sensing Entity Configuration, Sensing Entity Discovery and Selection, Sensing result generation, data delivery for sensing result exposure and storage

- Data Service Management Function (DSMF): Service Authurization and Exposure, Data Collection and Result Storage control

- NF for data collection/storage: Data Collection, Result Storage

### 6.X.1	Description

This solution is for Key Issue#1 on SF distribution in the general architecture.

Figure 6.X.1-1 Logical functions to support sensing service

As shown in Figure 6.X.1-1, after receiving a sensing service request, the network authorizes the request. Upon successful authorization, the network selects sensing entities, which perform sensing of target objects or environments in the designated sensing service area. These entities conduct sensing measurements within the target area, and the network collects and processes the measurement data. Based on this data, the network generates sensing results and delivers them to sensing service consumers. A complete sensing service implementation includes the following key logical functions:

- Sensing Entity Configuration: configure parameters and policies to sensing entities (e.g., transmitters/receivers).

- Sensing Entity Discovery and Selection: identifies and selects sensing transmitters and sensing receivers.

- Sensing result generation: generate sensing result based on the collected sensing data

- Data Collection: collect sensing data to generate sensing results, which may require triggering sensing transmitters/receivers for measurements.

- Service Authurization and Exposure: Provides sensing results to consumers based on their service requests if successfully authurised.

- Result Storage: Stores the sensing results for future use on the service request of the same requirements.

These logical functions can be distribued to either newly introduced NFs or existing 3GPP NFs.

Figure 6.X.1-2 Logical functions to support sensing service

Figure 6.X.1-2 shows the overall sensing service architecture, where 2 new NFs are introduced, and their functionalities are defined in Table 6.X.1-1:

- Sensing Service Management Function (SSMF): Sensing Entity Configuration, Sensing Entity Discovery and Selection, Sensing result generation, data delivery for sensing result exposure and storage

- Data Service Management Function (DSMF): Service Authurization and Exposure, Data Collection and Result Storage control

Table 6.X.1-1 Distribution of Sensing Function in 3GPP entities

|  | Sensing Entity Configuration | Sensing Entity Discovery and Selection | Sensing result generation | Data Collection | Service Authurization and Exposure | Result Storage |
| --- | --- | --- | --- | --- | --- | --- |
| DSMF |  |  |  | X (CP) | X (CP) | X (CP) |
| NF for data collection/storage (e.g. AIML data collection entity, O&M data collection entity, or others) |  |  |  | X |  | X |
| SSMF | X | X | X | X (UP/Data connection) | X (UP/Data connection) | X (UP/Data connection) |

Table 6.X.1-1 shows how the Sensing Functions are distributed on to the 3GPP entities.

### 6.X.2	Procedures

None.

### 6.X.3	Impacts on services, entities and interfaces

As specified in 6.X.1.

* * * End of Change * * * *