SA WG2 Meeting #170	S2-2506288

25 – 29 Aug, 2025, Goteborg , SE

Source:	Verizon

Title:	[WT#1.2, NF stateless] Improving the resiliency of an NF

Document for:	Discussion

Agenda Item:	20.6.1.2

Work Item / Release:	FS_6G_ARC/Rel-20

Abstract of the contribution: To improve resiliency of an NF proposes a WI/KI to look at how to make an NF more “stateless”.

# 1. Justifications

Explain the technical relevance of the proposed Work Task (WT) or Key Issue (KI) in the context of SA2’s scope (i.e., system architecture).

Architectural Motivation: The 6G SID as documented in SP-250806 has the following text in the objective
The study should consider at least the following aspects: cloud native, sustainability and energy efficiency, robustness and resiliency, etc

This architectural opportunity addresses the issue of resiliency of an NF and the impact of failure of that NF on a network should it fail.

Gap Analysis: There is an expectation from users that the cellular network is “always available”, there are many instances when there has been network outages and users have even called emergency services to report a loss of connectivity.  Cellular connectivity is a “must have” by society.  When an NF fails, depending on which NF it is it can affect a significant number of users e.g. 3GPP TS 23.502 table  5.2.2.2.2-1 contains information that needs to be communicated, some of this is static in nature.  Improving the availability of static information can greatly improve the resiliency of the NF.  NFSets is specified in 3GPP TS 23.501 sub-clause 4.2.5 and 3GPP TS 23.501 sub-clause 6.2.12 defines UDSF but as the specification says “Storage and retrieval of information as unstructured data by any NF” meaning there is no specified data format, it is left to implementation.  An MNO cannot mix and match UDSF from different vendors, nor share USDF across different NFs.

Technical Impact: How would this WT/KI influence:

- Overall system architecture (e.g., reference points, functional models)? This could create an updated API (See 3GPP TS 23.502 sub-clause 5.2.14) between an NF and an entity where that NFs quasi state information is stored.

- Interaction between network functions/services?  New interaction between NF and state repository

- Interworking or migration from 5G to 6G? – N/A

# 2. WT Scope

3GPP NFs are composed of multiple microservices, some of these microservices have stateless information, while others manage databases of quasi stateful information.  Study should:

- Assess and identify criteria for NF to be stateless/Quasi Stateful

- Investigate if stateless/Quasi Stateful NF data can be externalized into a shared repository

- Define an architecture and security aspects to allow the stateless/Quasi stateful NF to access the shared repository

# 3. Potential KI(s) Scope

3GPP NFs are composed of multiple microservices, some of these microservices have stateless information, while others manage databases of quasi stateful information.  Study should:

- Assess and identify criteria for NF to be stateless/Quasi Stateful

- Investigate if stateless/Quasi Stateful NF data can be externalized into a shared repository

- Define an architecture and security aspects to allow the stateless/Quasi stateful NF to access the shared repository