3GPP SA WG2#170	S2-2506290

Goteborg, Sweden, 25-29 August, 2025	(revison of S2-2504672)

Source:	OPPO

Title:	New solution to KI#3 on Sensing Entity discovery and selection

Document for:	Approval

Agenda Item: 	20.2.1

Work Item / Release:	FS_Sensing_ARC/Rel-20

Abstract of the contribution: This paper proposes a new solution to KI#3 on Sensing Entity discovery and selection for the FS_Sensing_ARC TR 23.700-14.

1	Discussion

This paper proposes for a Sensing Entity discovery and selection procedure for area as sensing target use scenarios addressing KI#3.

Based on the requirements from TS 22.137, one of the most import scenario for the use cases is that the sensing target is a geographical area (i.e. target area). This solution resolves KI#3 for Sensing Entity discovery and selection according to target area.

This solution has the following assumptions:

- Sensing Entities register their sensing capabilities to AMF.

2)  Sensing Function registers to NRF with supported serving area.

3)  AMF provides service operation for Sensing Function to retrieve available Sensing Entities registered at the AMF within the correlated area.

4)  Sensing Function select the Sensing Entities from the available Sensing Entities within the target area based on the service requirements and Sensing Entities’ capability.

5)  Sensing Entity can be NG-RAN.

Based on the assumptions above, the following procedures are provided.

2	Proposal

It is proposed to include the following changes in TR 23.700-14 V0.2.0.

* * * * Start of Changes (All text new) * * * *

6.X	Solution #X: Sensing Entity discovery and selection procedure for area as sensing target scenarios

6.X.1	Description

This solution resolves KI#3 for Sensing Entity discovery and selection according to target area.

This solution has the following assumptions:

1)	Sensing Entities register their sensing capabilities to AMF.

2)	Sensing Function registers to NRF with supported serving area.

3)	AMF provides service operation for Sensing Function to retrieve available Sensing Entities registered at the AMF within the correlated area.

4)	Sensing Function select the Sensing Entities from the available Sensing Entities within the target area based on the service requirements and Sensing Entities’ capability.

5)	Sensing Entity can be NG-RAN.

6.X.2	Procedures

Figure 6.x.2-1: Procedure for Sensing Entity discovery and selection according to target area

- Sensing Entity registered sensing capability (e.g. supported sensing mode, sensing QoS) to AMF via NGAP setup procedures.

- AF sends sensing service request indicating a geographical target area and related sensing requirements (e.g. sensing mode, sensing QoS) to NEF.

2.	NEF queries NRF to select Sensing Function(s) for the sensing service request according to target area and Sensing Function’s serving area.

3.	NEF sends sensing service request to the selected Sensing Function(s) with target area and sensing requirements.

4.	Sensing Function queries NRF to select AMF(s) for the sensing service request according to the target area. Sensing Function generates correlated Area information for the selected AMF(s) according to the AMF’s serving area and target area.

5.	Sensing Function sends Sensing Entity Retrieval request to AMF to retrieve the Sensing Entities. The request includes the correlated Area and sensing requirements.

6.	AMF check the registered Sensing Entities’ contexts to find Sensing Entities that can fulfil the sensing requirements in the correlated Area.

7.	AMF respond to Sensing Function with the Sensing Entity list with location information and the sensing capability.

8.	Sensing Function selects Sensing Entities for the target area based on the Sensing Entity information received in step 7

9.	Sensing Function and selected Sensing Entities perform Sensing measurement and sensing result calculation.

10.	Sensing Function responds to NEF about the sensing result.

11.	NEF responds to AF about the sensing result.

6.X.3	Impacts on services, entities and interfaces

The solution has impacts on the following entities:

AMF:

-	Support to provide service operation for Sensing Function to retrieve available Sensing Entities registered at the AMF within the correlated area.

Sensing Entity:

-	Support to register the sensing capability to AMF.

Sensing Function:

-	Support to select the Sensing Entities from the available Sensing Entities within the target area based on the service requirements and Sensing Entities’ capability.

-	Support to register to NRF with supported serving area.

NRF:

- Support to store the Sensing Function’s serving area for NEF’s query.

NEF:

- Support to select Sensing Function based on target area and Sensing Function’s serving area.

* * * * End of Changes * * * *