3GPP TSG-SA2 Meeting #170	S2-2506313

# Goteborg, Sweden, 25 – 29 August 2025

Source: 	Transsion Holdings

Title: 	KI#1&3&4&5: NewSol: Sensing procedure for the sensing service by request with sensing area info

Document for: 	Approval

Agenda Item: 	20.2.1

Work Item / Release:	FS_Sensing_ARC / Rel-20

Abstract of the contribution: This paper proposes a new solution for KI#1&KI#3&KI#4&KI#5 in TR 23.700-14.

.

1	Discussion

This paper proposes a new solution for KI#1&KI#3&KI#4&KI#5 in TR 23.700-14. The solution focuses on enhancement to 5G system to provide sensing service by request with the expected sensing area info.

2. Proposal

It is proposed to add the following contents to TR 23.700-14.

Start of Change

Solutions

6.0	Mapping of Solutions to Key Issues

Table 6.0-1: Mapping of Solutions to Key Issues

| Solutions | Key Issues |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | #1 | #2 | #3 | #4 | #5 | #6 |
| #1 | x |  | x | x | x |  |

Next Change (all new text)

6.X	Solution #X: Sensing procedure for the sensing service request by request with sensing area info

6.X.1	High level principles

High-level principles of this solution are as below:

a)	AMF is enhanced to support sensing service and register to NRF with extended NF profile as well.

b)	Sensing Consumer requests sensing results to the core network by request with the sensing area info (i.e. longitude, latitude, and altitude and shape).

c)	GMSENC is introduced as the gateway for AMF discovery via NRF query.

d)	AMF decides the sensing entiti(es) (i.e. gNB and/or UE) to perform sensing activiti(es).

e)	SENF is introduced for providing commands to AMF and for sensing data processing.

It addresses the KI#1, KI#3, KI#4, and KI#5.

6.X.1	Description

This solution is for Key Issue#1&KI#3&KI#4&KI#5 for sensing procedure for the sensing service by request with sensing area info.

Figure 6.X.1-1 illustrates Non-roaming reference architecture for Sensing Services in SBI representation. New NFs, i.e. GMSENC and SENF, are introduced as new network functions to 5G enhanced SBI architecture for providing sensing services.

Figure 6.X.1-1: Non-roaming reference architecture for Sensing Services in SBI representation

NOTE 1: GMSENC, i.e. gateway mobile sensing center, is introduced as sensing gateway for sensing service.

NOTE 2: SENF, i.e. sensing function, is introduced as sensing function.

The existing NF Service Registration procedure in 23.502 (Figure 4.17.1-1) shown below is reused for AMF to register in NRF with enhancement of NF profile to include more information such as whether sensing service is supported by the AMF, the position and area info of sensing service provided by the AMF (if sensing service is supported).

The Sensing Consumer requests sensing service by request with sensing area info to the core network via the new introduced network function GMSENC. GMSENC finds the AMF via query NRF to perform sensing activities. The SENF is another newly introduced network function to command AMF to perform sensing activities and to further process sensing data to derive sensing results. Either control plane or user plane connection could be used between the gNB/UE and the core network for sensing data transmission.

6.X.2	Procedures

6.x.2.1 Sensing procedure for the sensing service by request with sensing area info

Figure 6.X.2-1 illustrates the general network sensing for the sensing consumer to the PLMN for the sensing service for non-roaming scenario service. It is assumed that sensing consumer requests sensing results to the core network by request with sensing area info (i.e. longitude, latitude, and altitude and shape). And it is assumed that the sensing consumer is authorized to use the sensing service and no privacy verification is required.

Figure 6.X.2-1: Sensing procedure for the sensing service by request with sensing area info

1.	The sensing consumer (e.g. external sensing services client, AF) sends a request to the GMSENC for the expected target sensing area. The request may include the required QoS, UE unaware indication and target sensing area info. If the Sensing Consumer is external network function, then the sensing consumer initiates the sensing service via NEF towards GMSENC.

2.	The GMSENC invokes a Namf_Discovery Request service operation towards the NRF with the expected target sensing area.

3.	The NRF returns the network addresses of the AMF which could perform sensing activities in Namf_Discovery Response.

4.	The GMSENC invokes the Namf_Sensing_ProvideInfo service operation towards the AMF to request the sensing service on the expected target sensing area. The service operation includes the sensing area info, client type and may include the required QoS, UE unaware indication.

5.	If the UE is selected as sensing entity but is in CM IDLE state and UE unaware indication is not included, the AMF may initiate a network triggered Sensing Service Request procedure.

However, the AMF may decide to skip steps 5 - 9 in step 5 based on the UE unaware indication and the UE state or the NG-RAN rejects the Network Sensing message with appropriate rejection cause (e.g. UE cannot be paged).

6.	The AMF selects an SENF based on the available information or based on AMF local configuration or NRF query. The SENF selection may take the 5G-AN currently serving the UE into account.

7.	The AMF invokes the Nsenf_Sening_Determine Request service operation towards the SENF to request the sensing operation in the target sensing area. The service operation may include the sensing cell identity, the client type, the required QoS, supported sensing area, etc.

8.	The SENF performs one or more of the sensing procedures. During this step the SENF may use the Namf_Communication_N1N2MessageTransfer service operation to request the transfer of a sensing related N1 message to the UE or the transfer of a Network Sensing message to the sensing NG-RAN node (e.g. gNB). The SENF shall determine a geographical sensing and optionally a sensing in local coordinates. The sensing data could be transferred via Control Plane (8.3a) or User Plane (8.3b).

9.	The SENF returns the Nsenf_Sensing_Determine Response towards the AMF to return the sensing results. The sensing service operation includes the sensing area, its age and accuracy and may include information about the sensing method and the timestamp of the sensing estimate.

10.	The AMF returns the Namf_Sensing_ProvideInfo Response towards the GMSENC to return the sensing results. The service operation may include the sensing area, its age and accuracy and may include information about the sensing method and the timestamp of the sensing estimate, etc.

11.	The GMSENC sends the sensing service response to the Sensing Consumer.

6.X.3	Impacts on services, entities and interfaces

GMSENF:

-	Receives sensing service request from sensing service consumer such as AF, external client and sends the request to AMF;

-	Discoveries the AMF via NRF with expected target sensing area info;

- -  Receives response on sensing service request from AMF and send the response to service consumer.

.

SENF:

-	Sends commands to AMF to perform sensing operation;

-	Receives sensing data collected by the gNB and UEs via CP or UP connection;

- -  Derives sensing results by processing sensing data.

.

AMF:

- -  Receives commands from SENF and sends the commands to gNB and/or UE to perform sensing operation;

- -  Collects sensing data if CP connection is applied;

- -  Receives sensing results from SENF and expose them to sensing consumer via GMSENF.

- .

NRF:

- -  Discoveries the AMF to be responsible for sensing activities with sensing area info.

NG-RAN:

- -  Receive sensing request(s) from AMF and send the requests to one UE or a group of UEs or all UEs in the cell if needed;

- -  Performs sensing activities according to commands received from AMF;

- -  Sends the sensing data to SENF via AMF (via CP connection) or via UPF (via UP connection).

- .

UE:

-	Receives sensing request(s) from AMF via NG-RAN;

-	Performs sensing activities according to commands received from AMF;

- 	Sends the sensing data to SENF via AMF (via CP connection) or via UPF (via UP connection).

End of Changes