3GPP TSG-SA WG2#170	S2-2506318

Gothenburg, Sweden, 25-29 August 2025	(was S2-250xxxx)

Source:	Qualcomm Incorporated

Title:	TR 23.700-04: Agreement for KI#1

Document for:	Discussion/Approval

Agenda Item:	20.3.1

Work Item / Release:	FS_AIML_CN_Ph2

Abstract of the contribution: This contribution proposed interim conclusion for KI#1 Transfer of data over UP for UE data.

# 1.	Discussion

There are 21 solutions were captured in TR 23.700-04 for KI#1, this contribution proposed the agreed principles for KI#1 Transfer of data over UP for UE data.

# 2.	Text proposal

It is proposed to agree the following changes vs. TR 23.700-04:

# >>>>BEGINNING OF CHANGES<<<<

## 7.1	Agreed Principles

### 7.1.1	Agreed Principles for KI#1

Architecture enhancement:

- define a new 5GC entity as the UP path termination entity to support the transfer of data over UP for UE data, called DCF (Data Collection Function).

- the DCF receives the UE data via UP path and exposes the received data to an AF that acts as UE server. The AF is either within the MNO or out of MNO network.

- if the AF is within the MNO network, the new 5G NF exposes the UE data to the AF directly

- if the AF is outside of the MNO network, the new 5G NF exposes the UE data to the AF via NEF.

- multiple DCFs may be deployed in the network, AF / NEF discovers the DCF for UE data transfer based on location information and operator’s configuration.

- if the AF is within the MNO network, the AF discovers the DCF for UE data transfer.

- if the AF is outide of the MNO network, the NEF discovers the DCF for UE data transfer

-	the DCF determines the UE list for UE data transfer based on location information

-	the DCF performs user consent before the data collection from UE

Traffic differentiation：

-	dedicated PDU session parameters (e.g., DNN and S-NSSAI) are set in the RSD for the traffic descriptor in the URSP rule sent to UE, or

-	the packet filter set (e.g., IP address / IPv6 prefix or port number of the new 5GC entity) is used in the QoS rule and the PDR to identify the packet flow used for UE data transfer.

UE path establishment to DCF：

-	the DCF provides the UP path information to UE via NAS message.

- the UE path information may include the destination IP address or FQDN

Data reporting：

-	The UE data transfer request is sent from DCF to UE via UP signalling.

-	the event ID is defined for each of the use cases which will be defined by RAN WGs.

-	the transferred parameters for each event ID are decided by RAN WGs

-	UE may negotiate with the DCF about how to report the UE data.

# >>>>NEXT CHANGE<<<<

# >>>>NEXT CHANGE<<<<

# >>>>END OF CHANGES<<<<