3GPP TSG-SA WG2#170	S2-2506319

Gothenburg, Sweden, 25-29 August 2025	(was S2-250xxxx)

Source:	Qualcomm Incorporated

Title:	TR 23.700-67: Agreement for KI#2

Document for:	Discussion/Approval

Agenda Item:	20.4.1

Work Item / Release:	FS_EnergySys_Ph2

Abstract of the contribution: This contribution proposes the agreement for KI#2 in TR 23.700-67.

# 1.	Discussion

This contribution proposes the agreement for KI#2 in TR 23.700-67.

# 2.	Text proposal

It is proposed to agree the following changes vs. TS 23.700-67:

# >>>>BEGINNING OF CHANGES<<<<

# 7	Interim agreements

## 7.1	Agreed Principles

### 7.1.2	Agreed Principles for KI#2

The following principles are agreed for KI#2:

- 	PCF generates the Energy Saving QoS profile and the priority list for the QoS flow based on AF’s request and operator policy. PCF sends the Energy Saving QoS profile(s) and the priority list to SMF and then to NG-RAN along with the legacy QoS profile. NG-RAN selects the Energy Saving QoS profile only if NG-RAN decides to downgrade the QoS parameters due to energy saving. NG-RAN notifies the selected Energy Saving QoS profile to SMF, SMF will further inform the selected Energy Saving QoS profile to PCF and to UPF for charging purpose.

## 7.2	Topics for further consideration

### 7.2.2	Topics for further consideration for KI#2

Editor's note:	This clause will include the topics for further consideration as work progresses for the specific KI#Z. Eventually this clause should only contain topics for further consideration that did not result in agreements (i.e. in agreed principle(s) in a clause 7.1.Z) and can either be then marked as not pursued or postponed to a future release.

-	Whether and what information exposed from EIF to PCF for policy consideration is related to the discussion in KI#1.

# >>>>NEXT CHANGE<<<<

# >>>>NEXT CHANGE<<<<

# >>>>END OF CHANGES<<<<