3GPP TSG-SA WG2#170	S2-2506322

Gothenburg, Sweden, 25-29 August 2025	(was S2-250xxxx)

Source:	Qualcomm Incorporated, Ericsson, Nokia, MediaTek, Verizon, Orange

Title:	Identifying 5G features to be supported by the 6G system

Document for:	Discussion/Approval

Agenda Item:	20.6.0

Work Item / Release:	FS_6G_ARC / Rel-20

Abstract of the contribution: This paper proposes an approach how to identify existing 5G features to be supported by the 6G system at Day 1 of 6G:

# 1.	Discussion

SA2’s study item for 6G [X] contains the following statement:

In addition to the work tasks above, this study will identify other 5G features that will be supported and/or adapted in 6G.

This paper proposes the following approach to identify existing 5G features that will be supported at Day 1 of 6G:

-	Document the list of 5G features that have been specified from Rel-15 through Rel-19 in a new Annex of TR 23.801-01.

-	Include an architectural requirement in TR 23.801-01 that the 6G system architecture shall also support the 5G features concluded to be supported at 6G Day 1 according to the new annex.

-	Collect company input on which features are expected to be supported at Day 1 of 6G.

-	Document the conclusion regarding the features to be supported at Day 1 of 6G in the new Annex.

-	Coordinate with other WGs for features that depend on those WGs.

# 2.	Text proposal

It is proposed to agree the following changes for TR 23.801-01.

# >>>> BEGINNING OF CHANGES <<<<

## 4.2	Architectural Requirements

Editor's Note: This clause defines the architectural requirements that serve as the foundation for the study.

The 6G system shall also support the existing 5G features according to Annex X.

NOTE 1:	Solutions for the 6G architecture are expected to illustrate how these existing features can still be supported.

NOTE 2:	SA2 will coordinate with other working groups for features that are dependent on other WGs.

# >>>> NEXT CHANGE (all new text) <<<<

# Annex X: Existing 5G features to be supported at Day 1 of 6G

This annex lists the features defined for 5G across the different releases and will identify, based on company views, which of those features need to be supported at Day 1 of 6G. The list does not include basic system features such as network access control, registration and connection management, UE reachability, basic session management (incl. IP address assignment), AM/SM policies, user-plane management procedures and charging as those are assumed to be defined as part of the 6G study.

It is expected that existing features may need updates to suit the 6G system needs, such aspects are handled within related key issues.

Some of the features are described/identified on the high level, further granularity of which of the functions within certain feature is required from Day 1, should be provided as input to the discussion.

|  | Feature name for 5GS | Reference to existing Feature description | Release |  |  |  |  | Impacts |  |  |  | Day 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 15 | 16 | 17 | 18 | 19 | UE (AS) | UE (NAS) | (R)AN | CN |  |
| 1 | Architecture model and concepts |  |  |  |  |  |  |  |  |  |  |  |
| 1.1 | Data Storage architectures | TS 23.501 clause 4.2.5 | X |  |  |  |  |  |  |  | X |  |
| 2 | Specific services |  |  |  |  |  |  |  |  |  |  |  |
| 2.1 | Public Warning System | TS 23.501 clause 4.4.1, 5.16.1 | X |  |  |  |  | X |  | X | X |  |
| 2.2 | SMS over NAS | TS 23.501 clause 4.4.2, 5.16.2 | X |  |  |  |  |  | X |  | X |  |
| 2.3 | IMS support | TS 23.501 clause 4.4.3, 5.16.3 | X |  |  |  |  |  | X |  | X |  |
| 2.4 | Application Triggering Services | TS 23.501 clause 4.4.5 | X |  |  |  |  |  | X |  | X |  |
| 2.5 | Emergency services | TS 23.501 clause 5.16.4 | X |  |  |  |  |  | X | X | X |  |
| 2.6 | Mission Critical Services | TS 23.501 clause 5.16.6 | X |  |  |  |  |  | X |  | X |  |
| 2.7 | 5G LAN-type Services | TS 23.501 clause 4.4.6, 5.29 |  | X |  |  |  |  |  |  | X |  |
| 2.8 | MSISDN-less MO SMS Service | TS 23.501 clause 4.4.7 |  | X |  |  |  |  | X |  | X |  |
| 3 | Multimedia Priority Services |  |  |  |  |  |  |  |  |  |  |  |
| 3.1 | Multimedia Priority Services | TS 23.501 clause 5.16.5, 5.22 | X |  |  |  |  |  | X |  | X |  |
| 3.2 | Priority Data Transport Service, Invocation of MPS priority | TS 23.501 clause 5.16.5 |  |  | X |  |  |  | X |  | X |  |
| 3.3 | MPS over WLAN | TS 23.501 clause 5.16.5 |  |  |  | X |  |  | X |  | X |  |
| 3.4 | MPS for IMS Messaging and SMS services | TS 23.501 clause 5.16.5 |  |  |  |  | X |  |  |  | X |  |
| 4 | Location services |  |  |  |  |  |  |  |  |  |  |  |
| 4.1 | Location services (regulatory service only) | TS 23.501 clause 4.4.4; TS 23.273 | X |  |  |  |  | X | X |  | X |  |
| 4.2 | Location service For all services, MT-LR, MO-LR, and NI-LR procedures, privacy support, broadcast of location assistance data, etc. | TS 23.273 clause 6 |  | X |  |  |  | X | X | X | X |  |
|  | (no normative work) |  |  |  |  |  |  |  |  |  |  |  |
| 4.4 | LCS Phase 3 (e.g., user-plane positioning, support of PRU, support of LPHAP (low power and high accuracy positioning), NWDAF interactions, LCS continuity between EPS and 5GS, etc.) | TS 23.273 |  |  |  | X |  | X | X | X | X |  |
| 4.5 | Multiple location procedure for the regulatory location service | TS 23.273 clause 6.1 |  |  |  | X |  |  |  |  | X |  |
| 4.6 | Periodic Location Events based NRPPa Periodic Measurement Reports | TS 23.273 clause 6.3.5 |  |  |  |  | X |  |  |  | X |  |
| 4.7 | On-demand broadcast of GNSS assistance data | TS 23.273 clause 6.14.1 |  |  |  |  | X |  |  |  | X |  |
| 4.8 | Multiple location procedure for multiple emergency LCS clients | TS 23.273 clause 6.10.5 |  |  |  |  | X |  |  |  | X |  |
| 4.9 | Support of PRU Usage Extension | TS 23.273 clause 4.3.8 |  |  |  |  | X |  | X |  | X |  |
| 5 | 3GPP access specific aspects |  |  |  |  |  |  |  |  |  |  |  |
| 5.1 | Paging strategy handling | TS 23.501 clause 5.4.3 | X |  |  |  |  |  |  |  | X |  |
| 5.2 | UE radio capability information storage in the AMF | TS 23.501 clause 5.4.4.1 | X |  |  |  |  |  |  | X | X |  |
| 5.3 | UE radio capability signalling optimisation (RACS) | TS 23.501 clause 5.4.4.1a |  | X |  |  |  | X | X | X | X |  |
| 5.4 | UE Radio Capability Match Request | TS 23.501 clause 5.4.4.2a | X |  |  |  |  |  |  | X | X |  |
| 5.5 | Paging assistance information | TS 23.501 clause 5.4.4.3 | X |  |  |  |  |  |  | X | X |  |
| 5.6 | Core Network assistance information for RAN optimization | TS 23.501 clause 5.4.6 | X |  |  |  |  |  |  | X | X |  |
| 5.7 | NG-RAN location reporting | TS 23.501 clause 5.4.7 | X |  |  |  |  |  |  | X | X |  |
| 5.8 | Support for identification and restriction of using unlicensed spectrum | TS 23.501 clause 5.4.8 |  | X |  |  |  | X |  | X | X |  |
| 6 | Non-3GPP access support |  |  |  |  |  |  |  |  |  |  |  |
| 6.1 | Untrusted non-3GPP access to 5GS based on N3IWF | TS 23.501 clause 4.2.8.1 | X |  |  |  |  |  | X |  | X |  |
| 6.2 | Interworking between ePDG connected to EPC and 5GS | TS 23.501 clause 4.3.4 | X |  |  |  |  |  | X |  | X |  |
| 6.3 | Trusted non-3GPP access to 5GS based on TNGF | TS 23.501 clause 4.2.8.1 |  | X |  |  |  |  | X |  |  |  |
| 6.4 | Support of Wireline Access | TS 23.501 clause 4.2.8.1A, TS 23.316 |  | X |  |  |  | X | X | X | X |  |
| 6.5 | Access to 5GC from devices that do not support 5GC NAS over WLAN access | TS 23.501 clause 4.2.8.5 |  | X |  |  |  |  |  | X | X |  |
| 6.6 | Support of Non-seamless WLAN offload | TS 23.501 clause 5.41 |  |  | X |  |  |  | X |  | X |  |
| 6.7 | Differentiated service for UE and Non-3GPP devices connected behind a 5G RG | TS 23.316 clauses 4.5.3, 4.10-4.10d |  |  |  | X |  |  |  |  |  |  |
| 6.8 | N3IWF selection based on slices requested by the UE | TS 23.501 clause 6.3.6 |  |  |  | X |  |  | X |  | X |  |
| 6.9 | TNGF selection based on slices requested by the UE | TS 23.501 clause 6.3.12 |  |  |  | X |  |  | X |  | X |  |
| 7 | ATSSS |  |  |  |  |  |  |  |  |  |  |  |
| 7.1 | Initial ATSSS support | TS 23.501 clause 4.2.10, 5.32 |  | X |  |  |  |  | X |  | X |  |
| 7.2 | ATSSS Phase 2 (e.g. new steering modes) | TS 23.501 clause 5.32.6, 5.32.7 |  |  | X |  |  |  | X |  | X |  |
| 7.3 | ATSSS Phase 3 (e.g., redundant steering mode, MP-QUIC support, Multi-Access PDU Sessions using one leg associated with PDN Connection in EPC and one leg associated with PDU Session in 5GC) | TS 23.501 clause 5.32 |  |  |  | X |  |  | X |  | X |  |
| 7.4 | Support of MPQUIC-IP and MPQUIC-E | TS 23.501 clause 5.32 |  |  |  |  | X |  | X |  | X |  |
| 8 | Session management related features |  |  |  |  |  |  |  |  |  |  |  |
| 8.1 | Usage of an UL Classifier for a PDU Session | TS 23.501 clause 5.6.4.2 | X |  |  |  |  |  |  |  | X |  |
| 8.2 | Usage of IPv6 multi-homing for a PDU Session | TS 23.501 clause 5.6.4.3 |  |  |  |  |  |  | X |  | X |  |
| 8.3 | Support for Local Area Data Network | TS 23.501 clause 5.6.5 | X |  |  |  |  |  | X |  | X |  |
| 8.4 | LADN per DNN/S-NSSAI | TS 23.501 clause 5.6.5a |  |  |  | X |  |  | X |  | X |  |
| 8.5 | Secondary authentication/authorization by a DN-AAA server during the establishment of a PDU Session | TS 23.501 clause 5.6.6 | X |  |  |  |  |  | X |  | X |  |
| 8.6 | Application Function influence on traffic routing | TS 23.501 clause 5.6.7 | X |  |  |  |  |  |  |  | X |  |
| 8.7 | Selective activation and deactivation of UP connection of existing PDU Session | TS 23.501 clause 5.6.8 | X |  |  |  |  | X | X | X | X |  |
| 8.8 | Session and Service Continuity (SSC) modes | TS 23.501 clause 5.6.9 | X |  |  |  |  |  | X |  | X |  |
| 8.9 | Support of Ethernet PDU Session type | TS 23.501 clause 5.6.10.2 | X |  |  |  |  |  | X |  | X |  |
| 8.10 | Support of Unstructured PDU Session type | TS 23.501 clause 5.6.10.3 | X |  |  |  |  |  | X |  | X |  |
| 8.11 | UE presence in Area of Interest reporting usage by SMF | TS 23.501 clause 5.6.11 | X |  |  |  |  |  |  |  |  |  |
| 8.12 | Use of Network Instance | TS 23.501 clause 5.6.12 | X |  |  |  |  |  |  |  | X |  |
| 8.13 | Always-on PDU session | TS 23.501 clause 5.6.13 | X |  |  |  |  |  | X |  | X |  |
| 8.14 | Support of Framed Routing | TS 23.501 clause 5.6.14 | X |  |  |  |  |  |  |  | X |  |
| 8.15 | PDU Session User Plane Security | TS 23.501 clause 5.10.3 |  | X |  |  |  | X | X | X | X |  |
| 9 | QoS model |  |  |  |  |  |  |  |  |  |  |  |
| 9.1 | General model, parameters, characteristics and mapping | TS 23.501 clause 5.7.1-5.7.4 | X |  |  |  |  | X | X | X | X |  |
| 9.2 | Reflective QoS | TS 23.501 clause 5.7.5 | X |  |  |  |  | X | X | X | X |  |
| 9.3 | Notification Control without Alternative QoS profiles | TS 23.501 clause 5.7.2.4.1a |  | X |  |  |  |  |  | X | X |  |
| 9.3 | Notification Control with Alternative QoS profiles | TS 23.501 clause 5.7.2.4.1b |  | X |  |  |  |  |  | X | X |  |
| 9.4 | PDU Set QoS Parameters | TS 23.501 clause 5.7.7 |  |  |  | X |  |  |  | X | X |  |
| 10 | Network slicing |  |  |  |  |  |  |  |  |  |  |  |
| 10.1 | Basic network slicing support | TS 23.501 clause 5.15 | X |  |  |  |  | X | X | X | X |  |
| 10.2 | Network Slice-Specific Authentication and Authorization | TS 23.501 clause 5.15.10 |  | X |  |  |  |  | X |  | X |  |
| 10.3 | Network Slice Simultaneous Registration Group (NSSRG) | TS 23.501 clause 5.15.12 |  |  | X |  |  |  | X | X | X |  |
| 10.4 | Data rate limitation per Network Slice for a UE | TS 23.501 clause 5.15.13 |  |  | X |  |  | X | X | X | X |  |
| 10.5 | Network Slice AS Groups support | TS 23.501 clause 5.15.14 |  |  | X |  |  | X | X | X | X |  |
| 10.6 | Support of Network Slice usage control | TS 23.501 clause 5.15.15 |  |  |  | X |  |  | X |  | X |  |
| 10.7 | Optimized handling of temporarily available network slices | TS 23.501 clause 5.15.16 |  |  |  | X |  |  | X |  | X |  |
| 10.8 | Partial Network Slice support in a Registration Area | TS 23.501 clause 5.15.17 |  |  |  | X |  |  | X |  | X |  |
| 10.9 | Support for Network Slices with Network Slice Area of Service not matching deployed Tracking Areas | TS 23.501 clause 5.15.18 |  |  |  | X |  |  | X |  | X |  |
| 10.10 | Support of Network Slice Replacement | TS 23.501 clause 5.15.19 |  |  |  | X |  |  | X |  | X |  |
| 10.11 | Support of Network Slice Instance Replacement | TS 23.501 clause 5.15.20 |  |  |  | X |  |  |  |  | X |  |
| 10.12 | Support of Network Slice Admission Control in 5GS for maximum number of UEs with at least one PDU Session/PDN Connection | TS 23.501 clause 5.15.15.5a |  |  |  | X |  |  |  |  | X |  |
| 10.13 | S-NSSAI selection while in EPS | TS 23.501 clause 5.15.7 |  |  |  |  | X |  | X |  | X |  |
| 10.14 | Network slice replacement based on AF request | TS 23.503 clause 6.1.2.1.1 |  |  |  |  | X |  |  |  | X |  |
| 11 | Control Plane Load Control, Congestion and Overload Control |  |  |  |  |  |  |  |  |  |  |  |
| 11.1 | TNLA Load Balancing and TNLA Load Re-Balancing | TS 23.501 clause 5.19.2 | X |  |  |  |  |  |  | X | X |  |
| 11.2 | AMF Load balancing and re-balancing | TS 23.501 clause 5.19.3, 5.19.4 | X |  |  |  |  |  |  | X | X |  |
| 11.3 | AMF control of overload | TS 23.501 clause 5.19.5 | X |  |  |  |  |  | X |  | X |  |
| 11.4 | SMF control of overload | TS 23.501 clause 5.19.6 | X |  |  |  |  |  | X |  | X |  |
| 11.5 | NAS level congestion control | TS 23.501 clause 5.19.7 | X |  |  |  |  |  | X |  |  |  |
| 12 | External Exposure of Network Capability (further granularity of what is required from Day 1 should be provided as input) | TS 23.501 clause 5.20 | X |  |  |  |  |  |  |  | X |  |
| 13 | Architectural support for virtualized deployments |  |  |  |  |  |  |  |  |  |  |  |
| 13.1 | TNL association addition/removal, selection, binding and redirection | TS 23.501 clause 5.21.1 | X |  |  |  |  |  |  | X | X |  |
| 13.2 | AMF Addition/Update | TS 23.501 clause 5.21.2.1 | X |  |  |  |  |  |  | X | X |  |
| 13.3 | AMF planned removal procedure | TS 23.501 clause 5.21.2.2 | X |  |  |  |  |  |  | X | X |  |
| 13.4 | Network Reliability support with Sets | TS 23.501 clause 5.21.3 |  | X |  |  |  |  |  |  |  |  |
| 14 | Other features |  |  |  |  |  |  |  |  |  |  |  |
| 14.1 | Supporting for Asynchronous Type Communication | TS 23.501 clause 5.23 | X |  |  |  |  |  |  |  | X |  |
| 14.2 | 3GPP PS Data Off | TS 23.501 clause 5.24 | X |  |  |  |  |  | X |  | X |  |
| 14.3 | Support of Tracing: Signalling Based Activation/Deactivation of Tracing | TS 23.501 clause 5.25 | X |  |  |  |  |  |  | X | X |  |
| 14.4 | Enhancement on policies for Background Data Transfer | TS 23.502 clause 4.15.6.8 |  | X |  |  |  |  | X |  | X |  |
| 14.5 | Support of Disaster Roaming with Minimization of Service Interruption | TS 23.501 clause 5.39 |  |  | X |  |  | X | X | X | X |  |
| 14.6 | NR RedCap UEs differentiation | TS 23.501 clause 5.41 |  |  | X |  |  | X |  | X | X |  |
| 14.7 | UE unreachability due to Unavailability Period | TS 23.501 clause 5.4.1.1 |  |  |  | X |  |  | X |  | X |  |
| 14.8 | Support for Service Function Chaining | TS 23.501 clause 5.6.16 |  |  |  | X |  |  |  |  | X |  |
| 14.9 | Architecture to support User Plane Information Exposure via a service-based interface | TS 23.501 clause 4.2.16 |  |  |  | X |  |  |  |  | X |  |
| 14.10 | Validity time for RFSP index | TS 23.501 clause 5.17.2.2.1 |  |  |  | X |  |  |  |  | X |  |
| 14.11 | Policy decisions based on spending limits | TS 23.503 clause 6.1.1.4 |  |  |  | X |  |  |  |  | X |  |
| 14.12 | IPv6 Prefix Delegation via DHCPv6 | TS 23.501 clause 5.8.2.2.4 |  |  |  | X |  |  |  |  | X |  |
| 14.13 | Enhancement of Application Detection Event Exposure | TS 23.503 clause 6.1.3.18 |  |  |  | X |  |  |  |  | X |  |
| 14.14 | Secondary DN authentication and authorization in EPS IWK case | TS 23.501 clause 5.17.2.5 |  |  |  | X |  |  | X |  | X |  |
| 14.15 | Indirect Network Sharing | TS 23.501 clause 5.18.1 |  |  |  |  | X |  |  |  | X |  |
| 14.16 | Support of roaming value-added services (welcome SMS, Subscription-based routing to a particular core network, etc.) | TS 23.501 clause |  |  |  |  | X |  |  |  | X |  |
| 14.17 | Minimization of the number of UE and AM Policy Associations | TS 23.501 clause 4.16.1.2 |  |  |  |  | X |  |  |  | X |  |
| 14.18 | Support of VLAN handling information in SM subscription data and DN-AAA | TS 23.501 clause 5.6.10.2 |  |  |  |  | X |  |  |  |  |  |
| 14.19 | Exposure enhancements for static UE IP address assignment and 5G VN group’s User Plane Security Policy | TS 23.502 clause 4.15.6.3i |  |  |  |  | X |  |  |  | X |  |
| 14.20 | Spending Limits for UE Policies in Roaming scenario | TS 23.501 clause 6.3.11 |  |  |  |  | X |  |  |  | X |  |
| 14.21 | Roaming traffic offloading via session breakout in HPLMN | TS 23.501 clause 5.8.2.2 |  |  |  |  | X |  |  |  | X |  |
| 14.22 | MSISDN verification operation support to Nnef_UEId Service | TS 23.501 clause 5.20 |  |  |  |  | X |  |  |  | X |  |
| 14.23 | Support of QoS monitoring capability configuration and transfer within 5GC | TS 23.501 clause 5.45 |  |  |  |  | X |  |  |  | X |  |
| 14.24 | NF discovery and selection by target PLMN | TS 23.501 clause 6.3.1 |  |  |  |  | X |  |  |  | X |  |
| 14.25 | Support for unavailability period not for satellite access (seamless UE recovery) | TS 23.501 clause 5.4.1.4 |  |  |  | X |  |  | X |  | X |  |
| 14.26 | 5G VN group communication | TS 23.501 clause 5.8.2.13 |  | X |  |  |  |  |  |  | X |  |
| 14.27 | Exposure of DNN and S-NSSAI specific Group Parameters | TS 23.501 clause 5.20b |  |  |  |  | X |  |  |  | X |  |
| 15 | Time Sensitive Communications and Time synchronization |  |  |  |  |  |  |  |  |  |  |  |
| 15.1 | AS-based time synchronization | TS 23.501 clause 5.27.1.2.1 |  | X |  |  |  | X |  | X |  |  |
| 15.2 | gPTP based downlink time synchronization | TS 23.501 clause 5.27.1.2.2 |  | X |  |  |  |  | X |  | X |  |
| 15.3 | TSN integration and related features | TS 23.501 clauses 5.27.1a-5.27.5, 5.28 |  | X |  |  |  |  | X |  | X |  |
| 15.4 | gPTP based uplink time synchronization, PTP support, UE-UE TSC communication, exposure of TSC services, survival time, time sync accuracy | TS 23.501 clauses 5.27.1.2.2, 5.27.1.4-9 |  |  | X |  |  | X | X | X | X |  |
| 15.5 | Support for coverage area filters for time synchronization service | TS 23.501 clause 5.27.1.10 |  |  |  | X |  |  |  |  | X |  |
| 15.6 | Controlling time synchronization service based on the Subscription | TS 23.501 clause 5.27.1.11 |  |  |  | X |  |  |  | X | X |  |
| 15.7 | Network timing synchronization status monitoring | TS 23.501 clause 5.27.1.12 |  |  |  | X |  | X | X | X | X |  |
| 15.8 | Support for TSN enabled Transport Network | TS 23.501 clause 5.28a |  |  |  | X |  |  |  | X | X |  |
| 15.9 | RAN feedback for Burst Arrival Time offset and adjusted Periodicity | TS 23.501 clause 5.27.2.5 |  |  |  | X |  |  |  | X | X |  |
| 15.10 | Integration with IETF Deterministic Networking | TS 23.501 clause 5.27.28.5 |  |  |  | X |  |  |  |  | X |  |
| 16 | Support for non-public networks |  |  |  |  |  |  |  |  |  |  |  |
| 16.1 | Support for SNPN | TS 23.501 clause 5.30.2 |  | X |  |  |  | X | X | X | X |  |
| 16.2 | Support for SNPN access mode | TS 23.501 clause 5.30.2 |  | X |  |  |  | X |  |  |  |  |
| 16.3 | SNPN connectivity for UEs with credentials owned by Credentials Holder | TS 23.501 clause 5.30.2.9 |  |  | X |  |  | X | X | X | X |  |
| 16.4 | Onboarding of UEs in SNPNs | TS 23.501 clause 5.30.2.10 |  |  | X |  |  | X | X | X | X |  |
| 16.5 | Equivalent SNPNs | TS 23.501 clause 5.30.2.6 |  |  |  | X |  | X | X |  | X |  |
| 16.6 | Non-3GPP access to SNPNs | TS 23.501 clause 5.30.2.12-15 |  |  |  | X |  |  | X |  | X |  |
| 16.7 | Access to SNPNs providing access for Localized Services | TS 23.501 clause 5.30.2.3-4 |  |  |  | X |  | X | X |  | X |  |
| 16.8 | Public Network Integrated NPN | TS 23.501 clause 5.30.2 |  | X |  |  |  | X | X | X | X |  |
| 16.9 | Validity information for CAG | TS 23.501 clause 5.30.3.3-4 |  |  |  | X |  | X | X |  | X |  |
| 16.10 | Remote provisioning of credentials for NSSAA or secondary authentication/authorization | TS 23.501 clause 5.39 |  |  | X |  |  |  | X |  | X |  |
| 17 | Support for Cellular IoT |  |  |  |  |  |  |  |  |  |  |  |
| 17.1 | Preferred and Supported Network Behaviour | TS 23.501 clause 5.31.2 |  | X |  |  |  |  | X |  | X |  |
| 17.2 | Selection, steering and redirection between EPS and 5GS | TS 23.501 clause 5.31.3 |  | X |  |  |  |  | X |  | X |  |
| 17.3 | Control Plane CIoT 5GS Optimisation | TS 23.501 clause 5.31.4 |  | X |  |  |  | X | X | X | X |  |
| 17.4 | Non-IP Data Delivery (NIDD) | TS 23.501 clause 5.31.5 |  | X |  |  |  |  | X |  | X |  |
| 17.5 | Reliable Data Service | TS 23.501 clause 5.31.5 |  | X |  |  |  |  | X |  | X |  |
| 17.6 | Extended Discontinuous Reception (DRX) for CM-IDLE and CM-CONNECTED with RRC-INACTIVE | TS 23.501 clause 5.31.7.2 |  | X |  |  |  | X | X | X | X |  |
| 17.7 | Paging for extended DRX for RRC_INACTIVE in NR connected to 5GC | TS 23.501 clause 5.31.7.2.4 |  |  |  | X |  |  |  | X | X |  |
| 17.8 | MICO mode | TS 23.501 clause 5.31.7.3-5 |  | X |  |  |  |  | X |  | X |  |
| 17.9 | High latency communication | TS 23.501 clause 5.31.8 |  | X |  |  |  |  | X |  | X |  |
| 17.10 | Support for Monitoring Events | TS 23.501 clause 5.31.9 |  | X |  |  |  |  |  |  | X |  |
| 17.11 | NB-IoT UE Radio Capability Handling | TS 23.501 clause 5.31.10 |  | X |  |  |  |  |  |  | X |  |
| 17.12 | Inter-RAT idle mode mobility to and from NB-IoT | TS 23.501 clause 5.31.11 |  | X |  |  |  |  |  |  | X |  |
| 17.13 | Restriction of use of Enhanced Coverage | TS 23.501 clause 5.31.12 |  | X |  |  |  |  | X |  | X |  |
| 17.14 | Paging for Enhanced Coverage | TS 23.501 clause 5.31.13 |  | X |  |  |  |  |  | X | X |  |
| 17.15 | Support of rate control of user data | TS 23.501 clause 5.31.14 |  | X |  |  |  |  | X |  | X |  |
| 17.16 | Control Plane Data Transfer Congestion Control | TS 23.501 clause 5.31.15 |  | X |  |  |  |  | X |  | X |  |
| 17.17 | Service Gap Control | TS5 23.501 clause 5.31.16 |  | X |  |  |  |  | X |  | X |  |
| 17.18 | Inter-UE QoS for NB-IoT | TS 23.501 clause 5.31.17 |  | X |  |  |  |  |  | X | X |  |
| 17.19 | User Plane CIoT 5GS Optimisation | TS 23.501 clause 5.31.18 |  | X |  |  |  | X | X | X | X |  |
| 17.20 | QoS model for NB-IoT | TS 23.501 clause 5.31.19 |  | X |  |  |  |  | X |  | X |  |
| 17.21 | Category M UEs differentiation | TS 23.501 clause 5.31.20 |  | X |  |  |  | X |  | X | X |  |
| 18 | Support for Ultra Reliable Low Latency Communication |  |  |  |  |  |  |  |  |  |  |  |
| 18.1 | Dual Connectivity based end to end Redundant User Plane Paths | TS 23.501 clause 5.33.2.1 |  | X |  |  |  |  |  | X | X |  |
| 18.2 | Support of redundant transmission on N3/N9 interfaces | TS 23.501 clause 5.33.2.2 |  | X |  |  |  |  |  | X | X |  |
| 18.3 | Per QoS Flow per UE QoS Monitoring | TS 23.501 clause 5.33.3.2 |  | X |  |  |  | X |  | X | X |  |
| 18.4 | GTP-U Path Monitoring | TS 23.501 clause 5.33.3.3 |  | X |  |  |  |  |  | X | X |  |
| 18.5 | Support of deployments topologies with specific SMF Service Areas | TS 23.501 clause 5.34 |  | X |  |  |  |  |  |  | X |  |
| 19 | Integrated access and backhaul (IAB) |  |  |  |  |  |  |  |  |  |  |  |
| 19.1 | Support for Integrated access and backhaul (IAB) | TS 23.501 clause 5.34 |  | X |  |  |  | X | X | X | X |  |
| 20 | Edge Computing |  |  |  |  |  |  |  |  |  |  |  |
| 20.1 | EASDF: Discovery of EAS using DNS request | TS 23.548 clause 6.2.3.2.2 |  |  | X |  |  |  |  |  | X |  |
| 20.2 | EAS Discovery Procedure with Local DNS Server/Resolver | TS 23.548 clause 6.2.3.2.3 |  |  | X |  |  |  |  |  | X |  |
| 20.3 | EAS Deployment Information (EDI) (i.e., AF providing EDI, capability to obtain and maintain EAS Deployment Information mapping table between IP address/range with DNAI) | TS 23.548 clause 6.2.3.4 |  |  | X |  |  |  |  |  | X |  |
| 20.4 | Edge relocation: Simultaneous Connectivity over source and target EAS | TS 23.548 clause 6.3.4 and Annex F |  |  | X |  |  |  |  |  | X |  |
| 20.5 | EAS IP replacement | TS 23.548 clause 6.3.3 |  |  | X |  |  |  |  |  | X |  |
| 20.6 | HR-SBO | TS 23.548 clause 6.7 |  |  |  | X |  |  |  |  | X |  |
| 20.7 | I-SMF insertion for local offloading management | TS 23.548 clause 6.10 |  |  |  |  | X |  |  |  | X |  |
| 20.8 | Common EAS/DNAI selection for group of UEs | TS 23.548 clause 6.2.3.2.4 and 6.2.3.2.5 |  |  |  |  |  |  |  |  | X |  |
| 21 | Multi-SIM |  |  |  |  |  |  |  |  |  |  |  |
| 21.1 | UE and network enhancements for Multi-USIM UEs | TS 23.501 clause 5.38 |  |  | X |  |  | X | X | X | X |  |
| 22 | Network analytics, AIML and related services |  |  |  |  |  |  |  |  |  |  |  |
| 22.1 | Network Analytics architecture | TS 23.501 clause 4.2.9 | X |  |  |  |  |  |  |  | X |  |
| 22.2 | Data collection from NFs and OAM | TS 23.288 clause 6.2.2 and 6.2.3 |  | X |  |  |  |  |  |  | X |  |
| 22.3 | Correlation between network data and service data | TS 23.288 clause 6.2.4 |  |  | X |  |  |  |  |  |  |  |
| 22.4 | Time coordination across multiple NWDAF instances | TS 23.288 clause 6.2.5 |  |  | X |  |  |  |  |  | X |  |
| 22.5 | Enhanced Procedures for Data Collection | TS 23.288 clause 6.2.6 |  |  | X |  |  |  |  |  | X |  |
| 22.6 | Data Collection with Event Muting Mechanism | TS 23.288 clause 6.2.7 |  |  | X |  |  |  |  |  | X |  |
| 22.7 | Data Collection from the UE Application | TS 23.288 clause 6.2.8 |  |  | X |  |  |  |  |  | X |  |
| 22.8 | User consent for analytics | TS 23.288 clause 6.2.9 |  |  | X |  |  |  |  |  | X |  |
| 22.9 | Roaming support for data collection | TS 23.288 clause 6.2.10 and 6.2.11 |  |  |  | X |  |  |  |  | X |  |
| 22.10 | Data Collection using LCS | TS 23.288 clause 6.2.12 |  |  |  | X |  |  |  |  | X |  |
| 22.11 | Rating untrusted AF data sources | TS 23.288 clause 6.2.13 |  |  |  | X |  |  |  |  | X |  |
| 22.12 | Analytics Collection from MDAF | TS 23.288 clause 6.2.14 |  |  |  | X |  |  |  |  | X |  |
| 22.13 | Data analytics (Slice load level, observed Service Experience, NF load, UE related, User Data Congestion, QoS Sustainability) | TS 23.288 clause 6.3-6.9 |  | X |  |  |  |  |  |  | X |  |
| 22.14 | Data analytics (Dispersion analytics, WLAN performance, Session Management Congestion Control Experience, Redundant Transmission Experience, DN Performance) | TS 23.288 clause 6.10-6.14 |  |  | X |  |  |  |  |  | X |  |
| 22.15 | Data analytics (PFD Determination, Location Accuracy, End-to-end data volume transfer time, Relative Proximity, PDU Session traffic, Movement Behaviour) | TS 23.288 clause 6.16-6.21 |  |  |  | X |  |  |  |  | X |  |
| 22.16 | Data analytics (Signalling Storm, QoS and Policy Assistance) | TS 23.288 clause 6.22-6.23 |  |  |  |  | X |  |  |  | X |  |
| 22.17 | Data Collection Coordination and Delivery | TS 23.288 clause 5A |  |  | X |  |  |  |  |  | X |  |
| 22.18 | Analytics Data Repository | TS 23.288 clause 5B |  |  | X |  |  |  |  |  | X |  |
| 22.19 | Multiple NWDAF support | TS 23.288 clause 6.1A |  |  | X |  |  |  |  |  | X |  |
| 22.20 | Trained ML Model Provisioning | TS 23.288 clause 4.2.0 |  |  | X |  |  |  |  |  | X |  |
| 22.21 | Data Collection from the UE Application | TS 23.288 clause 6.2.8 |  |  | X |  |  |  |  |  | X |  |
| 22.22 | Horizontal federated learning) | TS 23.288 clause 5.3 |  |  |  | X |  |  |  |  | X |  |
| 22.23 | Procedure for ML Model Training | TS 23.288 clause 6.2F |  |  |  | X |  |  |  |  | X |  |
| 22.24 | Analytics / ML model Accuracy Monitoring | TS 23.288 clause 5C |  |  |  | X |  |  |  |  | X |  |
| 22.25 | Member UE selection assistance functionality for application operation | TS 23.501 clause 5.46.2 |  |  |  | X |  |  |  |  | X |  |
| 22.26 | AI/ML based Positioning | TS 23.273 clause 5.18, 6.22 |  |  |  |  | X |  | X | X | X |  |
| 22.27 | Vertical Federated Learning | TS 23.288 clause 5.4 and clause 6.2H |  |  |  |  | X |  |  |  | X |  |
| 23 | V2X |  |  |  |  |  |  |  |  |  |  |  |
| 23.1 | Support for V2X in 5GS (fundamental NR Sidelink support, including unicast, groupcast, and new PC5 QoS model; Additionally over Uu: UE triggered Policy provisioning, QoS sustainability analytics support; ) | TS 23.287 |  | X |  |  |  | X | X | X | X |  |
| 23.2 | eV2X phase 2 enhancements (PC5 DRX, etc.) | TS 23.287 clause 4.4.2 |  |  | X |  |  | X | X |  | X |  |
| 23.3 | V2X communication with MBS | TS 23.287 clause 5.10 |  |  |  | X |  |  | X |  | X |  |
| 23.4 | V2X communication with PC5 Carrier Aggregation (configuration and mapping aspect to control the use of CA for some applications) | TS 23.287 clause 5.11 |  |  |  | X |  | X | X |  | X |  |
| 24 | ProSe |  |  |  |  |  |  |  |  |  |  |  |
| 24.1 | Support for ProSe in 5G (incl. Discovery - model A&B, Communication service, UE-to-Network Relay support, L2/3) | TS 23.304 |  |  | X |  |  | X | X | X | X |  |
| 24.2 | ProSe Phase 2 (e.g., 5G ProSe UE-to-UE Relay) | TS 23.304 |  |  |  | X |  | X | X |  | X |  |
| 24.3 | ProSe Phase 2 (UE-to-Network Relay enhancements, e.g. Multi-path, PWS support, emergency service support, mobility support) | TS 23.304 |  |  |  | X |  | X | X | X | X |  |
| 24.4 | Multi-hop UE-to-UE Relay | TS 23.304 |  |  |  |  | X |  | X |  | X |  |
| 24.5 | Multi-hop Layer-3 UE-to-Network Relay | TS 23.304 |  |  |  |  | X |  | X |  | X |  |
| 24.6 | ProSe support for NPNs | TS 23.304 clause 4.2.9 |  |  |  |  | X |  | X |  | X |  |
| 25 | Support of UAS connectivity, identification and tracking |  |  |  |  |  |  |  |  |  |  |  |
| 25.1 | UAV identification, authentication, authorization and tracking | TS 23.256 |  |  | X |  |  |  | X |  | X |  |
| 25.2 | Direct C2 Communication | TS 23.256 clause 5.4 |  |  |  | X |  | X | X |  | X |  |
| 25.3 | Broadcast Remote ID | TS 23.256 clause 5.5 |  |  |  | X |  | X | X |  | X |  |
| 25.4 | Detect and Avoid based on PC5 | TS 23.256 clause 5.6 |  |  |  | X |  |  |  |  |  |  |
| 25.5 | Ground-based DAA for an Area | TS 23.256 clause 5.7 |  |  |  | X |  |  |  |  |  |  |
| 25.6 | Pre-flight Planning and In-flight Monitoring for UAVs | TS 23.256 clause 5.12 |  |  |  |  | X |  |  |  | X |  |
| 25.7 | Multiple USS serving different geographical areas | TS 23.256 clause 5.13 |  |  |  |  | X |  |  |  | X |  |
| 25.8 | Support of Network-Based/Assisted DAA | TS 23.256 clause 5.14 |  |  |  |  | X |  |  |  | X |  |
| 25.9 | Altitude reporting for aerial UEs | TS 23.256 clause 5.16 |  |  |  |  |  | X |  | X | X |  |
| 26 | Multicast Broadcast Services |  |  |  |  |  |  |  |  |  |  |  |
| 26.1 | Multicast Broadcast Services | TS 23.247 |  |  | X |  |  | X | X | X | X |  |
| 26.2 | Group Message Delivery | TS 23.247 clause 6.15 |  |  |  |  |  |  |  |  | X |  |
| 26.3 | Support of MBS data reception for UEs using power saving functions | TS 23.247 clause 6.16 |  |  |  |  |  |  |  |  | X |  |
| 26.4 | Support of Multicast MBS session data reception in UE with RRC_INACTIVE state | TS 23.247 clause 6.17 |  |  |  | X |  | X |  | X | X |  |
| 26.5 | Resource sharing across broadcast MBS Sessions during network sharing | TS 23.247 clause 6.18 |  |  |  | X |  |  |  | X | X |  |
| 26.6 | NR RedCap and eRedCap support | TS 23.247 clause 6.19 |  |  |  | X |  | X |  | X | X |  |
| 26.7 | QoE Measurement Collection Support for MBS | TS 23.247 clause 6.20 |  |  |  | X |  | X |  |  | X |  |
| 27 | Advanced Interactive Services (AIS) / eXtended Reality (XR) |  |  |  |  |  |  |  |  |  |  |  |
| 27.1 | AIS-related 5QIs | TS 23.501 clause 5.37 |  |  | X |  |  |  | X | X | X |  |
| 27.2 | Policy control enhancements to support multi-modal services | TS 23.501 clause 5.37.2 |  |  |  | X |  |  |  |  | X |  |
| 27.3 | L4S | TS 23.501 clause 5.37.3 |  |  |  | X |  |  |  | X | X |  |
| 27.4 | Network Exposure of 5GS information | TS 23.501 clause 5.37.4 |  |  |  | X |  |  |  |  | X |  |
| 27.5 | PDU Set based Handling | TS 23.501 clause 5.37.5 |  |  |  | X |  | X | X | X | X |  |
| 27.6 | UL/DL policy control based on round-trip latency requirement | TS 23.501 clause 5.37.6 |  |  |  | X |  |  |  | X | X |  |
| 27.7 | 5GS Packet Delay Variation monitoring and reporting | TS 23.501 clause 5.37.7 |  |  |  | X |  |  |  | X | X |  |
| 27.8 | Periodicity and N6 Jitter Information associated with Periodicity | TS 23.501 clause 5.37.8.2 |  |  |  | X |  |  |  | X | X |  |
| 27.9 | End of Data Burst Indication | TS 23.501 clause 5.37.8.3 |  |  |  | X |  |  |  | X | X |  |
| 27.10 | Multi-modal Service ID to NG-RAN | TS 23.501 clause 5.37.2 |  |  |  |  | X |  |  | X | X |  |
| 27.11 | PDU Set Information Marking Support | TS 23.501 clause 5.37.5 |  |  |  |  | X |  |  |  | X |  |
| 27.12 | L4S support for N3IWF/TNGF | TS 23.501 clause 5.37.3.4 |  |  |  |  | X |  |  |  | X |  |
| 27.13 | DSCP marking based on PDU Set Information | TS 23.501 clause 5.8.2.7 |  |  |  |  | X |  |  |  | X |  |
| 27.14 | Support of transferring media related information over N6 | TS 23.501 clause 5.37.9 |  |  |  |  | X |  |  |  | X |  |
| 27.15 | Supporting dynamically changing traffic characteristics via the User Plane | TS 23.501 clause 5.37.10 |  |  |  |  | X |  |  | X | X |  |
| 27.16 | Packet filters for (S)RTP packet headers | TS 23.501 clause 5.7.6.2 |  |  |  |  | X |  | X |  | X |  |
| 27.17 | Expose UL and DL data rate limitation information | TS 23.501 clause 5.37.4 |  |  |  |  | X |  |  |  |  |  |
| 27.18 | Traffic identification and differentiated QoS for multiplexed media flows | TS 23.501 clause 5.37.11 |  |  |  |  | X |  | X |  | X |  |
| 28 | NR satellite access |  |  |  |  |  |  |  |  |  |  |  |
| 28.1 | NR satellite access | TS 23.501 clause 5.4.19, 5.4.11 |  |  | X |  |  | X | X | X | X |  |
| 28.2 | Support of discontinuous network coverage for satellite access | TS 23.501 clause 5.4.13 |  |  |  | X |  |  | X | X | X |  |
| 29 | Support for 5G Satellite Backhaul |  |  |  |  | X |  |  |  |  |  |  |
| 29.1 | Edge Computing via UPF deployed on satellite | TS 23.501 clause 5.43.2 |  |  |  | X |  |  |  |  | X |  |
| 29.2 | Local switch with PSA UPF deployed on satellite | TS 23.501 clause 5.43.3 |  |  |  | X |  |  |  |  | X |  |
| 29.3 | Reporting of satellite backhaul to SMF | TS 23.501 clause 5.43.4 |  |  |  | X |  |  |  |  | X |  |
| 29.4 | QoS monitoring when dynamic Satellite Backhaul is used | TS 23.501 clause 5.43.5 |  |  |  | X |  |  |  |  | X |  |
| 30 | Support of Personal IoT network service |  |  |  |  |  |  |  |  |  |  |  |
| 30.1 | UE policies for PIN | TS 23.501 clause 5.44.2 |  |  |  | X |  |  | X |  | X |  |
| 30.2 | Session management enhancement for PIN service support | TS 23.501 clause 5.44.3 |  |  |  | X |  |  | X |  | X |  |
| 30.3 | QoS differentiation of traffic for Non-3GPP Device Identifier | TS 23.501 clause 5.52 |  |  |  |  | X |  | X |  | X |  |
| 31 | Support for Mobile Base Station Relay (MBSR) |  |  |  |  |  |  |  |  |  |  |  |
| 31.1 | MBSR configuration, authorization, access control, mobility support, location service, etc. | TS 23.501 clause 5.35A |  |  |  | X |  |  | X |  | X |  |
| 32 | Ranging based services and sidelink positioning |  |  |  |  |  |  |  |  |  |  |  |
| 32.1 | Architecture Enhancements to support SL Ranging and positioning | TS 23.586 TS 23.273 (e.g. clause 5.17 and 6.20) |  |  |  | X |  | X | X | X | X |  |
| 33 | UE policy support |  |  |  |  |  |  |  |  |  |  |  |
| 33.1 | UE policy delivery | TS 23.503 | X |  |  |  |  |  | X |  | X |  |
| 33.2 | URSP Provisioning in EPS | TS 23.501 clause 5.17.8; TS 23.503 clause 6.1.2.2.3 |  |  |  | X |  |  | X |  | X |  |
| 33.3 | VPLMN specific URSP rules | TS 23.503 clause 6.1.2.2.4 |  |  |  | X |  |  | X |  | X |  |
| 33.4 | URSP rule enforcement reporting | TS 23.503 clause 6.6.2. 4 |  |  |  | X |  |  | X |  | X |  |
| 33.5 | Policy control decisions based on awareness of URSP rule enforcement | TS 23.503 clause 6.1.1.5 |  |  |  | X |  |  | X |  | X |  |
| 33.6 | Handling of the UE Policy Association | TS 23.503 clause 6.1.2.2.5 |  |  |  |  | X |  |  |  | X |  |
| 33.7 | Ranging/Sidelink Positioning Policy information | TS 23.503 clause 6.6.5 |  |  |  | X |  |  |  |  | X |  |
| 33.8 | A2X Policy information | TS 23.503 clause 6.6.6 |  |  |  | X |  |  |  |  | X |  |
| 34 | UPF enhancement for Exposure and SBA |  |  |  |  |  |  |  |  |  |  |  |
| 34.1 | Data exposure via SBI interface | TS 23.501 clause 5.8.2.17 |  |  |  | X |  |  |  |  | X |  |
| 34.2 | UE IP address and port exposure | TS 23.501 clause 5.8.2.17 |  |  |  |  | X |  |  |  | X |  |
| 34.3 | Handling of Payload Headers | TS 23.501 clause 5.8.2.22 and 5.6.17 |  |  |  |  | X |  |  |  | X |  |
| 34.4 | Operator configurable UPF capabilities | TS 23.501 clause 5.8.2.21 |  |  |  |  | X |  |  |  | X |  |
| 35 | Support of Energy Efficiency and Energy Saving |  |  |  |  |  |  |  |  |  |  |  |
| 35.1 | Support of Energy Information Function (EIF) and related energy consumption information collection, calculation and exposure | TS 23.501 clause 5.51.2 |  |  |  |  | X |  |  |  | X |  |
| 35.2 | Energy Saving subscription and indicator | TS 23.501 clause 5.51.5 |  |  |  |  | X |  |  | X | X |  |
| 36 | Support for Mobile gNB with Wireless Access Backhauling (MWAB) |  |  |  |  |  |  |  |  |  |  |  |
| 36.1 | MWAB architecture, authorization, configuration, etc. | TS 23.501 clause 5.49 |  |  |  |  | X |  | X | X | X |  |
| 37 | Support for NR Femto |  |  |  |  |  |  |  |  |  |  |  |
| 37.1 | 5GS architecture to support NR Femto | TS 23.501 clause 5.50.2 |  |  |  |  | X |  |  | X | X |  |
| 37.2 | CAG Information Provisioning | TS 23.501 clause 5.50.3 |  |  |  |  | X |  |  |  | X |  |
| 38 | Support of Ambient IoT |  |  |  |  |  |  |  |  |  |  |  |
| 38.1 | Architecture support for Ambient IoT | TS 23.369 |  |  |  |  | X |  |  | X | X |  |
| 39 | Policy and Charging Control Framework |  |  |  |  |  |  |  |  |  |  |  |
| 39.1 | Negotiation for future background data transfer | TS 23.503 clause 6.1.2.4 | X |  |  |  |  |  |  |  | X |  |
| 39.2 | AF influence on Access and Mobility related policy control | TS 23.503 clause 6.1.2.6 |  |  | X |  |  |  |  |  | X |  |
| 39.3 | Negotiation for planned data transfer with QoS requirements | TS 23.503 clause 6.1.2.7 |  |  |  | X |  |  |  |  | X |  |
| 39.4 | Policy control for redundant PDU Sessions | TS 23.503 clause 6.1.3.24 |  | X |  |  |  |  |  |  | X |  |
| 39.5 | User Plane Remote Provisioning of UE SNPN Credentials in Onboarding Network | TS 23.503 clause 6.1.3.25 |  |  | X |  |  |  |  |  | X |  |
| 39.6 | UL/DL policy control based on round-trip latency requirement | TS 23.503 clause 6.1.3.27.2 |  |  |  | X |  |  |  |  | X |  |
| 39.7 | Expedited Data Transfer with Reflective QoS | TS 23.503 clause 6.1.3.27.9 |  |  |  |  | X |  |  |  | X |  |
| 39.8 | AF requested QoS for a UE or group of UEs not identified by a UE address | TS 23.503 clause 6.1.3.28 |  |  |  | X |  |  |  |  | X |  |
| 39.9 | Network Slice replacement for a PDU Session | TS 23.503 clause 6.1.3.29 |  |  |  | X |  |  |  |  | X |  |
| 39.10 | Handling of Payload Headers Control | TS 23.503 clause 6.1.3.30 |  |  |  |  | X |  |  |  | X |  |
| 39.11 | Policy control for non-3GPP devices connecting behind a UE | TS 23.503 clause 6.1.3.31 |  |  |  |  | X |  |  |  | X |  |

# >>>>END OF CHANGES<<<<