3GPP TSG-WG2 Meeting #170	S2-2506329

# Göteborg, Sweden, August 25-29, 2025

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.256 | CR | 0202 | rev | - | Current version: | 19.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Removing the Nnef_UAVFlightAssistance_Delete operation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, LG Electronics |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | UAS_Ph3 |  |  |  |  |  | Date: |  |  | 2025-08-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | CT3 has agreed to use UAV Flight Assistance Configuration Update to both update and/or stop the altitude information reporting for UAV UEs (see details in C3-252419 (https://www.3gpp.org/ftp/tsg_ct/WG3_interworking_ex-CN3/TSGC3_141_Bratislava/Docs/C3-252419.zip ), since the conclusions have been that Delete operation is not fit stop reporting of the altitude information. Consequently, the Nnef_UAVFlightAssistance_Delete becomes redundant and needs to be removed from the SA2 spec so that it does not create descrepancy with Stage 3 description. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Removing Nnef_UAVFlightAssistance_Delete and the related description from the respective clauses. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Misalignment/descrepancy with Stage 3 decision and specifications. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.4.1.1.1, 4.4.1.1.3.2, 4.4.1.1.3.4, 5.16.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

>>>> Start of Changes <<<<

##### 4.4.1.1.1	General

In addition to those defined in TS 23.501 [2] clause 7.2.8 and TS 23.502 [3] clause 5.2.6, the following table illustrates additional NEF services to support UAS.

Table 4.4.2.1.1-1: NF Services provided by NEF

| Service Name | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| Nnef_Authentication | AuthenticateAuthorize | Request/Response | AMF, SMF |
|  | Notification | Subscribe/Notify | AMF, SMF |
| Nnef_UAVFlightAssistance |  |  |  |
|  | Create | Subscribe/Notify | AF/USS |
|  | Update | Subscribe/Notify | AF/USS |
|  | Notify | Subscribe/Notify | AF/USS |
| Nnef_RetrieveInfoUAVFlight | Get | Request/Response | AF/USS |

>>>> Next Change <<<<

4.4.1.1.3.2

Void

>>>> Next Change <<<<

4.4.1.1.3.4	Nnef_UAVFlightAssistance_Update service operation

Service operation name: Nnef_UAVFlightAssistance_Update

Description: The consumer can use this service operation to delete/update its previously provided information about UAV flight path(s) and/or time schedule and to cancel the associated subscriptions, when, for instance, the serving USS determines that it does not need the assistance from the 5GS, and it can be used by the service consumer to inform the UAS NF/NEF about UAV's altitude.

Input, Required: Flight plan configuration ID.

Input, Optional: Flight path information, which may include e.g. a list of TA, UAV's speed, flight height/altitude and/or time schedule for crossing/spending at each of the TAs/cells), Notification Target Address, UAV's altitude information, indication to stop the configured altitude information reporting.

Output, Required: None.

Output, Optional: None.

>>>> Next Change <<<<

### 5.16.4	Stop and update altitude reporting for aerial UEs

Procedure shown in Figure 5.16.4-1 specifies how the 5GS instructs the aerial UEs to stop and/or update their altitude reporting due to a trigger from NG-RAN and/or a request from the USS.

Figure 5.16.4-1: Procedure for instructing aerial UEs to stop/update the altitude reporting

0.	The UAV UE is requested to perform altitude information reporting as specified in clause 5.16.2, and the UAV UE performs its altitude information reporting as specified in clause 5.16.3.

1.	If the UAV UE fails to report the altitude information and/or the NG-RAN to receive the altitude information, the UAS NF/NEF or the USS may decide to stop/update the UAV UE's altitude reporting.

NOTE 1:	The UAS NF/ NEF or the serving USS can also decide to stop UAV UE's altitude reporting in cases when the UAV UE deviates from the assigned flight trajectory, which can be detected during the in-flight monitoring (clause 5.12.3) or the USS changeover (clause 5.13.2), or when the DAA procedure is triggered to avoid a collision (clause 5.14).

Option A: Application layer.

2a.	The UAV UE informs the USS over the application layer about an issue to provide altitude information reporting as requested/configured earlier.

3a.	The USS decides either to update the altitude reporting configuration at the UAV UE or to stop the altitude information reporting. The respective request is sent by the USS to the UAV UE over the application layer.

4a. The USS may invoke the Nnef_UAVFlightAssistance_Update service operation to inform the UAS NF/ NEF about the altitude reporting of the UAV UE and/or to update/delete the UAV UE's context.

Option B: 3GPP-defined signalling.

2b.	The NG-RAN node sends an N2 message(LOCATION REPORTING FAILURE INDICATION message, see clause 8.12.2.2 of TS 38.413 [24]) to the AMF with information about the configured altitude information reporting.

3b.	The AMF invokes the Namf_EventExposure_Notify service operation towards UAS NF/NEF to inform about the configured altitude information reporting.

4b.	The UAS NF / NEF may decide to invoke the Nnef_UAVAssistance_Notify service operation to inform the USS about the altitude information reporting event. The UAS NF / NEF may also skip this step in cases that the UAS NF / NEF determines there is no longer a need for altitude information reporting from the UAV UE, for instance, when the UAV UE has reached the flight's destination; in such a case, the procedure proceeds with step 6b.

5b.	Based on the Nnef_UAVAssistance_Notify request from the UAS NF/NEF (step 4b) or USS's own decision (outside the 3GPP scope), the USS may request the 5GS to update/stop UAV UE's altitude reporting.

The USS invokes the Nnef_UAVFlightAssistance_Update service operation with the indication to update/stop the altitude information reporting for this UAV UE.

6b.	The UAS NF / NEF invokes the Namf_EventExposure_Unsubscribe service operation to the AMF to unsubscribe from the altitude information reports; for this purpose, the UAS NF /NEF may include values of the altitude thresholds and/or periodicity, if applicable, indicating to stop the altitude information reporting.

NOTE 2:	The UAS NF /NEF can indicate the AMF to stop altitude information reporting, for example, by setting the altitude thresholds and/or reporting periodicity to some maximum values; the exact details and attributes are described in Stage 3 specifications.

If the USS decides to change altitude reporting information (e.g. to request new altitude reporting thresholds and/or reporting periodicity) and includes new altitude reporting thresholds and/or reporting periodicity values in the Nnef_UAVFlightAssistance_Update request in step 5b, the AMF needs to send a new Namf_EventExposure_Subscribe request, see Steps 3b-5b in clause 5.16.2.

7b-8b.	The AMF sends the corresponding N2 message (LOCATION REPORTING CONTROL message, see clause 8.12.1.2 of TS 38.413 [24]) to the NG-RAN node and the NG-RAN triggers the release of the configured altitude reporting for the UAV UE, thereby stop the altitude reporting, as specified in TS 38.331 [23].

NOTE 3:	Aspects related to AMF informing NG-RAN regarding start/stop of UAV UE's altitude reporting are specified in TS 38.413 [24].

>>>> End of Changes <<<<