SA WG2 Meeting #170	S2-2506356

25 – 29 Aug, 2025, Goteborg , SE

Source:	China Unicom, China Telecom, CATT

Title:	[WT#1.2, Network Sharing] Architectural requirements on 6G network sharing

Document for:	Agreement

Agenda Item:	20.6.1.2

Work Item / Release:	FS_6G_ARC/Rel-20

Abstract of the contribution: This pCR is intending to justify on why and how to consider the 6G network sharing in SA2 R20 6G study and give the architectural requirements for 6G network sharing.

1. Justifications

1.1 Observations on 6G network sharing

With the resources and infrastructures of 6G identified, they need to be efficiently utilized. Network sharing technology is well known as a typical technology in this area that needs to be considered by service providers. Previously, network sharing technology has been incorporated into 3GPP standards as a fundamental feature in various generations of mobile communication technologies.

Considering the time plan for industrialization of the future 6G system, the network sharing technology needs to be included at the beginning Release of the 6G network design, including:

    - Like traditional 5G network sharing technologies, 6G network sharing technologies enable multiple operators to share network access resources such as new 6G radio access network, NTN access network, etc.

    - Extending the scope and types of resources and infrastructures to be shared, such as sharing network resources of new feature (e.g. sharing sensing resources to enable the sensing feature).

    - Extending network sharing to support on demand network sharing or event triggered network sharing (e.g. disaster occurrence, network failure, overloaded situation, resource constraints).

- 6G network sharing can enhance the utilization of 6G network through strengthening inter-operator collaboration, thickening the value of network capacity and reducing the energy consumption of 6G networks, which is in line with the design concept of low-carbon and environmental protection for future networks.

1.2 Analysis on the architectural impact

Based on the observations above, the architectural impacts on 6G network sharing are considered as below:

1.2.1) There are two existing network sharing technologies in 5G (5G MOCN and INS), they have their own scope of application to support the NG-RAN sharing. How to achieve them to support the 6G access resources (e.g., new 6G radio access resource, NTN access resource) sharing on the basis of 6G network architecture should be considered.

1.2.2) 6G system will support sensing and other features, the operators need to deploy the new network resources or infrastructures to support these features. In order to promote the rapid commercial deployment of these features, how to support to share these network resources, e.g. access resource with sensing capability in 6G network needs to be considered.

1.2.3) In order to support the on-demand network sharing or event triggered network sharing, the architecture design of 6G network sharing could be more flexible, e.g. considering the flexible trigger to turn on/off the 6G network sharing and reducing network sharing pre-configurations between the operators.

1.3 Proposal on the architectural requirements

For 1.2.1) and 1.2.2), the relevant SA2 works on 6G network sharing are dependent on the progress of the study on the overall network architecture and other features (e.g. NTN, sensing). Therefore, 1.2.1) and 1.2.2) can be summarized as the architectural requirements recorded in the SA2 R20 6G TR, i.e., the study on the overall network architecture and other features (e.g. NTN, sensing) needs to consider how to support the network sharing on the relevant network resources.

For 1.2.3), when designing the architecture for 6G network sharing, it is important to consider the flexibility of network sharing trigger and the need to reduce network sharing pre-configurations between the operators. 1.2.3) is dependent on the related work on 6G network sharing and can also be summarized as a architectural requirement included in the SA2 R20 6G TR.

Considering the factual deployment plan of 6G network, based on the progress of the SA2 R20 6G study and considering the architectural requirements on 6G network sharing, the SA2 normative work on 6G network sharing should start in the SA2 R21 6G TS.

In summary, this pCR proposes the architectural requirements for 1.2.1), 1.2.2) and 1.2.3).

**** First Change ****

4.2	Architectural Requirements

Editor's Note: This clause defines the architectural requirements that serve as the foundation for the study.

The architecture of 6G System shall:

Support network sharing:

- Support the network sharing architectures to share the 6G access resources (e.g., new 6G radio access resource,      	NTN access resource), at least including 6G Multi-Operator Core Network (6G MOCN) and 6G Indirect Network 	Sharing (6G INS).

- Support to extend the scope and types of network resources and infrastructures to be shared in 6G, e.g. network 	sharing on 6G access network with sensing capability.

- Support the on-demand network sharing or event triggered network sharing, e.g. considering the flexible trigger   	to turn on/off the 6G network sharing and reducing network sharing pre-configurations between the operators.

**** End of Changes ****