3GPP TSG SA WG2 Meeting #170	S2-2506361

Goteborg, Sweden, 25 – 29 August 2025	(revision of S2-250xxxx)

Source:	China Mobile, AT&T, Rakuten Mobile, OPPO

Title:	[WT#6] WT and key issues for supporting 6G computing

Document for:	Agreement

Agenda Item:	20.6.6

Work Item / Release:	FS_6G_ARC/Rel-20

Abstract of the contribution: This paper proposes the discussion of 6G architecture to support computing, and proposes the WT and KI for 6G computing

1. Justifications

1.1 Summary of SA1 requirements related to computing

As discussed in TR 22.870 of SA1, some of the solutions are related to the computing requirements aspects. In Table 1.1-1, it shows the computing related PRs in SA1 that till SA1#100 meeting.

Table 1.1-1: SA1 computing related requirements collection till SA1#110 meeting

| Category | PR details |
| --- | --- |
| Supported services | - Supported AI computing (e.g.: training) and rendering (e.g.: 6GoF, spatial audio). - Support computing offloading from UE to network - Support providing computing service to user. |
| Computing resource management | - Support to collect and maintain the computing resource related status information and network status information. - Support to expose to an authorized 3rd party the information of computing resources. |
| Computing Schedule | - Support to receive the computing requirements. And map the UE side computing requirements to network side requirement. - Support computing node selection and reselection according to some of the information - Support minimum interruption, continuity and coordination when UE mobility. |
| Others PRs | - Support charging. - Support to transfer the offloading task to computing node with certain degrees of QoS. - Guarantee the QoS of computing task, e.g.: E2E time delay. - Has the priority to maintain the computing task first. - Exposure the UE and computing node time delay information to assist the computing node selection. - Security and privacy. |

As shown above, there are 2 main uses cases that related to computing, one is AI capabilities and another one is rendering service in immersive.

(1) Computing resource for AI tasks

From the UE perspective, the UE now consistently leverages certain AI models (possibly small LLMs), while performing AI tasks faces some challenges. As described in section 6.32.1 and 6.33.1, to improve the accuracy of AI inference and meet the requirements of AI tasks, the UE can collaborate with the network and offload some AI tasks. Some reference of text is copied below:

Refer to section 6.32.1 of TR 22.870[x]:

Considering the limited capabilities, computing resources, and battery power of the terminal devices, these complex AI computing might not be able to run locally on the terminals with an ideal experience.

Refer to section 6.33.1 of TR 22.870[x]:

In many circumstances, the required computing support for an AI inference (e.g. AI model/Gen AI based inference) task is so significant that the UE may not have sufficient computing power and knowledge data base to run the AI model inference.

In many circumstances, the required computing support for some AI operations, such as AI inference (e.g. AI model/Gen AI based inference) task, is so significant that the UE may not have sufficient computing resources to run the AI model inference by itself. The computing resources consumed for a specific AI model inference is proportional to the size of the AI model and the input/output tokens.

(2) Immersive service requires computing service for rendering.

The immersive services require high-quality image processing capability like high order super-resolution and de-noising algorithm, as well as high-quality rendering capability like ray tracing effect for gaming. And this image processing capability has high reliability to computing resource for example GPU. It is shown in Figure 9.2.1-1 of TR 22.870[x] that the computing resources required by immersive services has increased exponentially, far exceeding the computing resources that can be provided by the UE side. Therefore, offloading high computing task to the site that providing sufficient computing capabilities, is a promising trend to support advanced immersive services in the future, with lowering the weight and the power consumption of the XR glasses and mobile phones.

In SA1, the 6G computing service is defined as below:

6G Computing Service: A service provided by 6G network utilizing computing resources in Service Hosting Environment, which can be used by a subscriber (via UE)/3rd party.

So in conclusion, the new 6G architecture should consider the new requirements and support the new 6G computing service for the subscriber.

Observation 1: It may be required by the UE to offload the computing task from UE to the network to support some use cases.

1.2 Motivation of introducing computing in 6G

(1) Offloading computing Service requirement to operator’s computing resources may have the benefits to guarantee user experiences.

According to the SA1 requirement in TR 22.870 that related to computing aspect, such as PR 6.33.6-1, it says:

[PR 6.33.6-1] Subject to operator policy and user consent, the 6G network shall be able to authorize a user to offload task from a 3rd party application (e.g. a AI inference workload) to the Service Hosting Environment.

That the Service Hosting Environment can have different understanding. The computing resource can be provided by the computing resource deployed internally to core network (e.g.: computing resources deployed behind the UPF), or provided by edge cloud owned by operator, even the public cloud provided by 3rd party. While the computing resource is provided by computing resource deployed internal core network may have benefits that described below.

The operator can guarantee the user experience and E2E QoS if offloading to computing resource deployed in the core network.

As shown in Fig 1.2-1, the computing resource in internal core network is deployed after the PSA. This is totally different from the Internet service that the server is deployed in data network. The interface between PSA and computing node can be seen as the internal interfaces. It means that the E2E QoS and user experience can be controllable and optimized by operator. But the N6 delay between PSA and data network is uncontrolled.

Figure 1.2-1: Example of computing resource deployment in operator’s domain

(2) The operators can deploy computing resources at different levels in the of network.

The operator network architecture has several levels, such as national or provincial data centre, regional data centre, and access site. Therefore, operators have flexible places to deploy computing resources, and at the same time, they can provide users with ubiquitous access to computing resources. For the computing resource provided by 3rd party, public cloud computing resources are usually located in centralized data centres, which brings larger E2E latency.

1.3 Gap analysis: 5G edge computing vs 6G Computing

One of the key features introduced in 5G is edge computing. In R15 and R16, fundamental edge computing features are introduced including traffic impact, UL CL and etc to support traffic steering to edge. And in R17, new network function EASDF is introduced to act as the DNS resolver inside 5GC to assist to resolve the FQDN in UE DNS query to the EAS IP address which the edge application server near the UE in location.

There are some differences between 5G edge computing and 6G computing:

(1) The purpose of 5G edge computing is to address the issues of local breakout to 3rd party’s application server.

5G edge computing focuses on the edge application server discovery near the UE location, and requires the 5GC to establish the traffic routing between UE and EAS. In 5G, application doesn’t have the requirements to computing, and whether the edge server satisfies the computing or not is also not discussed. But in 6G, based on SA1 requirements, some high computing required services have the requirements on computing, and the edge application server may not be the optimal on computing. Also, what 5G computing define is to select the edge application server located locally. It depends on that the 3rd party has already deployed application instance locally and operator cannot control this.

(2) Latency on N6 interface is uncontrollable.

Considering that the edge server is deployed in data network that behind the N6 interface, and the N6 interface is always not controllable by operator. It means that the time delay on N6 interface is also not controllable. So, the E2E service experience between UE and edge server is also uncontrollable. This cannot meet the requirements of 6G new services, such as 6G system should guarantee the QoS of computing Service requirement, e.g.: E2E time delay.

(3) According to the SLA with all kinds of application provider, the operator-controlled computing nodes can deploy the application instance to serve the UE at several location.

In 5G edge computing, the edge application server is deployed by 3rd party, and if the service provider doesn’t deploy the server on this location, there will be no edge computing. While compared with the 5G, in 6G we are discussing about the computing resources in core network. That is to say the operators can control the edge application deployment (based on the SLA) and provide the services to UE based on UE/application request.

4.4 Analysis of technical impacts

The overall network architecture and network functions need to be enhanced to support the computing features in 6G.

Firstly, the 6G network should support to manage the trusted computing resources that deployed by operator internally to the core network. The 6G network can support the registration of computing node and maintain the status information of computing node.

Secondly, in order to support computing offloading from UE, the network should support to receive the computing Service requirement from UE. And the details of the parameters of computing task requirements need to be defined. And the 6G network should consume certain metrics (e.g., compute, service, etc.) to selects/reselects the compute nodes that are deployed internally to the core network for computing Service requirement.

At last, the overall computing features involved in network also needs to adapt to both UE mobility and service continuity.

The security, privacy and charging issues also have potential impact to network architecture and service procedure.

**** First Change ****

Annex A.X. WT#X Scope

WT#6: Study aspects on support of computing for UE, core network and application server in 6G (e.g., coordination between UE, core network and application server, exposure of computing service in the core network, etc.).

NOTE 1: Application layer mechanism and exposure framework may require coordination with SA6.

**** Second Change ****

5.X. Key Issue #X: Support of computing services in 6G network

This key issue will address the system architecture and function required to provide end-to-end support for computing.

The aspects to be studied in this key issue include:

-	What information can be used to represent the computing service requirement from UE.

-	How to handle the UE’s request and requirement for computing services.

-	What network functions can be introduced to handle the computing service requirement, and when and how to select the network functions.

-	Whether and how to support the traffic transmission between the UE and computing nodes.

-	How to collect and manage the status information of computing resource from computing nodes, and how to expose the information to an authorized 3rd party.

-	Whether and how to define the QoS for computing services e.g., bandwidth, processing latency.

-	How to guarantee the QoS for computing service during UE mobility.

**** End of Changes ****