3GPP SA WG2 Meeting #170	S2-2506367

# Gothernburg, Sweden, 25-29 August, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.304 | CR | 0557 | rev |  | Current version: | 19.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification of Identifiers for 5G ProSe Multi-hop UE-to-UE Relay for IP PDU type |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_ProSe_Ph3 |  |  |  |  |  | Date: |  |  | 2025-08-25 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | - The use of optional target Multi-hop UE-to-UE relay info is incomplete and it is unclear how a Discoverer UE (doing a Model B discovery) can receive this information beforehand. In SA2#169 there were some discussions on the use of this information but it was not specified how an end UE (doing Model B discovery) could (optionally) gather this information of a target Relay. - Various clauses mentioned in 5.8.6 are incorrect. Clause 5.1.5.1 describes Policy/Parameter provisioning for 5G ProSe UE-to-UE Relay while the correct clause number should be 5.1.5.1a Policy/Parameter provisioning for 5G ProSe Layer-3 Multi-hop UE-to-UE Relay - Editorial mistakes are there in clause 5.8.6.2 and 5.8.3.1 |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Clarified in a note that the Target Relay UE may be obtained by a Discoverer UE using an earlier Model A discovery (while still keeping the parameter optional, and keeping Model B discovery independent from Model A discovery) or it may have been obtained via an earlier connection established between the UE and that target relay. - Corrected referece to clause 5.1.5.1->5.1.5.1a in clauses 5.8.6.1 and 5.8.6.2 - Other minor corrections in few other clauses |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incorrect and unclear specifications |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.8.3.1;5.8.6.1;5.8.6.2; 6.3.2.6.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

CHANGE 1

#### 5.8.3.1	Common identifiers for 5G ProSe UE-to-Network Relay

The following parameters are used for the 5G ProSe UE-to-Network Relay Discovery Announcement message (Model A), where Source Layer-2 ID and Destination Layer-2 ID are used for sending and receiving the message and Announcer Info and Relay Service Code are contained in the message:

-	Source Layer-2 ID: the 5G ProSe UE-to-Network Relay self-selects a Source Layer-2 ID for 5G ProSe UE-to-Network Relay Discovery.

-	Destination Layer-2 ID: the Destination Layer-2 ID for 5G ProSe UE-to-Network Relay Discovery is selected based on the configuration as described in clause 5.1.4.1.

-	Announcer Info: provides information (i.e. User Info ID) about the announcing user.

-	Relay Service Code: parameter identifying a connectivity service the 5G ProSe UE-to-Network Relay provides to a 5G ProSe Remote UE. The Relay Service Codes are configured in a 5G ProSe UE-to-Network Relay for advertisement. Additionally, the Relay Service Code may also identifies authorized users the 5G ProSe UE-to-Network Relay would offer service to and may be used to select the related security policies or information e.g. necessary for authentication and authorization between the 5G ProSe Remote UE and the 5G ProSe UE-to-Network Relay (e.g. a Relay Service Code for relays for police members only would be different than a Relay Service Code for relays for Fire Fighters only, even though potentially they provided connectivity to same DN e.g. to support Internet Access).

The 5G ProSe UE-to-Network Relay Discovery Announcement message (Model A) is extended and modified as follows when used for 5G ProSe Multi-hop UE-to-Network Relay discovery:

-	Hop-Count: This value reflects the number of PC5 hops for the 5G ProSe Remote UE to reach the network. It is set to 1 by the 5G ProSe UE-to-Network Relay and shall be incremented by 1 every time the 5G ProSe UE-to-Network Relay Discovery Announcement message is forwarded by a 5G ProSe Intermediate UE-to-Network Relay.

A 5G ProSe Intermediate UE-to-Network Relay shall only process the 5G ProSe UE-to-Network Relay Discovery Announcement if the Hop-Count is present and the value is less than the (pre-)configured maximum number of hops for the associated RSC and the optional Hop-Limit in the message.

-	Source Layer-2 ID: the 5G ProSe UE-to-Network Relay or the 5G ProSe Intermediate UE-to-Network Relay self-selects a Source Layer-2 ID when sending the discovery message.

-	Destination Layer-2 ID: the 5G ProSe Intermediate UE-to-Network Relay sets the Destination Layer-2 ID of the 5G ProSe UE-to-Network Relay Discovery Announcement message based on configuration described in clause 5.1.4.1.

-	Announcer Info: identify information (i.e. User Info ID) of the announcing 5G ProSe UE-to-Network Relay or the 5G ProSe Intermediate UE-to-Network Relay.

-	(optional) Root Relay Info: this is the User Info ID of the 5G ProSe UE-to-Network Relay. A 5G ProSe Intermediate UE-to-Network Relay may stored it in the discovery entry and included it in the 5G ProSe UE-to-Network Relay Discovery Announcement message. This information can be used in the relay selection at the 5G ProSe Remote UE or other 5G ProSe Intermediate UE-to-Network Relays.

-	(optional) Accumulated QoS for PC5 link: this reflects the QoS supported over all the PC5 links to the Root Relay, i.e. the 5G ProSe UE-to-Network Relay, for this RSC.

-	(optional) Hop-Limit: this is the hop limit set by the announcing 5G ProSe UE-to-Network Relay to a value smaller than the configured maximum number of hops.

The following parameters are used for the 5G ProSe UE-to-Network Relay Discovery Solicitation message (Model B), where Source Layer-2 ID and Destination Layer-2 ID are used for sending and receiving the message and Discoverer Info and Relay Service Code are contained in the message:

-	Source Layer-2 ID: the 5G ProSe Remote-UE self-selects a Source Layer-2 ID for 5G ProSe UE-to-Network Relay Discovery.

-	Destination Layer-2 ID: the Destination Layer-2 ID for 5G ProSe UE-to-Network Relay Discovery is selected based on the configuration as described in clause 5.1.4.1.

-	Discoverer Info: provides information (i.e. User Info ID) about the discoverer user.

-	Target Info: provides information (i.e. User Info ID) about the targeted discoveree user.

-	Relay Service Code: information about connectivity that the discoverer UE is interested in. The Relay Service Codes are configured in the 5G ProSe Remote UEs interested in related connectivity services.

To support Multi-hop 5G ProSe UE-to-Network Relay discovery the following parameters are added:

-	(Optional) Hop-Count: indicates the number of hops that the message is already relayed. It is increased by 1 per hop.

-	(Optional) Hop-Limit: an unmodified value that indicates the hop limit of the message. It is set, by the 5G ProSe Remote UE, to a value smaller than the (pre)configured maximum number of hops.

-	Path information: an (ordered) list of User Info ID(s) of Intermediate UE-to-Network Relay(s) that indicates the transmitted path of the message.

-	(optional) Accumulated QoS for PC5 link: this reflects the QoS supported over all the PC5 links starting from the PC5 link between the Remote UE and an Intermediate UE-to-Network Relay in the path information for this RSC.

The following parameters are used in the 5G ProSe UE-to-Network Relay Discovery Response message (Model B), where Source Layer-2 ID and Destination Layer-2 ID are used for sending and receiving the message and Discoveree Info and Relay Service Code are contained in the message:

-	Source Layer-2 ID: the 5G ProSe UE-to-Network Relay self-selects a Source Layer-2 ID for 5G ProSe UE-to-Network Relay Discovery.

-	Destination Layer-2 ID: set to the Source Layer-2 ID of the received 5G ProSe UE-to-Network Relay Discovery Solicitation message.

-	Relay Service Code: identifies the connectivity service the 5G ProSe UE-to-Network Relay provides to 5G ProSe Remote UEs that matches the Relay Service Code from the corresponding Discovery Solicitation message.

-	Discoveree Info: provides information (i.e. User Info ID) about the discoveree.

To support Multi-hop 5G ProSe UE-to-Network Relay discovery the following parameters are added:

-	(Optional) Hop-Count: indicates the number of hops between the 5G ProSe Remote UE and the 5G ProSe UE-to-Network Relay on the path selected by the 5G ProSe UE-to-Network Relay.

-	Path information: an (ordered) list of User Info ID(s) of the discoverer user and the Intermediate UE-to-Network Relay(s) on the path selected by the 5G ProSe UE-to-Network Relay.

-	(optional) Accumulated QoS for PC5 link: this reflects the QoS supported over all the PC5 links along the path from Remote UE to the 5G ProSe UE-to-Network Relay for this RSC.

The following parameters may be used in the Relay Discovery Additional Information message (using Model A) based on the procedure defined in clause 6.5.1.3 for 5G ProSe UE-to-Network Relay and clause 6.3.2.5.4 for 5G ProSe Multi-hop UE-to-Network Relay where Source Layer-2 ID and Destination Layer-2 ID are used for sending and receiving the message and the other parameters are contained in the message:

-	Source Layer-2 ID: the 5G ProSe UE-to-Network Relay or the 5G ProSe Intermediate UE-to-Network Relay self-selects a Source Layer-2 ID to send the Relay Discovery Additional Information message.

-	Destination Layer-2 ID: the Destination Layer-2 ID to send the Relay Discovery Additional Information message is selected based on the configuration as described in clause 5.1.4.1.

-	Relay Service Code: the Relay Service Code associated with the message. The Relay Service Code is used to identify the security parameters needed by the receiving UE to process the discovery message.

Announcer Info: provides information about the announcing user that transmits or forwards the Relay Discovery Additional Information message (i.e. User Info ID of the 5G ProSe UE-to-Network Relay or 5G ProSe Intermediate UE-to-Network Relay). User Info ID of the 5G ProSe Intermediate UE-to-Network Relay is only applicable to the additional parameter announcement procedure over 5G ProSe Multi-hop UE-to-Network Relay specified in clause 6.3.2.5.4.

-	Hop-Count: This value reflects the number of PC5 hops for the 5G ProSe Remote UE to reach the network. It is set to 1 by the 5G ProSe UE-to-Network Relay and shall be incremented by 1 every time the Relay Discovery Additional Information message is forwarded by a 5G ProSe Intermediate UE-to-Network Relay.

A 5G ProSe Intermediate UE-to-Network Relay shall only process the Relay Discovery Additional Information message if the Hop-Count is present and the value is less than the (pre-)configured maximum number of hops for the associated RSC and the optional Hop-Limit in the message.

-	(optional) Root Relay Info: provides the User Info ID of the 5G ProSe UE-to-Network Relay.

-	(optional) Announcer Info of 5G ProSe Intermediate UE-to-Network Relay: identify information (i.e. User Info ID) of the 5G ProSe Intermediate UE-to-Network Relay connected to 5G ProSe UE-to-Network Relay. This parameter is only applicable to the additional parameter announcement procedure over 5G ProSe Multi-hop UE-to-Network Relay specified in clause 6.3.2.5.4.

-	(optional) Hop-Limit: this is the hop limit set by the announcing 5G ProSe UE-to-Network Relay to a value smaller than the configured maximum number of hops.

-	Additional parameters: the additional parameters for 5G ProSe Layer-3 UE-to-Network Relay (when applicable) are defined in clause 5.8.3.2.

NOTE 1:	The UE implementation needs to ensure that when the UE self-selects Source Layer-2 IDs, the self-selected Source Layer-2 IDs are different between 5G ProSe Direct Discovery (including 5G ProSe UE-to-Network Relay Discovery) in clause 6.3.2 and 5G ProSe Direct Communication (including 5G ProSe UE-to-Network Relay Communication) in clause 6.4 and are different from any other provisioned Destination Layer-2 IDs as described in clause 5.1 and any other self-selected Source Layer-2 IDs used in a simultaneous 5G ProSe Direct Discovery (including 5G ProSe UE-to-Network Relay Discovery) with a different discovery model.

NOTE 2:	If a 5G ProSe UE-to-Network Relay and 5G ProSe Remote UE from different PLMNs discover each other, it means that the Relay Service Code is associated with the same connectivity service and the same Relay Service Code is provisioned based on Service Level Agreement among PLMNs.

NOTE 3:	The Hop count, Hop-Limit and Path information are used for Multi-hop 5G ProSe UE-to-Network Relay Discovery with Model B. The Multi-hop UE-to-Network Relay Discovery message has no impact on 5G ProSe UE-to-Network Relay or 5G ProSe Remote UE that only supports the single-hop UE-to-Network Relay. Single-hop and Multi-hop UE-to-Network Relays discoveries can be distinguished based on RSC.

CHANGE 2

### 5.8.6	Identifiers for 5G ProSe Multi-hop UE-to-UE Relay Discovery

#### 5.8.6.1	General

There are two types of discovery mechanisms for the support of 5G ProSe Multi-hop UE-to-UE Relays, i.e. for IP PDU type and for non-IP PDU type (Ethernet or Unstructured). The 5G ProSe End UE and 5G ProSe Multi-hop UE-to-UE Relay choose the discovery mechanism based on the associated indication of the RSC, as defined in clause 5.1.5.1a.

The identifiers used for the two types of discovery mechanisms are specified in clause 5.8.6.2 and clause 5.8.6.3 respectively.

CHANGE 3

#### 5.8.6.2	Identifiers for 5G ProSe Multi-hop UE-to-UE Relay discovery of IP PDU type

The Multi-hop UE-to-UE Relay Discovery Announcement message (Model A) is sent by the relay and contains the following information:

-	Source Layer-2 ID: the 5G ProSe Layer-3 Multi-hop UE-to-UE Relay self-selects a Source Layer-2 ID. It is used for the transmitting the message at AS layer and included in the AS layer headers only.

-	Destination Layer-2 ID: the Destination Layer-2 ID for Multi-hop UE-to-UE Relay Discovery Announcement message (Model A) is selected based on the configuration (associated with the RSC) as defined in clause 5.1.5.1a.

-	User Info ID of 5G ProSe Multi-hop UE-to-UE Relay: provides information about the 5G ProSe Layer-3 Multi-hop UE-to-UE Relay.

-	Relay Service Code: information to indicate the connectivity service the 5G ProSe Layer-3 Multi-hop UE-to-UE Relay provides to the 5G ProSe End UEs.

The 5G ProSe Multi-hop UE-to-UE Relay Discovery Solicitation message (Model B) is sent by the source 5G ProSe End UE or a 5G ProSe Layer-3 Multi-hop UE-to-UE Relay and includes the following information:

-	Source Layer-2 ID: the discoverer 5G ProSe End UE or 5G ProSe Layer-3 Multi-hop UE-to-UE Relay self-selects a Source Layer-2 ID.

-	Destination Layer-2 ID: the Destination Layer-2 ID for 5G ProSe Multi-hop UE-to-UE Relay Discovery Solicitation message is selected based on the configuration (associated with the RSC) as defined in clause 5.1.5.1a.

User Info of discoverer: provides information about the sender of the Solicitation message.

-	(optional) User Info ID of 5G Prose UE-to-UE Relay: this may be used by the receiver of the message to determine whether to respond to the solicitation. It is only included when the sender wants to find a specific target relay.

-	Relay Service Code: information about connectivity service offered by the 5G ProSe Layer-3 Multi-hop UE-to-UE Relay(s).

The 5G ProSe UE-to-UE Relay Discovery Response message (Model B) sent by the 5G ProSe Layer-3 Multi-hop UE-to-UE Relay(s) matching the RSC includes the following:

-	Source Layer-2 ID: the 5G ProSe Layer-3 Multi-hop UE-to-UE Relay self-selects a Source Layer-2 ID.

-	Destination Layer-2 ID: set to the Source Layer-2 ID of the received 5G ProSe Multi-hop UE-to-UE Relay Discovery Solicitation message.

-	User Info of discoverer: the User Info from the received Solicitation message, to ensure the response can be matched at the receiver.

-	User Info ID of 5G ProSe UE-to-UE Relay: provides information about the 5G ProSe Multi-hop UE-to-UE Relay sending the Response message.

-	Relay Service Code: information about connectivity service offered by the 5G ProSe Layer-3 Multi-hop UE-to-UE Relay.

CHANGE 4

##### 6.3.2.6.2	Procedure for 5G ProSe Multi-hop UE-to-UE Relay discovery of IP PDU type

Depicted in figure 6.3.2.6.2-1 is the procedure for 5G ProSe Multi-hop UE-to-UE Relay discovery of IP PDU type, with Model A.

Figure 6.3.2.6.2-1: 5G ProSe Multi-hop UE-to-UE Relay Discovery of IP PDU type with Model A

1.	The 5G ProSe Multi-hop UE-to-UE Relay is configured to operate IP PDU type of discovery based on the indication associated with the RSC, as defined in defined in clause 5.1.5.1.

2.	The 5G ProSe Multi-hop UE-to-UE Relay sends a Multi-hop UE-to-UE Relay Discovery Announcement message. This message contains the Type of Discovery Message, User Info ID of the 5G ProSe Multi-hop UE-to-UE Relay and the RSC, as defined in clause 5.8.6.2.

The 5G ProSe Multi-hop UE-to-UE Relay self generates a Source Layer-2 ID and decides a Destination Layer-2 ID as defined in clause 5.8.6.2.

The 5G ProSe End UE(s) or other 5G ProSe Multi-hop UE-to-UE Relay(s) in proximity monitoring the same RSC may receive the announcement message and process it accordingly, e.g. initiates a Layer-2 Link establishment with the announcing 5G ProSe Multi-hop UE-to-UE Relay.

Depicted in figure 6.3.2.6.2-2 is the procedure for 5G ProSe Multi-hop UE-to-UE Relay discovery of IP PDU type, with Model B. A 5G ProSe Multi-hop UE-to-UE Relay may also use the procedure, i.e. send the Multi-hop UE-to-UE Relay Discovery Solicitation (taking the role of the 5G ProSe End UE (Discoverer)), if it needs to discover other 5G ProSe Multi-hop UE-to-UE Relays.

Figure 6.3.2.6.2-2: 5G ProSe Multi-hop UE-to-UE Relay Discovery of IP PDU type with Model B

1.	The 5G ProSe End UE is configured to operate IP PDU type of discovery based on the indication associated with the RSC, as defined in defined in clause 5.1.5.1.

2.	The 5G ProSe End UE sends a Multi-hop UE-to-UE Relay Discovery Solicitation message. This message contains the Type of Discovery Message, RSC and (optional) User Info ID of the 5G ProSe Multi-hop UE-to-UE Relay, as defined in clause 5.8.6.2.

The User Info ID of the 5G ProSe Multi-hop UE-to-UE Relay is only included if the 5G ProSe End UE wants to find a specific target relay.

NOTE 1:	The Discoverer UE can obtain the User Info ID of the specific target relay e.g. via an earlier Multi-hop UE-to-UE relay Discovery Announcement message (Model A) sent by that relay or via an earlier communication procedure(s) with that relay.

The 5G ProSe End UE self generates a Source Layer-2 ID and decides a Destination Layer-2 ID as defined in clause 5.8.6.2.

3.	The 5G ProSe Multi-hop UE-to-UE Relay-1 may send a Multi-hop UE-to-UE Relay Discovery Response message, if the RSC contained in the solicitation message matches any of the (pre)configured RSC(s), as specified in clause 5.1.5.1. If User Info ID is included in the solicitation message, 5G ProSe Multi-hop UE-to-UE Relay-1 only sends the response message when it matches its own User Info ID. This response message contains the User Info of discoverer, User Info ID of the 5G ProSe Multi-hop UE-to-UE Relay, the RSC and type of Discovery message, as defined in clause 5.8.6.2.

The 5G ProSe Multi-hop UE-to-UE Relay self generates a Source Layer-2 ID and decides a Destination Layer-2 ID as defined in clause 5.8.6.2.

The 5G ProSe Multi-hop UE-to-UE Relay-1 in proximity may decide not to respond if the criteria are not met, or due to UE local configurations.

END of Changes