3GPP TSG-SA2 Meeting #170	S2-2506382

Gothenburg, Sweden, August 25-29 2025

Source:	vivo

Title:	KI#1 Interim agreements

Document for:	Approval

Agenda Item:	20.3.1

Work Item / Release:	FS_AIML_CN_Ph2/ Rel-20

Abstract: This document proposes some interim agreements for KI#1: Transfer of data over UP for UE data collection.

1.	Introduction

Some interim principles are proposed for KI#1: Transfer of data over UP for UE data collection.

2. Text Proposal

It is proposed to capture the following changes in TR 23.700-04.

# * * * * First change * * * *

# 7	Interim agreements

## 7.1	Agreed Principles

### 7.1.1	Agreed Principles for KI#1

Editor's note:	This clause will include the principles that are agreed as work progresses for the specific KI#Y. This may be populated directly or e.g. also when a topic in clause 7.2.Y gets resolved and a principle is agreed.

- Subject to user consent, a termination entity (e.g. DCCF) in CN is utilized to control the standardized data transferring from UE to data consumer, i.e. UE model training entity server.

-   The termination entity sends its UP information to UE via NAS, which includes target address for data reporting.

- Based on the UP information for data reporting, the UE establishes a logical UP connection with the termination entity via a PDU session, which is selected or established based on URSP. The URSP can be enhanced to include the FQDN/ target address for data reporting in TD and the DNN/S-NSSAI in RSD.

- To differentiate the traffic for UE data collection from the UE regular traffic in the 5GC, dedicated DNN and S-NSSAI (optional) could be used for the UP connection for UE data transferring.

- According to data request from the data consumer, the DCCF can request the UE to transfer data with specific requirements e.g. data type, reporting time, data format and etc.

- According to the information and requirements in the data transferring request from the termination entity, the UE reports the collected data to the termination entity via the UP connection.

NOTE: which standardized data can be collected by the UE will be defined by RAN1. The mechanism how UE collects data and corresponding RRC configuration depend on RAN1 and RAN2’s decision.

- After receiving the collected data from UE, the termination entity can expose the data to the UE model training entity server with the authorization of NEF.

- The termination entity may store the collected data from UE to ADRF.

# * * * * End of change * * * *