3GPP TSG-SA2 Meeting #170	S2-2506383

25 - 29 August, 2025, Goteborg, Sweden	(revison of xxxx)

Source: 	vivo

Title: 	KI#2 Solution #24 Updates

Document for: 	Approval

Agenda Item: 	20.3.1

Work Item / Release:	FS_AIML_CN_Ph2 / Rel-20

Abstract of the contribution: This contribution proposes an update on KI#2 solution#24 to address ENs.

1	Discussion

This paper proposes to update the solution#24 of KI#2 to address some Editor’s notes.

To address the following EN, some new clause clarifying the input data should be collected, the possible output data, and the possible actions of NFs are added.

The User Plane Traffic Exception can be classified into three types: burst, malicious, and suspicious traffic. Corresponding mitigation actions include traffic splitting or switching for burst traffic, traffic dropping for malicious traffic, and traffic redirection for suspicious traffic.

Editor's note:	Input data and output data of the new analytics ID are FFS. What actions can be done by which consumers based on corresponding user plane traffic exception is FFS.

Editor's note:	What specified user plane traffic pattern data should be collected to train ML model is FFS.

For the last EN, the collocated UPF and AnLF can interact with each other based on existing SBA service, e.g. Nnwdaf_AnalyticsSubscription server and Nupf_EventExposure service.

Editor's note:	Whether and how to do interaction between the collocated UPF and AnLF is FFS.

2. Proposal

It is proposed to add the following contents to TR 23.700-04 0.2.0.

Start of Change

## 6.24	Solution #24: Collocation of UPF and NWDAF containing AnLF to support efficient performance of User Plane

### 6.24.1	High level principles

The NWDAF containing MTLF is used to train ML model of User Plane Traffic Exception.

The UPF is collocated with NWDAF containing AnLF and it is used to generate analytics result of User Plane Traffic Exception based on the ML model from the NWDAF containing MTLF.

Based on the analytics result of User Plane Traffic Exception, the consumer performs corresponding mitigation actions to avoid burst, malicious or suspicious traffic.

### 6.24.2	Description

#### 6.24.2.0	General

This solution is to address Key Issue#2: AI/ML-assisted user plane traffic pattern and behaviour analysis to support efficient performance of User Plane.

Since UPFs in the network have plenty observed information and measurements data about UP traffic, the NWDAF containing MTLF can use these input data from UPFs to train ML model of User Plane Traffic Exception. This ML model will be provided to NWDAF containing AnLF to generate analytics of User Plane Traffic Exception by performing ML model inference process.

However, considering latency and big data volume of input data reporting from UPF to the NWDAF containing AnLF during ML model inference process, this solution proposes to collocate the UPF and the NWDAF containing AnLF together. In this case, the ML model from the NWDAF containing MTLF will be offloaded to UPF collocated with AnLF, then the UPF can generate analytics result itself and this result can assist the consumer to perform some corresponding mitigation actions, e.g. avoid burst, malicious or suspicious traffic.

#### 6.24.2.1	Input Data

Table 6.24.2.1-1: UP Pattern data collected by NWDAF

| Information | Source | Description |
| --- | --- | --- |
| UE ID | SMF, UPF | Identifier of the UE (e.g. SUPI) associated with the collected UP-related data. |
| UE IP Address | SMF, UPF | IP address assigned to the UE for the observed UP-related data. |
| S-NSSAI | SMF, UPF | Single Network Slice Selection Assistance Information associated with the observed UP-related data. |
| DNN | SMF, UPF | Data Network Name corresponding to the PDU Session. |
| PDU Session ID | SMF | Identifier of the PDU Session. |
| UPF ID | UPF | Identifier of the UPF (e.g. IP address or FQDN). |
| UP related data | SMF, UPF | User Plane–related data collected for the session. May include one or more of the following: |
| > Traffic description | UPF | Detailed description of the observed traffic, e.g. IP tuple (SrcIP, DstIP, SrcPort, DstPort, Protocol). (IP 5-tuple or 3-tuple) |
| > Traffic direction | UPF | Indicates the direction of traffic, i.e. uplink or downlink |
| > Duration | SMF, UPF | Observation time range for the collected data (start and end timestamps). |
| > UPF interface info | SMF, UPF | Identifier and address of the UPF interface where traffic was observed (e.g. N3 for RAN side, N6 for DN side). |
| > Delay information | SMF, UPF | Latency observed at relevant interfaces, e.g. N6 (N6 Delay), N3/N9 (GTP-U path delay), N4 (PFCP Heartbeat delay), as defined in clauses 5.33.8, 5.24.5, and 6.2.2 of TS 29.244 [14]. |
| > PFCP session info | SMF, UPF | Summary of PFCP session context (e.g. PDR/FAR/QER/URR/BAR) status and any PFCP errors per clause 7.6 of TS 29.244 [14] for the UP pattern. |
| > Sampled info | UPF | Information related to the traffic sample rate, if sampling is used. |
| > Traffic volume | UPF | Total traffic size (including average, maximum, minimum). |
| > Number of packets | UPF | Total number of packets within the observed traffic. |

Note: Additional input data types and their descriptions can be considered as described in Solution #23.

#### 6.24.2.2	Output data

Table 6.24.2.2-1: Output Analytics of UP Traffic Exception statistics

| Information | Description |
| --- | --- |
| Time slot entry (1..max) | List of time slots during the Analytics target period. |
| > Exception type | Indicates the exception, i.e. one of burst, malicious, suspicious. |
| > Time slot start | Time slot start within the Analytics target period. |
| > Duration | Duration of the time slot. |
| > traffic description | Information describing the observed traffic for identification. May include one or more of the following: |
| >> UPF ID | Identifier of the UPF where the traffic was observed. |
| >> PDU session ID | Identifier of the PDU Session related to the traffic. |
| >> Qos flow ID | Identifier of the QoS Flow related to the traffic. |
| >> IP tuple | IP flow information, e.g. SrcIP, DstIP, SrcPort, DstPort, Protocol. (IP 5-tuple or 3-tuple) |

Table 6.24.2.2-2: Output Analytics of UP Traffic Exception predictions

| Information | Description |
| --- | --- |
| Time slot entry (1..max) | List of time slots during the Analytics target period. |
| > Exception type | Indicates the type of exception, i.e. one of burst, malicious, suspicious. |
| > Time slot start | Time slot start within the Analytics target period. |
| > Duration | Duration of the time slot. |
| > traffic description | Information describing the observed traffic for identification. May include one or more of the following: |
| >> UPF ID | Identifier of the UPF where the traffic was observed. |
| >> PDU session ID | Identifier of the PDU Session related to the traffic. |
| >> Qos flow ID | Identifier of the QoS Flow related to the traffic. |
| >> IP tuple | IP flow information, e.g. SrcIP, DstIP, SrcPort, DstPort, Protocol. (IP 5-tuple or 3-tuple) |
| > Confidence | Confidence of this prediction. |

#### 6.24.2.3	Example actions

Table 6.24.2.3-1: Examples of NF actions for exception traffic mitigation

| Exception type | Analytics consumer | Example actions of consumers |
| --- | --- | --- |
| Burst traffic | SMF | The SMF may perform traffic splitting or traffic switching. - For the traffic splitting, the SMF may use ULCL mechanism to route some traffic to a new UPF to mitigate the load of the original UPF, as described in clause 4.3.5.4 of TS 23.502 [3]. - For the traffic switching, the SMF may change the PSA UPF from the UPF suffering from burst traffic to a new UPF for some PDU session to mitigate the load of original UPF. The SMF may perform different session modification procedures based on the different SSC mode of PDU session as described in clause 4.3.5 of TS 23.502 [3]. |
| Malicious traffic | UPF | The UPF may drop the traffic based on malicious traffic description. |
|  | AF | The AF may stop the related application. |
| Suspicious traffic | UPF | The UPF may send suspicious traffic information to SMF, and the SMF may perform traffic redirection for further traffic analysis or traffic isolation. - For the traffic redirection, the SMF may redirect the suspicious traffic to another network entity or a specific UPF for further traffic analysis or traffic isolation based on the redirection information which may be obtained from the interactions between SMF and PCF as described in clause 6.1.3.12 of TS 23.503 [4]. |

### 6.24.3	Procedures

#### 6.24.3.1	Support of efficient performance of User Plane by collocating UPF and NWDAF containing AnLF

Figure 6.24.3.1-1: Support of efficient performance of User Plane by collocating UPF and NWDAF containing AnLF

0.	[Optional] SMF could be the analytics consumer and subscribes to the NWDAF containing AnLF collocating with UPF for analytics result of User Plane Traffic Exception, by invoking the Nnwdaf_AnalyticsSubcription_Subscribe service operation. The subscription request may include the exception type to indicate the type of exception, e.g. burst, malicious and suspicious, and filter information, e.g. target time interval and reporting information.

1.	AnLF collocated with UPF subscribes to NWDAF containing MTLF for a trained ML Model of User Plane Traffic Exception, by invoking the Nnwdaf_MLModelProvision_Subscribe service operation.

2.	To train the ML model of User Plane Traffic Exception, the NWDAF containing MTLF collects input data from UPF(s), by invoking the Nupf_EventExposure_Subscribe service operation.

3-4.	The UPF(s) collects user plane traffic pattern data (e.g. UPF local data, N3 measurements, N6 measurements, as defined in table 6.24.2.1-1) and notifies the NWDAF containing MTLF by invoking the Nupf_EventExposure_Notify service operation.

NOTE 1:	To avoid much impacts on UPF performance and reduce signalling load for data reporting, the data reporting from UPF for ML model training can be done during off-peak time.

NOTE 2:	N3 measurements, e.g. UL/DL packet drop rate GTP-U, UL/DL packet delay GTP as defined in existing specifications don't have new RAN impacts.

Editor's note:	If it is feasible to train models outside peak hours but still update models quick enough for new observed malicious traffic is FFS. Whether and what other solutions to reduce data reporting load for model training are FFS.

5.	NWDAF containing MTLF trains ML model of User Plane Traffic Exception based on training data collected from data sources e.g. UPFs.

6.	The NWDAF containing MTLF provides the trained ML model to the AnLF collocated with UPF, by invoking Nnwdaf_MLModelProvision_Notify service operation.

7.	Based on the ML model from the NWDAF containing MTLF, the UPF collocated with AnLF collects user plane traffic pattern data, e.g. UPF local data, N3 measurements, N6 measurements, etc. as input data for ML model inference. Then the AnLF collocated with UPF generates analytics result of User Plane Traffic Exception based on the ML model. The consumer (e.g. the UPF) performs specific action related to the User Plane Traffic Exception.

The interactions between a collocated UPF and AnLF are performed using existing SBA services. For example, the UPF may invoke the Nnwdaf_AnalyticsSubscription service to request analytics results for a User Plane Traffic Exception, while the NWDAF containing the AnLF may invoke the Nupf_EventExposure service to collect the corresponding input data.

8.	[Optional] After receiving the analytics results of malicious or suspicious traffic, the UPF may perform actions related to the User Plane Traffic Exception, such as dropping malicious traffic or send the suspicious traffic information to the SMF, as described in table 6.24.2.3-1.

9a.	[Conditional] If the exception type of User Plane Traffic Exception analytics results is suspicious, the UPF may inform the SMF of the suspicious traffic information via an N4 message for further traffic analysis or traffic isolation. The suspicious traffic information could include suspicious traffic exception indication, the traffic description and the corresponding time.

9b.	[Conditional] After receiving the suspicious traffic information from UPF, the SMF may perform specific actions related to the suspicious traffic information, such as traffic redirection as described in table 6.24.2.3-1.

10a.	[Optional] If the SMF has subscribed to the analytics result of User Plane Traffic Exception in step 0, the NWDAF containing AnLF shall send the analytics results to the SMF to indicate the User Plane Traffic Exception.

10b.	[Optional] After receiving the User Plane Traffic Exception notification from the NWDAF containing AnLF, the SMF may perform specific actions based on exception type, such as traffic splitting and traffic switching for burst traffic, as described in table 6.24.2.3-1.

### 6.24.4	Impacts on services, entities and interfaces

NWDAF containing AnLF (collocated with UPF):

-	Retrieves ML model for Traffic pattern from NWDAF containing MTLF.

-	Generates analytics of User Plane Traffic Exception based on the ML model.

NWDAF containing MTLF:

-	Trains and provides ML model of User Plane Traffic Exception.

UPF:

-	Executes mitigation actions corresponding to the traffic exception notification.

-	Sends suspicious traffic information to the SMF.

SMF:

-	Executes mitigation actions corresponding to the suspicious traffic information.

End of Changes