SA WG2 Meeting #170	S2-2506388

25 – 29 Aug, 2025, Goteborg, SE

Source:	OPPO

Title:	[WT#1.2] Session and QoS enhancement for Error Tolerant Communication

Document for:	Discussion

Agenda Item:	20.6.1

Work Item / Release:	FS_6G_ARC/Rel-20

Abstract of the contribution: This contribution proposes WT update and Key Issue description for WT#1.2 of the SA2 6G SID to support Error Tolerant Communication.

1. Justifications

  - Motivation

In 6G, the real-time service such as XR rendering and video streaming will become more and more popular, which requires a high bandwidth and sensitive service continuity. However the bandwidth limitation makes it difficult to widely deploy the services for a high density of end-users to use it simultaneously. On the other hand, with the advent of advanced algorithm (AI or non-AI based), the application layer can recover some kind of the information missed due to data transmission which it is called “Error Tolerant Communication (ETC)”. The Error Tolerant Communication can tolerate a data transmission with a much lower QoS (e.g., a high data transmission error rate with 20%) while the Quality of User Experience is little or not affected.

There are some related use cases agreed in TR 22.870-031 such as clause 9.10, 9.13, 9.14, and 6.27. The basic idea of those requirements is to give the 6G network some flexibility to lower the QoS (e.g. reliability) based on specific kind of data being transmitted for the same real-time service (i.e., not the highest QoS is always needed for Error Tolerant Communication). Then QoS resource can be saved for more users and other services, and it is an effective way to resolve the bandwidth limitation.

Technical Impact: Compared to 5G, there are some gaps including the following aspects:

1)	In 5G QoS, the highest PER of standardized 5QI is 10-2 and the Maximum PLR is 3%, however the Error Tolerant Communication can apply 10% (or even much higher) data transmission error rate.

2)	Current PER (Packer Error Rate) is per packet (e.g. IP, Ethernet) granularity, and PDU Set Error Rate is per PDU set granularity, however it is unsure what granularity of data transmission error rate is suitable for the Error Tolerant Communication. It is also unclear if the current criterion of successful data packet transmission is suitable for the Error Tolerant Communication.

3) 	Given a certain kind of the service data can be applied to Error Tolerant Communication, the 6G system need to get aware of when and which service data is for “Error Tolerant Communication” and apply the corresponding adaptive QoS on demand.

**** First Change (all new text) ****

Annex A.X. WT#X Scope

Editor's Note: Describe the technical scope of the proposed Work Task. If applicable, suggest logical subdivision of this WT into smaller sub-WT. This clause is part of the TR Annex.

WT#1.2.	Study whether and how to support and/or enhance the following aspects in 6G: the SBA framework, network slicing, network sharing, user plane architecture, QoS framework, policy framework, network exposure framework, architecture for specific scenarios e.g. fixed wireless access, localized service access.

Under the work task#1.2, it should study the QoS framework and session management for Error Tolerant Communication allowing data transmission with high error rate, including

-	Whether and how to enhance the QoS parameter (e.g. PER) to transmit the data traffic with adaptive QoS which is used for Error Tolerant Communication allowing data transmission with high error rate.

-	Whether and how to support 6G system to get aware of when and which data traffic needs to be applied for data transmission with high error rate for the Error Tolerant Communication.

-	Whether and how to enhance session management related procedure to support Error Tolerant Communication.

**** Second Change (all new text) ****

5.X. Key Issue #X: Session and QoS enhancement for Error Tolerant Communication

Editor's Note: This clause defines the potential scope of KI(s) and is part of the TR.

Study whether and how to support and/or enhance the QoS framework and session management for Error Tolerant Communication allowing data transmission with high error rate, including:

-	Whether and how to enhance the QoS parameter (e.g. PER) to transmit the data traffic with adaptive QoS which is used for Error Tolerant Communication allowing data transmission with high error rate.

-	Whether and how to support 6G system to get aware of when and which data traffic needs to be applied for data transmission with high error rate for the Error Tolerant Communication.

-	Whether and how to enhance session management related procedure to support Error Tolerant Communication.

**** End of Changes ****