SA WG2 Meeting #170	S2-2506393

25 – 29 Aug, 2025, Goteborg, SE

Source:	OPPO

Title:	[WT#1.1] Support of Control Signalling in 6GS

Document for:	Agreement

Agenda Item:	20.6.1.1

Work Item / Release:	FS_6G_ARC/Rel-20

Abstract of the contribution: This paper proposes the update on the WT#1.1 description and new KI for the support of control signalling aspect in 6G system.

# 1. Justifications

Editor's Note: This justification section is not included in the TR; it is intended solely to justify the scope of the proposed WT.

  - Motivation

The volume of UEs is expected to be significantly boosted due to wide use of Robots and Automobiles in the 6G era, thus the NAS signaling may be overloaded in 6G system. If all NAS signaling piggybacked in Access and Mobility Management NAS messages goes over a centralized AMF in cloud, the AMF will become a bottleneck for NAS routing.

Furthermore, routing all NAS messages over a centralized AMF between the UE and a localized NF (e.g., I-SMF, local SMF, UE-PCF, LMF) will lead to a certain level of message routing delay.

For the NAS messages for core functionality (e.g., RM, MM, SM and UE Policy Management, etc), they can still be routed over Control Plane, but always routing the NAS messages over a centralized AMF in cloud doesn’t seem to be a scalable solution for 6GS, thus a de-centralized NAS routing mechanism is expected to be on board.

For the MNO services (e.g., LCS, AI and Sensing, etc.), the control signaling interaction between UE and the corresponding NFs is expected to be more frequent and have a bigger message size than the NAS signaling for core functionality (e.g., RM, MM, SM and UE Policy Management, etc.), thus it may cause more burden for Control Plane. In order to reduce the burden for NAS transfer over Control Plane, the signaling for MNO services can be routed over the User Plane instead of Control Plane.

  - Technical Impact

New functions may need to be introduced into 6GS to support an evolved control signalling transfer, which will require new interfaces and corresponding interaction among the UE and network functions.

**** First Change (all new text) ****

It’s proposed to update the Work Task#1.1 as following:

# Annex A.X. WT#1.1 Scope

Editor's Note: Describe the technical scope of the proposed Work Task. If applicable, suggest logical subdivision of this WT into smaller sub-WT. This clause is part of the TR Annex.

WT#1: Define the overall 6G architecture as collection of capabilities and high level functionalities considering the following sub work tasks and other work tasks to support 6G access network:

NOTE  1:	The duplication of functionality in RAN and CN will be avoided, while maintaining the existing RAN and CN functionality split.

1.1.	Study the support for control signalling for 6G for connectivity and/or beyond connectivity services, including at least the following:

- a.	Whether and how to enable the introduction of a new non-access stratum functionality without impacting other non-access stratum functionalities; How to route the NAS signalling for basic connectivity between UE and the according NFs with less NAS signalling routing delay and avoiding potential NAS signalling congestion via a centralized AMF.

- b.	Whether and how to identify a minimal set of non-access stratum functionalities that does not get impacted by additional non-access stratum functionalities.

- c.	How to support the control signalling (i.e., beyond connectivity service message) interaction for beyond connectivity services (e.g., LCS, Sensing, AI, etc.) between UE and the according NFs; Whether and how to develop generic mechanisms for UE-Core Network interaction to support beyond connectivity services.

**** Second Change (all new text) ****

# 5.X. Key Issue #X:  NAS signalling routing for connectivity

Editor's Note: This clause defines the potential scope of KI(s) and is part of the TR.

This key issue proposes to study how to route the NAS signalling for connectivity between UE and the according NFs. In this key issue, the NAS layer signalling is still transferred over control plane. The key issue will investigate the following aspects:

- How to route the NAS signalling for basic connectivity between UE and the according NFs with less NAS signalling routing delay and avoiding potential NAS signalling congestion via a centralized AMF; This aspect can investigate whether new network function for NAS signalling routing is needed.

- The UE ID (e.g., 6G-GUTI) allocation and structure.

- The network function selection and UE context storage.

**** Third Change (all new text) ****

# 5.y. Key Issue #y:  Control signalling routing for Beyond Connectivity services

This key issue proposes to study how to support the control signalling (i.e., beyond connectivity service message) interaction for MNO service between UE and the according NFs. The key issue will investigate following aspects:

- Whether and how to route the control signalling for Beyond Connectivity services (e.g., LCS, Sensing, AI, etc.) between UE and the according NFs over user plane; This aspect can investigate whether new network function for control signalling routing is needed.

NOTE 1: Whether a generic mechanism should be defined for Beyond Connectivity service signalling transfer will be determined in TR conclusion.

- Network function selection and UE context storage.

**** End of Changes ****