3GPP TSG-WG2 Meeting #170	S2-2506394

# 25th – 29th August, 2025, Goteborg, Sweden

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.273 | CR | 0750 | rev | - | Current version: | 19.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections on user consent and privacy profile for AIML-based positioning |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Apple, Mediatek Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S2 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIML_CN |  |  |  |  |  | Date: |  |  | 2025-08-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | This CR implements the proposals outlined in the discussion paper in S2-2506386, namely the SA3 decision on user consent and LMF checking the privacy profile for training and performance monitoring. Failing the privacy profile check, no ground truth data can be collected from the UE and training/performance monitoring cannot happen at NWDAF. |  |  |  |  |  |  |  |  |
| Summary of changes: |  | - 5.18.0: removal of ENs; adopting SA3 decision for UE positioning calculation to not check user consent and indicating the LMF check on privacy profile for training and performance monitoring - 5.18.2: UE LCS privacy profile is checked for performance monitoring - 6.12.1: changes to indicate that LMF can subscribe to updates on privacy profile from UDM; such subscription may happen while data collection is on-going - 6.22.3: removal of EN and call flow insertion of privacy profile check by LMF - 6.22.4: clarifying location data is not shared with NWDAF if user consent/privacy profile checks are negative |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | User consent and privacy for AI/ML positioning Case 3b not correctly specified |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.18.0, 5.18.2, 6.12.1, 6.22.3, 6.22.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 23.502 CR 5562 |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * Start of Changes * * *

### 5.18.0	General

The LMF may calculate the UE location and estimate the achieved accuracy by using LMF-based AI/ML Positioning. When receiving the request from AMF for determining a UE location, the LMF selects an appropriate positioning method as described in clause 5.2 to determine the result of the positioning. The result of the positioning may be calculated by using LMF-based AI/ML Positioning ML model supported by LMF. The LMF collects input data from UE or NG-RAN for the LMF-based AI/ML Positioning to perform location calculation and provide the location to the consumer.

NOTE 1:	Whether to select LMF-based AI/ML Positioning for location result calculation is determined by LMF.

NOTE 2:	The specific measurement data collected by LMF from NG-RAN for LMF-based AI/ML Positioning are in the scope of RAN specifications and not in the scope of this specification.

Editor's note:	What input data collected from UE and NG-RAN to LMF for LMF-based AI/ML Positioning will be determined by RAN WG1.

The ML model that is used for LMF-based AI/ML Positioning may be trained by LMF. The trigger for data collection required for model training in LMF is up to implementation. LMF collects input data from UE for ML model training as described in clause 6.22.2. The LMF collects input data from NG-RAN for ML model training as described in clause 6.22.3.

The LMF may also request a trained ML model for LMF-based AI/ML Positioning from NWDAF containing MTLF as described in clause 6.22.5. The LMF discovers a suitable NWDAF containing MTLF via NRF as described in clause 5.2 of TS 23.288 [37] with the following considerations:

-	The LMF may provide the positioning case information(i.e. NG RAN node assisted LMF-based AI/ML Positioning case).

NOTE 3:	The positioning case information only indicates the NG RAN node assisted LMF-based AI/ML Positioning in this release.

-	The LMF provides an Area of Interest, and may include the ML Model Interoperability indicator to discover an NWDAF that can provide an AI/ML Model.

-	The LMF may request a NWDAF with ML Model accuracy checking capability to report when the model is degraded (i.e. for ML model performance monitoring).

NOTE 4:	Other NWDAF discovery parameters listed in clause 5.2 of TS 23.288 [37] such as Analytics ID, FL capability type and related time period, S-NSSAI or any roaming capabilities are not included by LMF.

The LMF requests the NWDAF containing MTLF to provide an ML Model for LMF-based AI/ML Positioning as described in clause 6.2A of TS 23.288 [37] with the following considerations:

-	The LMF provides the following input parameters in the Nnwdaf_MLModelProvision_Subscribe or Nnwdaf_MLModelInfo_Request:

-	LMF-based AI/ML positioning indication.

-	Optionally, Vendor ID, ML Model Filter Information (e.g. Area of Interest), Positioning case information, Target of ML Model Reporting, ML Model Target Period, Time when model is needed, Inference Input Data information and ML Model Monitoring Information.

-	If vendor specific information is required, then the ML Model Interoperability Information is included.

-	If the LMF supports multiple AI/ML Models, indication of support for multiple ML Models, optionally with Number of ML Models and Accuracy level(s) of Interest.

The NWDAF containing MTLF collects input data to perform the ML model training as described in clause 6.22.4, and the NWDAF containing MTLF performs ML model provision to LMF as described in clause 6.2A of TS 23.288 [37] with the following considerations:

-	The NWDAF containing MTLF provides the ML Model identifier and ML Model Information for the ML Model for UE Positioning, and optionally, the following parameters:

-	ML Model Filter Information and/or Target of ML Model Reporting, if the ML Model provisioning request includes multiple ML Model Filter Information and/or Target of ML Model Reporting;

-	Indication of whether the ML Model identifier is updated (e.g. retrained ML model).

-	Validity period, Spatial validity, Training Input Data Information, ML Model accuracy Information.

Once the ML model for LMF-based AI/ML Positioning is trained and available in the LMF, the LMF may use it to perform UE Positioning after receiving a location determination request from AMF. In this case, the legacy LCS procedures in clause 6 still apply, independently of whether the LMF uses any AI/ML model for UE Positioning calculation.

For ML model training and performance monitoring using LMF-based AI/ML Positioning, the LMF checks with UDM the user consent status before collecting UE related data. In addition, LMF also checks the UE LCS privacy profile to collect ground truth data from the UE and/or to share location measurement data with NWDAF. Details can be found in clauses 6.22.3 and 6.22.4.

For UE Positioning calculation, it is sufficient with the GMLC checking the UE LCS privacy profile as in legacy LCS procedures.

Either LMF or NWDAF containing MTLF may perform performance monitoring for LMF-based AI/ML Positioning. When the ML model that is used for LMF-based AI/ML Positioning is trained by LMF the LMF monitors the performance of the ML model. When the ML model that is used for LMF-based AI/ML Positioning in LMF is trained by NWDAF containing MTLF, then the NWDAF containing MTLF monitors the performance of the ML model. LMF may determine whether to use the LMF-based AI/ML Positioning to perform location calculation based on the model performance monitoring result. LMF or the NWDAF containing MTLF may also trigger the ML model retraining based on the model performance monitoring result.

* * * Next Change * * *

### 5.18.2	AI/ML model performance monitoring for LMF-based AI/ML Positioning

When the AI/ML model that is used for LMF-based AI/ML Positioning is trained by LMF, the LMF may perform performance monitoring for AI/ML model by collecting the measurements data and ground truth data of PRU(s)/UE(s) by using the procedures of data collection for LMF-based AI/ML Positioning as specified in clause 6.22.3. Before collecting the data of UE(s), LMF needs to check the user consent and UE LCS privacy profile in UDM, however LMF does not need to check user consent or UE LCS privacy profile for data of PRU. LMF calculates the location information based on the collected measurement data by using AI/ML model. Based on the calculated location information and ground truth data, LMF evaluates the AI/ML model performance to generate the performance monitoring result. The result may trigger LMF to change the positioning method, e.g. from LMF-based AI/ML Positioning to the legacy positioning or retrain the AI/ML model in LMF.

When the AI/ML model that is used for LMF-based AI/ML Positioning in LMF is trained by NWDAF containing MTLF, then the NWDAF containing MTLF may monitor the performance of the AI/ML model as specified in clause 6.2E.4 of TS 23.288 [37] based on the data collection via the LMF as specified in clause 6.22.4. If the data collection is not granted, the LMF does not perform any data collection procedure for the ML model performance monitoring for this UE.

* * * Next Change * * *

### 6.12.1	UE Location Privacy Setting Procedure Initiated by UE

Figure 6.12.1-1: UE Location Privacy Setting procedure initiated by UE

0.	NF (e.g. GMLC, NEF, LMF) may subscribe to UDM notifications of UE LCS privacy profile updates.

NOTE 1:	NF other than LMF can decide to subscribe to such notifications while a deferred location request is on-going.

NOTE 2:	LMF can decide to subscribe to such notifications while the procedure for data collection for model training or performance monitoring for LMF-based AI/ML Positioning in clause 6.22.3 is on-going.

1.	If the UE has generated or updated the UE Location Privacy Indication, the UE sends the Location Privacy Indication to the AMF via UE Location Privacy Setting Request in N1 NAS message. The UE Location Privacy Indication indicates whether allows or disallows the subsequent LCS requests for the UE, as defined in clause 5.4. If the UE has generated or updated the event report expected area and optionally the area usage indication, e.g. based on UE power status, the UE Location Privacy Setting Request includes the event report expected area and the area usage indication.

2.	The AMF invokes a Nudm_ParameterProvision_Update (LCS privacy) service operation towards the UDM and the service operation carries the Location Privacy Indication information and may include event report expected area and the area usage indication. The UDM stores or updates the UE LCS privacy profile in the UDR by invoking a Nudr_DM_Update (SUPI, Subscription Data) service operation accordingly.

3.	The AMF responses to the UE via UE Location Privacy Setting Response in N1 NAS message.

4.	UDM notifies the subscribed Network Function (e.g. GMLC, NEF, LMF) of the updated UE LCS privacy profile via Nudm_SDM_Notification Notify message.

5.	NF (e.g. GMLC, NEF, LMF) may unsubscribe to UDM notifications of UE LCS privacy profile updates e.g. if a deferred location procedure is cancelled.

* * * Next Change * * *

### 6.22.3	Data collection to train models for LMF-based AI/ML positioning based on NG RAN measurements

The procedure for data collection from NG-RAN is used to e.g. train the ML Model for LMF-based AI/ML positioning.

Figure 6.22.3-1: Data collection by LMF to train the AI/ML based positioning model using NG-RAN measurements

1.	The LMF determines that data collection from the NG-RAN is required e.g. to train an ML Model for UE positioning for a number of UEs or to monitor the ML Model performance. The LMF may also initiate the data collection upon the request of an NWDAF containing MTLF as described in step 3 in figure 6.22.4-1.

2.	The LMF may know the SUPIs of the UEs for which to collect location measurement data from the NG-RAN, e.g. when training an ML model using the PRU(s) associated to this LMF. The LMF may optionally invoke an Nnrf_NFDiscovery Request service operation to an NRF to discover other PRU serving LMF(s) which has associated PRUs in the area of interest and send an Nlmf_Location_MeasurementData Request service operation to the selected PRU serving LMFs to collect the measurement data from the NG-RAN for the PRUs and the PRU locations.

If the LMF does not know the SUPIs of the UEs for which to collect location measurement data from the NG-RAN, the LMF tries to get the list of SUPIs from the AMF. Before that the LMF discovers the AMF(s) that serves the area of interest via the NRF using Nnrf_NFDiscoveryRequest.

3.	The LMF subscribes to the list of SUPIs in an area of interest from the AMF(s) using Namf_EventExposure_Subscribe request (Target of Event Reporting = "any UE", Event ID = "UEs in/out area of interest").

The AMF sends Namf_EventExposure_Subscribe response or Namf_EventExposure_Notify (list of SUPIs in the area of interest).

For each SUPI in the area of interest, the following steps are performed.

4.	The LMF checks whether the SUPI provided user consent for data collection for the purpose of model training/performance monitoring with UDM using Nudm_SDM_Get including subscription data type set to "User consent" for this SUPI. If user consent has not been granted at this stage, the procedure stops here.

In addition, the LMF may check with UDM the privacy settings of the SUPI via invoking Nudm_SDM_Get service operation including subscription data type set to "LCS privacy" for the SUPI. The UDM returns the target UE Privacy setting of the UE. The LMF checks the UE LCS privacy profile. If the UE is not allowed to be located, then step 7 is skipped.

NOTE 1:	Neither user consent check nor UE LCS privacy profile check are needed at LMF when data is collected from PRU.

NOTE 2:	The User consent and UE LCS privacy information can be retrieved simultaneously in one Nudm_SDM_Get service operation invocation.

The LMF may further determine the UEs from the list of SUPIs that are received from AMF in step 3 for data collection based on e.g. UE Positioning Capability, UE User Plane Positioning Capabilities, the PRU information available in the LMF and operator's policy. The LMF may query the UE for UE Positioning Capability and optionally UE User Plane Positioning Capabilities if not received in step 3.

5.	The LMF subscribes to UDM to notifications of changes on subscription data type "User consent" for this SUPI using Nudm_SDM_Subscribe. If user consent is granted, then step 6 follows, otherwise no data is collected for this SUPI, i.e. the following steps are not performed.

6.	[Conditional] The LMF requests input data for the UE from the NG-RAN using the Measurement Information Transfer procedures in NRPPa as specified in clause 8.5 of TS 38.455 [15].

NOTE 3:	The NG-RAN can reject the data collection request from the LMF (e.g. considering current NG-RAN load status).

7.	[Conditional] The LMF requests ground truth data from the UE. The UE may reject the data collection request from the LMF (e.g. considering UE status, user's input). If the UE accepts data collection request, the UE may cancel the data collection later as defined in clause 6.3.4.

8.	The LMF may determine that the UE is no longer in the area of interest, based on the AMF notification using Namf_EventExposure service, then the LMF performs step 10 and step 11, the LMF may unsubscribe to be notified on user consent and LCS privacy updates if the UE is not in the area of interest any longer.

9.	The UDM may notify the LMF on changes of user consent at any time after step 5 using Nudm_SDM_Notification including SUPI and Subscription data type set to "User consent". If user consent is no longer granted for a user for which data has been collected the LMF performs step 10 and step 11. The LMF may unsubscribe to be notified of user consent updates from UDM for each SUPI for which data consent has been revoked, using Nudm_SDM_Unsubscribe including SUPI and Subscription data type set to "User consent".

The LMF may also receive updates of the UE LCS privacy profile if the LMF has subscribed previously to UDM as described in clause 6.12.1. If the LMF learns that the UE is no longer allowed to be located, then step 11 is performed.

10.	The LMF requests NG-RAN to stop reporting input data for the UE.

11.	The LMF stops any retrieval of ground truth data for the UE.

The measurements from NG-RAN and ground truth data from PRU/UE are used for ML model training. The UE location is derived from the measurements data by using LMF-based AI/ML Positioning. The derived UE location and ground truth data are used for ML model performance monitoring.

The LMF may initiate data collection for multiple UEs simultaneously, as such steps 6 and 7 may occur in parallel for a number of SUPIs as determined by the LMF.

Editor's note:	The procedure to collect the input data for AI/ML based positioning calculation by the LMF from the NG-RAN is subject to RAN WG progress.

* * * Next Change * * *

### 6.22.4	Input data collection by NWDAF for AI/ML positioning ML model training or ML model performance monitoring

The NWDAF containing MTLF may subscribe to input data (i.e. location measurement data) from LMF for ML model training or ML model performance monitoring for LMF-based AI/ML Positioning.

Figure 6.22.4-1: Procedure of input data collection from LMF

1.	NWDAF containing MTLF determines to train a ML model for LMF-based AI/ML Positioning based on the request from LMF or internal trigger, or the NWDAF containing MTLF determines to perform ML model performance monitoring for LMF-based AI/ML Positioning.

2.	The NWDAF invokes an Nnrf_NFDiscovery_Request service operation to an NRF to discover an LMF, the service operation includes an AoI and the Nlmf_DataExposure service as discovery parameters. If the NWDAF wants to collect the input data of PRUs, the NWDAF may also include a PRU existence indication for discovering the LMF(s) associated with PRUs (the PRU association procedures are defined in clause 6.17). The NRF selects one or more LMFs based on the AoI, the Nlmf_DataExposure service, and the PRU existence indication (if available), and sends an Nnrf_NFDiscovery_Request Response which includes the profiles of the selected LMFs to the NWDAF.

3.	The NWDAF subscribes to or cancels subscription to input data from LMF by invoking Nlmf_DataExposure_Subscribe / Nlmf_DataExposure_UnSubscribe service operation. The NWDAF includes an AoI and a notification target address to request the input data from LMF. The NWDAF may also include requested number of data samples, time window of data samples, quality threshold, ML model identifier (for ML model performance monitoring), data source type (i.e. NG-RAN measurement). The quality threshold indicates to the LMF to provide ground truth data only when the ground truth data meets the quality threshold. The detailed parameters are defined in clause 8.3.4.

NOTE 1:	If the quality threshold is included in step 3 and the collected data sample does not include quality indicator of the ground truth data, the LMF does not send this data sample to the NWDAF.

NOTE 2:	The data source type only indicates NG-RAN measurement in this release.

4.	For ML model training and ML model performance monitoring, the LMF performs the procedure for data collection in clause 6.22.3 to collect data from PRUs/UEs and/or the NG-RAN. For ML model performance monitoring, the LMF may also calculate the location estimation of PRU(s)/UE(s) using the collected data and the ML model identified by the ML model identifier if received in step 3.

If at any point LMF performs steps 10 or 11 in Figure 6.22.3-1 because user consent is not granted or the UE LCS privacy profile indicates that UE is not allowed to be located, then step 5 is skipped.

5.	The LMF sends the collected data samples (i.e. location measurement data from the NG-RAN, the corresponding ground truth data (i.e. location of PRUs or UEs) and the quality indicator of the ground truth data) to the NWDAF by invoking Nlmf_DataExposure_Notify service operation. The LMF may send a cause code to the NWDAF when the requested number of data samples cannot be met. The LMF may also send the location estimation of PRU(s)/UE(s) calculated in step 4 (if applicable). Then the NWDAF trains the ML model or performs ML model performance monitoring based on the data samples received from the LMF.

* * * End of Changes * * *