SA WG2 Meeting #170	S2-2506396

25 – 29 Aug, 2025, Goteborg, SE

Source:	Boost Mobile Network, EchoStar, China Mobile, Cisco Systems

Title:	[WT#3, AI for 6G] Support AI-Native Architecture in 6G

Document for:	Approval

Agenda Item:	20.6.3

Work Item / Release:	FS_6G_ARC/Rel-20

Abstract of the contribution: This contribution proposes WT#3 detailed scope and a Key Issue under AI in 6G WT. This key issue recommends the study of 6G system architecture support of AI natively where AI is used to enhance 6G system performance and 6G system provides the needed support for AI to achieve its functionality.

# 1. Justifications

After 3GPP has defined the 5G system architecture as per Release 15, there have been several studies to support AI/ML in 5G and some of them is still ongoing. The support of AI/ML in 5G is a centralized approach where all the AI/ML logic is concentrated in the NWDAF. One of the shortcomings of the 5G solution is that the inference and decision making through NWDAF is not as immediate as needed and requires at least one round trip communication between NWDAF and the respective 5G NF. The reason for this delay is that AI/ML support in 5G was done after the conclusion of 5G system architecture and was introduced as over the top rather than being natively integrated and supported.

Learning from 5G experience, 6G needs to support AI natively and extend over the UE, 6G RAN, and 6G core network, i.e., end-to-end, where AI functionalities is distributed while not preventing a possible centralized approach, e.g., using NWDAF. It is expected that AI in 6G becomes an integral part of every node of the 6G system decision making. Thus, it is expected to have AI agent across the 6G system from the UE to 6G RAN and 6G Core network. AI in 6G requires a data framework that enables the communication between the different AI entities, e.g., AI Agents, collection of the data, training the data, storing the data while at the same time being able to provide inference and decision almost immediately to the respective node.

It is imperative that the proposed key issue for AI in 6G has a dependency on the Data Framework in 6G that is handled under WT#5.

**** First Change ****

# 2. WT Scope

# Annex A.X. WT#3

Editor's Note: Describe the technical scope of the proposed Work Task. If applicable, suggest logical subdivision of this WT into smaller sub-WT. This clause is part of the TR Annex.

WT#3: Study how to support and enable use of AI in 6G (e.g. AI agent, framework).

NOTE 4: 	The term AI agent refers to the general concept of agents autonomously performing tasks on behalf of users, systems, and/or applications. As the SA1 work is still in progress, adapting the definition of AI agent from SA1 and the use of the term AI agent will be determined as part of the study. AI agent does not imply any specific solution.

The support of AI in 6G is required to enable 6G system utilize the potential and what AI can offer for enhancing the 6G system and network. This WT is meant to study how to garnish AI capabilities in 6G and make AI as integral part of the decision making of every node of the 6G system.

6G is expected to support AI natively where AI in 6G is used for the 6G System performance enhancement while at the same time 6G system supports the communication between the different entities of AI across 6G system.

It is proposed to study the following aspects of supporting AI in 6G as follows:

- Study how 6G system architecture can support AI natively where every node in the 6G system can utilize AI inference as part of its decision making.

- Study how 6G system can support AI where every node in the 6G system is able to receive AI inference as immediate as possible without delay, something similar to as if the node is making an internal decision.

- Study how 6G system can provide a data framework where all AI entities, e.g., AI Agents, across 6G system can communicate with each other, collect data, communicate its inference, store training data, and distribute data as needed.

- Study how 6G system can support AI across the 6G system where AI inference can be provided to the UE, 6G RAN, and 6G Core network, i.e., end-to-end.

- Study whether 6G system needs to support a centralized AI approach, e.g., using NWDAF, while being able to meet the immediate nature of AI inference without extended latency.

NOTE 5: This WT has a dependency on the data framework WT#5 and possibly require coordination with RAN WGs.

# 5.X. Key Issue #X: Supporting AI-Native Architecture in 6G

Editor's Note: This clause defines the potential scope of KI(s) and is part of the TR.

AI is becoming essential in every technology and every system. Therefore, the support of AI in 6G is required to enable 6G system utilize the potential and what AI can offer for enhancing the 6G system and network. This key issue proposes the detailed study for how to garnish AI capabilities in 6G and make AI as integral part of the decision making of every node of the 6G system.

6G is expected to support AI natively where AI in 6G is used for the 6G System performance enhancement while at the same time 6G system supports the communication between the different entities of AI across 6G system.

This key issue proposes to study the following aspects of supporting AI in 6G

- Study how 6G system architecture can support AI natively where every node in the 6G system can utilize AI inference as part of its decision making.

- Study how 6G system can support AI where every node in the 6G system is able to receive AI inference as immediate as possible without delay, something similar to as if the node is making an internal decision.

- Study how 6G system can provide a data framework where all AI entities, e.g., AI Agents, across 6G system can communicate with each other, collect data, communicate its inference, store training data, and distribute data as needed.

- Study how 6G system can support AI across the 6G system where AI inference can be provided to the UE, 6G RAN, and 6G Core network, i.e., end-to-end.

- Study whether 6G system needs to support a centralized AI approach, e.g., using NWDAF, while being able to meet the immediate nature of AI inference without extended latency.

NOTE: This key issue has a dependency on the data framework WT#5 and possibly require coordination with RAN WGs.

**** End of Changes ****