SA WG2 Meeting #170	S2-2506397

25 – 29 Aug, 2025, Goteborg, SE

Source:	Boost Mobile Network, EchoStar, China Mobile, Cisco Systems, Verizon

Title:	[WT#5, Data Framework] Supporting Data Framework for 6G

Document for:	Approval

Agenda Item:	20.6.5

Work Item / Release:	FS_6G_ARC/Rel-20

Abstract of the contribution: This contribution proposes a detailed scope and a Key Issue for Data Framework WT#5. This key issue recommends studying the data framework that is required to support 6G system architecture, e.g., AI in 6G (WT#3) and 6G Integrated Sensing Communication (WT#4). The study includes data collection, data storage, data distribution, data processing, data access, etc.

# 1. Justifications

Currently, up to 5G, there are two distinct planes for carrying data throughout the 3GPP mobile network. One is dedicated for the control plane (CP) and another for the user plane (UP). The CP is dedicated for communicating control messages between the UE, the RAN, and the CN. These messages follow a specific protocol and detailed structure. In 5G, most CP in the core network is SBI based. On the other hand, UP is used to carry the user data and deliver it to the user intended destination.

Traditionally, some control plane messages have been carried over the UP, e.g., IMS. Since IMS control messages are essential for establishing voice calls, it has been provided a higher priority than some other UP data that, for example the data the user intend to send to the internet.

In 6G, there are several fundamental technologies, e.g., Native AI, to be introduced and supported where these technologies produce and consume data from across the 6G system, i.e., UE, 6G RAN, and 6G core network. The amount of produced and collected data is anticipated to be huge and continuously growing. Since, these data do not fit neither the CP nor the UP, it is believed that 6G needs to define a data framework which is capable to handle the anticipated huge amount of data and its nature of continuously growing.

This contribution proposes a key issue for 6G Data framework (WT#5) where a framework is defined to handle the characteristics of the newly generated data and where it is being stored, distributed, processed, etc.

It is expected that the proposed key issue for AI in 6G (5.X) has a dependency on this Data Framework in 6G key issue.

**** First Change ****

# 2. WT Scope

# Annex A.X. WT#5

Editor's Note: Describe the technical scope of the proposed Work Task. If applicable, suggest logical subdivision of this WT into smaller sub-WT. This clause is part of the TR Annex.

WT#5: Study data framework for all aspects related to efficient and scalable data handling including, for example, data collection, distribution, processing, storage, data access and data exposure, with consideration of access control/user consent and privacy where relevant. The example of data may include data for AI and Sensing. This WT can also study any potential enhancements on system and procedure needed for user consent framework.

NOTE 6:	The work split with SA3, SA5 and RAN WGs will require TSG coordination

It is proposed to study the following aspects to support Data Framework for 6G.

- Study how 6G system architecture can support a data framework to handle the huge amount of data being generated by, for example, AI and sensing where it handles data collection, data distribution, data processing, e.g., feature engineering, data storage, data access, etc.

- Study whether the existing CP and UP can handle the demand of the huge data that is being generated and distributed for Native AI, sensing, etc. and whether it is suitable for such task.

- Study how to collect and distribute data from multi-sources across the 6G system from the UE, 6G RAN, and 6G core network.

- Study how to standardise unified data models for different data categories, e.g., telemetry, event logs, and application data, etc.

- Study all data security related aspects that covers data ownership, data privacy, data access control, data governance including governance of data exposure to 3rd party and cross-domain governance, user consent, etc.

- Study whether and how a new data/service plane is needed to handle the anticipated end-to-end data collection and distribution and processing where AI entities are able to use such data plane to train models and generate the proper inference and decision.

- Study how to handle data lifecycle and audit management including lifecycle management policy, audit trails, automated decision enforcement, etc.

- Study how 6G system can provide a data framework where all AI entities, e.g., AI Agents, Autonomous AI Agents, across 6G system can communicate with each other, e.g., semantic data fusion, AI-to-AI messages, collect data, communicate its inference, e.g., store training data, and distribute data based on its content, e.g., low latency delivery, context-aware routing, and edge-aware replication.

NOTE 7: Study of data security related aspects requires collaboration with SA3.

NOTE 8: Study of data lifecycle and audit management requires the collaboration with SA5 working group.

# 5.Y. Key Issue #Y: Supporting Data Framework for 6G

Editor's Note: This clause defines the potential scope of KI(s) and is part of the TR.

In 6G, there are several fundamental technologies, e.g., AI, to be introduced and supported. These technologies are expected to produce data from across the 6G system, i.e., UE, 6G RAN, and 6G core network. The amount of produced, collected, and to be distributed data is anticipated to be huge and continuously growing. At a first evaluation, these data do not fit neither the CP nor the UP. Thus, it is believed that 6G needs to define a data framework that is capable to handle the anticipated huge amount of data and its nature of being continuously growing.

This key issue is for 6G Data framework (WT#5) where a framework is defined to handle the characteristics of the newly generated and existed data and where it is being stored, distributed, processed, etc.

This key issue proposes to study the following aspects of supporting Data Framework for 6G.

- Study how 6G system architecture can support a data framework to handle the huge amount of data being generated by, for example, AI and sensing where it handles data collection, data distribution, data processing, e.g., feature engineering, data storage, data access, etc.

- Study whether the existing CP and UP can handle the demand of the huge data that is being generated and distributed for Native AI, sensing, etc. and whether it is suitable for such task.

- Study how to collect and distribute data from multi-sources across the 6G system from the UE, 6G RAN, and 6G core network.

- Study how to standardise unified data models for different data categories, e.g., telemetry, event logs, and application data, etc.

- Study all data security related aspects that covers data ownership, data privacy, data access control, data governance including governance of data exposure to 3rd party and cross-domain governance, user consent, etc.

- Study whether and how a new data/service plane is needed to handle the anticipated end-to-end data collection and distribution and processing where AI entities are able to use such data plane to train models and generate the proper inference and decision.

- Study how to handle data lifecycle and audit management including lifecycle management policy, audit trails, automated decision enforcement, etc.

- Study how 6G system can provide a data framework where all AI entities, e.g., AI Agents, Autonomous AI Agents, across 6G system can communicate with each other, e.g., semantic data fusion, AI-to-AI messages, collect data, communicate its inference, e.g., store training data, and distribute data based on its content, e.g., low latency delivery, context-aware routing, and edge-aware replication.

NOTE 1: Study of data security related aspects requires collaboration with SA3.

NOTE 2: Study of data lifecycle and audit management requires the collaboration with SA5 working group.

NOTE 3: Key Issue (5.X) of WT#3 AI in 6G has a dependency on this data framework WT#5 key issue and possibly require coordination with RAN WGs.

**** End of Changes ****